3GPP TSG-RAN WG4 Meeting # 107	R4-2307057

# Incheon, KR, May 22 – May 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Input on CA_n28-n105 requirements

Agenda Item:	8.4.2.1	CA configuration of CA_n5A-n105A	[FS_NR_sub1GHz_combo_enh]

Document for:	Approval

# Introduction

RAN4#106b-e, the support of 1UL and 2UL configurations for CA_n28-n105 was discussed in [2] and a few agreements were captured in way forward [1]. In this contribution, we provide our input on the remaining 1UL and delta T, Delta R evaluation that is further captured as a place holder for CR [3].

# Discussion

In RAN4#106b-e, the way forward [1] had the following agreements:

- Only Option 2 (two antennas dual triplexer) and Option 3 (three antennas) UE architectures will be captured in the study item.

- Baseline architecture for UE RF requirements development will be further discussed.

- Only single UL configuration is considered in the study item where UL can be in either n28 or n105.

- Further study on REFSENS impact due to cross-band UL to DL interference.

Based on the above agreements, only 1 UL MSD and Delta T and Delta R need further evaluation.

## Delta T and Delta R

For CA_n28-n105 the 2 antenna implementation should drive the TIB,c and RIB,c values. It is suggested to reuse the similar band combination CA_n71-n85 TIB,c and RIB,c values agreed in RAN4#106 based on [4, 5], with an additional 0.2dB to n28 to enable the two-antenna dual tri-plexers architecture in the future including the support of full band n28.

Proposal on Delta T and Delta R: the values in Table 1 and 2 are used for CA_n28-n105

Table 1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n105 | [1.0] | 1.0 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n105 | [1.0] | 1.0 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

## 1UL cross band MSD

Since the two bands are very close, cross-band isolation may be an issue as shown in Figure 1 below, the Table 3 provides the analysis for the potential IMD order of the UL configuration and its image for the largest channel bandwidths.

Figure 1: IMD landscape for 1UL and 2UL configurations of CA_n28-n105

Table 3: Cross band isolation issues

| UL Band | UL Low Band Edge | UL High Band Edge | DL Band | DL Low Band Edge | DL High Band Edge | UL to DL distance | Maximum UL BW | Source of interference (IMD order of UL RBs and their image) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 703 | 748 | n105 | 612 | 652 | 51 | 30 | ACLR2 (IMD5) |
| n105 | 663 | 703 | n28 | 758 | 803 | 55 | 20 | ACLR3 (IMD7) |

As shown in the above table, the band n28 30MHz CBW UL ACLR2 overlaps with the band n105 DL while the band n105 20MHz CBW UL ACLR3 overlaps with band n28 DL. Both MSDs need to be evaluated.

Based on the very similar case CA_n7-n40 where n7 UL has ACLR3 overlap with n40 DL, the cross-band isolation MSD of band n28 with n105 UL can be derived.

Proposal on n28 MSD with n105 1UL: MSD test point and value in Table 4 is proposed

Table 4: n28 MSD due to cross band isolation for CA_n5-n28 with n105 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | [3.7] | >ACLR2 |

Based on the very similar case CA_n18-n28 where n18 UL has the same RB allocation than n28 UL with similar overlap, the cross-band isolation MSD of band n105 with n28 UL can be derived.

Proposal on n105 MSD with n28 1UL: MSD test point and value in Table 5 is proposed

Table 5: n105 MSD due to cross band isolation for CA_n5-n28 with n28 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | [12.7] | ACLR2 |

# Conclusions

In this contribution, we have discussed the requirement to complete CA_n28-n105 study and make the following proposals.

Proposal on Delta T and Delta R: the values in Table 1 and 2 are used for CA_n28-n105

Table 1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n105 | [1.0] | 1.0 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n105 | [1.0] | 1.0 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Proposal on n28 MSD with n105 1UL: MSD test point and value in Table 4 is proposed

Table 4: n28 MSD due to cross band isolation for CA_n5-n28 with n105 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | [3.7] | >ACLR2 |

Proposal on n105 MSD with n28 1UL: MSD test point and value in Table 5 is proposed

Table 5: n105 MSD due to cross band isolation for CA_n5-n28 with n28 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | [12.7] | ACLR2 |

# References

- R4-2306470 WF on RF architecture and critical issues for CA_n28-n105, Apple, RAN4#106b-e

- R4-2304561 Input on new CA_n28A-n105A band combinations study, Skyworks Solutions, Inc., RAN4#106b-e

- R4-237058 DraftCR for TR 38.872 on CA_n28-n105 1UL and 2UL configurations, Skyworks Solutions, Inc., RAN4#107

- R4-2300414 TP to TR 38.717-02-01: Addition of CA_n71-n85 Nokia, T-Mobile USA, RAN4#106

- R4-2300758 1UL cross band MSDs for CA_n71-n85 and DC_12_n71, Skyworks Solutions, Inc., RAN4#106