3GPP TSG-RAN WG4 Meeting #107                                          R4-2307143

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On UE RF requirements for 4Tx

Agenda Item:	8.5.1.1

Document for:	Approval

Introduction

According to the WID, the requirements for 4Tx are split to two phases. In the latest WF, some issues are left to be discussed in this meeting, concerning which would some views will be provided in this contribution. Besides, while the progress of phase 1, which focuses on 4x4 MIMO with 4x23dBm implementation for PC1.5, has been well developed in the recent several meetings, we do believe it is about time to preheat for the phase 2, which further takes 2x23dBm + 2x26dBm UE and 4x26dBm UE into consideration.

This contribution provides analysis on addressing the remaining issues for phase 2.

Discussion

Some issues w/o consensus

According to the WF, the following issues could be further discussed in RAN4.

Issue 1-3-1: Whether power scaling related to ULFPTx TPMI(s) should be considered in Configured transmitted power definition

- Proposals

  - Option 1: No.

    - This is the current 38.101-1 status.

  - Option 2: Yes

    - Not align with current 38.101-1

    - It should be noted Mode 2 would be much more complicated compared to Mode 1.

  - Option 3: Others

Issue 1-3-2: Whether acceptable for UE not able to achieve Pcmax it reports, e.g. for certain TPMI configurations;

- Proposals

  - Option 1: No.

    - This is the current 38.101-1 status.

  - Option 2: Yes

    - Not align with current 38.101-1

    - It should be noted Mode 2 would be much more complicated compared to Mode 1.

  - Option 3: Others

Issue 1-3-3: What kind of solution might be way forward?

- Proposals

  - Option 1: Keep the current RAN4 spec, that Pcmax would not be impacted by TPMI

    - Pcmax may not be always achievable under certain configuration

    - Corresponds to option 1 in Issue 1-1 / 1-2

  - Option 2: Consider revising RAN4 spec, that Pcmax can be adjusted by TPMI

    - Pcmax can always be achievable, or introduce a maximum achievable output power for a configuration via ΔPpower_class in PCMAX calculation

    - Corresponds to option 2 in Issue 1-1 / 1-2

  - Option 3: Others

For above issues, we stand pat on our position and don't think further consideration is needed unless consensus among companies reached for the raised issues with further clarification. On the other hand, as already mentioned by some companies in the contributions, the ULFPTx modes and TPMI for different PA configurations should be discussed in phase 2, which will be discussed in section 2.2.

Overview of 4Tx requirements towards phase 2

ULFTPx relevant spec impact

While a draft CR [2] has been submitted in last meeting to proceed the discussion of 4Tx requirements in phase 1, 2x23dBm+2x26dBm UE and 4x26dBm UE has not been considered yet. As it is about time to wrap up for phase 1, this meeting would be a great opportunity to kick off some discussions for phase 2.

Concerning PUSCH Configuration for uplink full power transmission (ULFPTx), further supplement shall be made to accommodate 2x23dBm+2x26dBm UE and 4x26dBm UE. To be more specific, in order to meet with PC 1.5 requirements, these UEs would need to adopt FullPwrMode2 to achieve ULFPTx as they are “overqualified” for FullPwrMode1 yet not able to support FullPwrMode. However, the entry for ULFPTx Mode2 with 4 Tx ports transmission in Table 6.2D.1-3 in TS 38.101-1 is still missing, which is supposed to be added considering the completeness of the table.

Proposal 1: The entry for ULFPTx Mode2 with 4 Tx ports transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added concerning 2x23dBm+2x26dBm UE and 4x26dBm UE.

In 4.2.7.7, TS 38.306, it has been stated that feature ul-FullPwrMode2-TPMIGroup-r16 would indicate the UE supported TPMI group(s) which delivers full power. 7 TPMI groups G0-6 have been predefined, which are listed as below:

| ID | TPMI groups |
| --- | --- |
| G0 | 121000, |
| G1 | 121000, 120010, 1210000100, |
| G2 | 121000, 120100, 120010,1210000100, 1210010000,1200100100, 12100010001000 |
| G3 | 1210000100, 1210010000,1200100100, 12100010001000 |
| G4 | 121010, 1210-10,1210j0,1210-j0,1210000100 |
| G5 | 121010, 1210-10,1210j0,1210-j01210000100, 1210010000,1200100100, 12100010001000 |
| G6 | 121010, 1210-10,1210j0,1210-j0, 120101, 12010-1,12010j,12010-j 1210010000, 1210000100, 1210000001,1200100100,1200100001, 1200001001,12100010001000 |

gNB could have a basic assumption of UE’s PA architecture by acquiring this TPMI subset information.

For 2x23dBm+2x26dBm UE, only by reporting G4 in ul-FullPwrMode2-TPMIGroup-r16 could it achieve full power transmission for PC 1.5 under ULFPTx Mode2. Any other codebook subsets would require the UE to transmit with either two 23dBm ports or one 23dBm & one 26dBm port to implement 2-layer transmission. With this information, gNB shall be aware of the fact that this particular UE has only two ports which could add up to 29dBm transmission. And as the codebooks in G4 = {121010, 1210-10,1210j0,1210-j0,1210000100} implies, port 0 and port 2 are the ones with 26dBm PA. Same consensus would be held when scheduling SRS resource and PUSCH transmission.

For 4x26dBm UE, on the other hand, would report G6 in ul-FullPwrMode2-TPMIGroup-r16 as any two ports combination could achieve PC 1.5 under ULFPTx Mode2. With this information, gNB shall be aware of the fact that all ports of this particular UE could deliver 26dBm power. Same consensus would be held when scheduling SRS resource and PUSCH transmission.

Observation 1: 2x23dBm+2x26dBm UE would report G4 in ul-FullPwrMode2-TPMIGroup-r16. Based on this feedback, gNB shall recognize this UE as the one with two 26dBm ports and two less-power ports

Observation 2: 4x26dBm UE would report G6 in ul-FullPwrMode2-TPMIGroup-r16. Based on this feedback, gNB shall recognize this UE as the one with four 26dBm ports.

Given this consensus between UE and gNB, firstly for 2-layer transmission, all codebooks in 4Tx FC-CBS and 4Tx PC-CBS could enable ULFPTx. Considering codebooks in 4Tx NC-CBS:

For 2x23dBm+2x26dBm UE, as gNB believes only port 0 and port 2 could deliver 26dBm power, the only eligible codebook in NC-CBS for 2-layer transmission would be  with TPMI = 1 from Table 6.3.1.5-5 in TS 38.211.

For 4x26dBm UE, on the other hand, as all ports could deliver 26dBm power, any codebooks in 4Tx NC-CBS for 2-layer transmission, a.k.a:

, , , ,  and

corresponding to TPMI = 0, 1, 2, 3, 4 and 5 are eligible of ULFPTx for this UE.

Proposal 2: The entry for ULFPTx Mode2 with 4 Tx ports transmission and 2-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

For 1-layer transmission, none of the codebooks in 4Tx NC-CBS could enable ULFPTx while all codebooks in 4Tx FC-CBS could do so. Thus, considering codebooks in 4Tx PC-CBS:

For 2x23dBm+2x26dBm UE, as gNB believes only port 0 and port 2 could deliver 26dBm power, the eligible cookbooks in 4Tx PC-CBS would be the ones align with the 1-layer codebook in G4, a.k.a:

, , ,

corresponding to TPMI = 4, 5, 6 ,7.

For 4x26dBm UE, all ports could deliver 26dBm power, all cookbooks in 4Tx PC-CBS for 1-layer transmission, a.k.a:

, , , , , , ,

corresponding to TPMI = 4, 5, 6 ,7, 8, 9, 10, 11 are eligible of ULFPTx for this UE.

Proposal 3: The entry for ULFPTx Mode2 with 4 Tx ports transmission and 1-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

In addition, according to the WF, full power Rank 2 configuration was not considered in the draft CR for phase 1. However, layer 2 for UE supporting 4Tx is a typical scenario. For ULFPTx mode 1, we share similar view as those discussed in [3], TPMI index 6 could be considered for mode 1.

Proposal 4: The entry for ULFPTx Mode1 with 4 Tx ports transmission and 2-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 6NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Other

Apart from the main issue on UE supporting ULFPTx with different PA configurations under the same power class, another issue on PCMAX tolerance was raised in [4] and discussed in last meeting.

Table 1: Example of PCMAX,c tolerance for 4 Tx UL-MIMO [3]

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 25 ≤ PCMAX,c < 26 | 5.0 | 2.0 |
| 24 ≤ PCMAX,c < 25 | 5.0 | 3.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 4.0 |
| 19 ≤ PCMAX,c < 23 | 5.0 |  |
| 14 ≤ PCMAX,c < 19 | 6.0 |  |
| -40 ≤ PCMAX,c < 14 | 7.0 |  |

The basic consideration is to relax the tolerance for UE supporting more antennas. We think it is aligned with the methodology we adopted for 2Tx UL MIMO since keeping same power accuracy for UE would be more difficult for more RF chains. The values proposed in [4] can be considered as a starting point.

Proposal 5: Some relaxation should be considered for PCMAX tolerance for UE supporting 4Tx.

Conclusion

This contribution provides consideration for RF requirements for UE supporting 4Tx in phase 2.

Proposal 1: The entry for ULFPTx Mode2 with 4 Tx ports transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added concerning 2x23dBm+2x26dBm UE and 4x26dBm UE.

Observation 1: 2x23dBm+2x26dBm UE would report G4 in ul-FullPwrMode2-TPMIGroup-r16. Based on this feedback, gNB shall recognize this UE as the one with two 26dBm ports and two less-power ports

Observation 2: 4x26dBm UE would report G6 in ul-FullPwrMode2-TPMIGroup-r16. Based on this feedback, gNB shall recognize this UE as the one with four 26dBm ports.

Proposal 2: The entry for ULFPTx Mode2 with 4 Tx ports transmission and 2-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Proposal 3: The entry for ULFPTx Mode2 with 4 Tx ports transmission and 1-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Proposal 4: The entry for ULFPTx Mode1 with 4 Tx ports transmission and 2-layer transmission in Table 6.2D.1-3 in TS 38.101-1 shall be added as below：

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 6NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Proposal 5: Some relaxation should be considered for PCMAX tolerance for UE supporting 4Tx.

- References

[1] R4- 2306595, WF on FR1 4Tx UE RF requirements, vivo

[2] R4-2305562, draft CR to TS 38.101-1 4Tx requirements (phase 1), Huawei, HiSilicon

[3] R4-2305081, Discussion on 4Tx UE RF requirements, vivo

[3] R4-2304609, 4Tx UE RF requirements, LG Electronics