3GPP TSG-RAN WG4 Meeting #107	                                                  R4-2307232

# Incheon, South Korea, May 22 – May 26, 2023

Source:		Nokia, Nokia Shanghai Bell

Title:	Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements

Agenda item:	8.8.2.1

Document for:	Approval

# Introduction

The revised work item on Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was approved at TSG RAN#98e [1]. One of the objectives of this work item is to:

•	Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs.

Moreover, it is stated in the WID [1] that:

•	The legacy spherical coverage requirement for reception from a single direction will be kept

And that:

•	PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized.

The subject was discussed at TSG RAN4#106-bis-e and the WF was agreed [2].

# Discussion

In RAN4 #106-bis-e meeting, a common WF [2] for agenda items 5.8.2.1 and 5.8.2.2, i.e., ‘System parameter assumption, UE architecture and conditions of UE RF requirements’ and ‘UE RF requirements’ was approved, and the agreements related to the agenda item 5.8.2.1 are provided below.

This contribution provides Nokia’s further views for defining the RF requirements for FR2-1 multi-Rx chain DL reception from two directions for PC3 UEs.

### TRP to module pairing Option 1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP Option 2: Use max[log(1+SINR_AoA1) + log(1+SINR_AoA2)] to select the best module/beam for each TRP Option 3: Up to UE implementation. No need to reach an agreement. GTW Agreement: - Take Option 1 as the baseline assumption for simulation. - Companies are expected to clarify the assumptions whether RSRP or SINR is used for simulation. ### On module splitting Agreement: Up to UE implementation. No need to further discuss it given the RAN4 agreement on “antenna module” and “panel”

### UE capability for mDCI Agreement: - The same requirement shall be applied to the UEs supporting either of the following two capability combinations: - UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16”. - UE capability “multiDCI-MultiTRP-r16” and “overlapPDSCHsInTimePartiallyFreq-r16”. - No need to discuss and define the RF requirement for the UEs only supporting “multiDCI-MultiTRP-r16”. ### Impact on single AoA performance Agreement: It is up to UE implementation how many panels are activated for multi-Rx UEs during legacy (before R18) requirement verification. ### On MMSE-IRC Agreement: It is up to UE implementation what receiver is used meeting the RF requirement In the simulation, take min[SINR_AoA1, SINR_AoA2] >= -1dB as Pass/Fail criterion

## 2.1	Further views on UE capability for mDCI

One of the agreements for ‘UE capability for mDCI’ (Issue 1.3) is that there is no need to discuss and define the RF requirement for the UEs only supporting “multiDCI-MultiTRP-r16”.

The definition of multiDCI-MultiTRP-r16 capability as per 3GPP TS 38.306 is given below.

As per the above definition, for UEs only supporting “multiDCI-MultiTRP-r16” capability, the network will schedule multi-TRP PDSCHs only through non-overlapped frequency resources. Hence, there will be no inter-TRP/beam interference in this (non-overlapping frequency) case due to which same legacy requirements need to be individually applied for the AoA1 and AoA2.

Observation 1: There will be no inter-TRP/beam interference for the case when UE only supports “multiDCI-MultiTRP-r16” capability.

Proposal 1: RAN4 to clarify whether the agreement “No need to discuss and define the RF requirement for the UEs only supporting “multiDCI-MultiTRP-r16” means:

- No new requirements needed to be defined and the legacy requirements will be individually applied for testing AoA1 and AoA2, OR

- No RF requirements at all will be applicable for this scenario meaning this case will not be tested.

## 2.2	Simulation Results

In this section, we present simulation results to study the interference effect of AoA2 on AoA1. Therefore, we consider signal-to-interference ratio (SIR) metric for our study in this contribution. Some of the text and result(s) in this section reiterates our view from the previous RAN4 contribution.

We have assumed that the UE is equipped with three antenna arrays/modules (each on its top, left and right surfaces) where each antenna array can steer the beam in seven different directions as shown in Figure 1. Therefore, the UE has a total of 21 beams arranged in three groups, namely:

- Beam set for Antenna Array #1 (7 beams shown in variation of blue color)

- Beam set for Antenna Array #2 (7 beams shown in variation of green color)

- Beam set for Antenna Array #3 (7 beams shown in variation of red color)

Figure 1: FR2 UE with 3 antenna panels with 7 beams for each panel

The description of 7 beams selected from any beam set (group) is provided in Table 1.

| Beam Index | Description |
| --- | --- |
| Beam 1 | -45o with respect to Boresight Beam |
| Beam 2 | -30o with respect to Boresight Beam |
| Beam 3 | -15o with respect to Boresight Beam |
| Beam 4 | Boresight Beam |
| Beam 5 | +15o with respect to Boresight Beam |
| Beam 6 | +30o with respect to Boresight Beam |
| Beam 7 | +45o with respect to Boresight Beam |

Table 1: Beam index and description for a UE antenna panel

For Antenna Arrays #2 (right side UE antenna panel), clockwise direction from Boresight beam is assumed to be negative angle and counter-clockwise direction is taken as positive angle whereas for Antenna Array #1 and #3 (left side UE antenna panel), clockwise direction from Boresight beam is assumed to be positive angle and counter-clockwise direction is taken as negative angle.

### 2.2.1	Standalone Antenna Array Configuration (Ideal Case)

Figure 2 shows the standalone antenna array configuration with 1x4 UE antenna arrays and angular range over which each one of the 21 beams (7 beams on each of the three antenna modules) is dominant and has the highest RSRP over the other beams.

Figure 2: (a) Standalone antenna array configuration, (b) Angular range for dominant beams

Figure 3 demonstrates that the signal (S2) transmitted from TRP2 causes interference to the signal received on the Boresight beam of Antenna Array #1 which is used to receive the single polarized signal (S1) transmitted from TRP1.

Figure 3: UE beam radiation pattern for AoA1 and interference from AoA2

In Figure 3, S1 and S2 are, respectively, the signals transmitted from TRP1 and TRP2, h11 and h12 are the corresponding path gain/loss, and P_h11 and P_h12 are the corresponding powers of the desired and interference signals. Therefore, if the AoA1 and AoA2 has an angular separation of 90o and the beam radiation pattern of each UE panel is  as shown in Figure 3, then ‘Green Circle’ and ‘Red Circle’ on the beam radiation pattern of each UE panel denote the received power/antenna gain of the desired and interference signals for AoA1 and subsequently the SIR can be defined as the ratio of the antenna gain values corresponding to the ‘Green Circle’ to that of the ‘Red Circle’. In this case, signal transmitted from TRP2 is also received on the side lobe of the dominant beam of AoA1 which creates the interference. In general, this interference power/gain depends on the AoA separation and beam radiation pattern of each UE panel.

Next, we present the CCDF of SIR of AoA1 for the case when Array #1’s Boresight beam is used for the reception of the useful/desired signal from TRP1 whereas one of the 14 beams of Array #2/Array #3 is used for the reception of the desired signal from AoA2 which subsequently will become the interference for AoA1’s signal.

Figure 4: CCDF of AoA1’s SIR

To evaluate the CCDF, we used the antenna radiation pattern with 1o resolution in azimuth and elevation domain resulting in a total of 65160 constant step size grid points. Thereafter, a uniformly distributed reference grid with 2500 sample points is defined and the 2500 points out of the 65160 points (on the constant step size grid) which are closest to the 2500 points on the uniformly distributed reference grid are selected. These selected points are then sorted in a descending order and are plotted as the CCDF curves.

In the CCDF curve of Figure 4, the legend ‘Array x-Beam y’ where x∈{2,3} and y∈{1,2,3,4,5,6,7} can be interpreted as described in Figure 1 and Table 1. Figure 4 shows the CCDF of SIR of AoA1 for the case when the antenna panels are configured in a standalone fashion as shown in Figure 2(a) and each one of the 14 beams of Array#2 and Array#3 creates different amount of interference to the signal received at ‘Array 1-Beam 4’ resulting in different CCDF curves in Figure 4. The SIR in these figures is normalized to the maximum antenna gain values of the primary beam selected for AoA1. As such, the displayed results are best case results and can be 2 to 3 dB worse depending on the power ripple in the total combined power envelope of the configural beams (codebook) of the antenna array.

Observation 2: The interference from the TRP2 signals arriving from the AoA2 direction becomes significant when the angular separation between AoA1 and AoA2 is low.

Observation 3: The SIR values corresponding to the 50th percentile of the CCDF curves of Figure 4 is shown in Table 2.

| Beam Index (Array 2/Array 3) | SIR at 50-percentile of CCDF (dB) |
| --- | --- |
| Beam 7 | 10 |
| Beam 6 | 17 |
| Beam 5 | 21 |
| Beam 4 | 24 |
| Beam 3 | 24 |
| Beam 2 | 26 |
| Beam 1 | 26 |

Table 2: SIR at 50-percentile of CCDF of Figure 4

### 2.2.2 Realistic Formfactor Smart-Phone (Practical Case)

In this section, we present our simulation results considering realistic formfactor smart-phone UE. Our electromagnetic simulations of a realistic smartphone Mechanical Computer Aided Design (M-CAD) assumes that front and rear surfaces of the smartphone are made up of glass, chassis is metallic and frame is made up of plastic (see Figure 5 and Table 3 below for more details). Furthermore, the UE is assumed to have one antenna module on each of its left, top, and right surfaces; each antenna module is a standalone uniform linear array of four single polarized antenna elements placed half a wavelength apart from each other.

Figure 5: Details of a realistic smartphone M-CAD

Table 3: Material parameters for electromagnetic simulations

Figure 6 shows the angular range over which each one of the 21 beams of a realistic formfactor smart-phone UE is dominant.

Figure 6: Angular range for dominant beams of a realistic formfactor smart-phone UE

Similar to Section 2.2.1, we consider the scenario where boresight beam of Antenna Array #1 is the dominant beam used for receiving the signals from AoA1 whereas the AoA2’s dominant beam must be selected from the remaining 14 beams of Antenna Array #2 and #3.

We present the plot of CCDF of SIR of AoA1 for a realistic formfactor smart-phone UE.

Figure 7: CCDF of AoA1’s SIR for a realistic formfactor smart-phone UE

In Figure 7, we notice that due to the change of the beam shapes caused by the surrounding components in the real mobile phone, the difference of CCDF of SIR become less compared with those based on the analysis of the ideal beams in Figure 4. However, we again observed that the performance of Array 2-Beam 7 and Array 3-Beam 7 is worst compared to the other 12 beams of Array 2 and Array 3 as the CCDF curves for these two beams is relatively located in the leftmost region. And, the SIRs corresponding to ‘Array 2-Beam 5’/‘Array 3-Beam 5’ and ‘Array 2-Beam 6’/‘Array 3-Beam 6’ are higher compared with that of ‘Array 2-Beam 7’/‘Array 3-Beam 7’. This is aligned with the previous results that the SIR increases as the angular separation between AoAs increases.

Observation 4: The SIR values corresponding to the 50th percentile of the CCDF curves of Figure 7 is shown in Table 2.

| Beam Index (Array 2/Array 3) | SIR at 50-percentile of CCDF (dB) |
| --- | --- |
| Beam 7 | 13 |
| Beam 6 | 15.5 |
| Beam 5 | 16 |
| Beam 4 | 17.5 |
| Beam 3 | 17.5 |
| Beam 2 | 18 |
| Beam 1 | 16 |

Table 4: SIR at 50-percentile of CCDF of Figure 9

Observation 5: It is observed from Table 2 and Table 4 that there is a significant difference (up to 16 dB in the ideal case and up to 5 dB in the practical case) between the SIR at 50-percentile of CCDF (dB) for different AoA offset scenarios. One RF requirement is not suitable to be used for all the AoA offsets, otherwise the requirement will be too relaxed at one offset or too stringent at another offset.

Proposal 2: RAN4 to define separate RF requirements for different AoA offsets (one requirement for each AoA offset) and the UE will be tested against the requirement(s) which correspond to the chosen AoA offset(s).

# Conclusions

Observation 1: There will be no inter-TRP/beam interference for the case when UE only supports “multiDCI-MultiTRP-r16” capability.

Observation 2: The interference from the TRP2 signals arriving from the AoA2 direction becomes significant when the angular separation between AoA1 and AoA2 is low.

Observation 3: The SIR values corresponding to the 50th percentile of the CCDF curves of Figure 4 is shown in Table 2.

| Beam Index (Array 2/Array 3) | SIR at 50-percentile of CCDF (dB) |
| --- | --- |
| Beam 7 | 10 |
| Beam 6 | 17 |
| Beam 5 | 21 |
| Beam 4 | 24 |
| Beam 3 | 24 |
| Beam 2 | 26 |
| Beam 1 | 26 |

Table 2: SIR at 50-percentile of CCDF of Figure 4

Observation 4: The SIR values corresponding to the 50th percentile of the CCDF curves of Figure 7 is shown in Table 2.

| Beam Index (Array 2/Array 3) | SIR at 50-percentile of CCDF (dB) |
| --- | --- |
| Beam 7 | 13 |
| Beam 6 | 15.5 |
| Beam 5 | 16 |
| Beam 4 | 17.5 |
| Beam 3 | 17.5 |
| Beam 2 | 18 |
| Beam 1 | 16 |

Table 4: SIR at 50-percentile of CCDF of Figure 9

Observation 5: It is observed from Table 2 and Table 4 that there is a significant difference (up to 16 dB in the ideal case and up to 5 dB in the practical case) between the SIR at 50-percentile of CCDF (dB) for different AoA offset scenarios. One RF requirement is not suitable to be used for all the AoA offsets, otherwise the requirement will be too relaxed at one offset or too stringent at another offset.

Proposal 1: RAN4 to clarify whether the agreement “No need to discuss and define the RF requirement for the UEs only supporting “multiDCI-MultiTRP-r16” means:

- No new requirements needed to be defined and the legacy requirements will be individually applied for testing AoA1 and AoA2, OR

- No RF requirements at all will be applicable for this scenario meaning this case will not be tested.

Proposal 2: RAN4 to define separate RF requirements for different AoA offsets (one requirement for each AoA offset) and the UE will be tested against the requirement(s) which correspond to the chosen AoA offset(s).

# References

[1]	RP-223527, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

[2]	R4-2306604 “WF on FR2-1 UERF requirements for FR2 multi-Rx”, Vivo, Apple