3GPP TSG-RAN WG4 Meeting #107 					                  R4-2307254

Incheon, KR, 22 - 26 May, 2023

Source:	China Telecom

Title:	Remaining issues for NR Multi-carrier enhancements

Agenda Item:	8.24.2.1

Document for:	Discussion

- Introduction

In RAN4 #107, the WF on NR Multi-carrier enhancements was approved in [1], the LS to RAN1 and RAN2 was approved in [2].

This contribution discusses the remaining open issues for UL Tx switching across 3/4 bands.

- Discussion

- 2.1  Exact location of switching period

The following was captured in the WF:

- For Rel-18 Tx switching, further discuss the following options taking into account RAN1 response:

  - Option 1: the switching period immediately precedes the time T0 at which a transmission starts on a carrier following a Tx switch from a preceding transmission

    - Option 1a: the switching period ends at time T0

  - Option 2: define the time relation of the switching period location and T0 for two cases with the switching period located on the switch-from carrier and switch-to carrier respectively.

    - The switching period location is determined based on the following RAN1 agreement:

The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

  - Option 3: No need to define the time relationship between the location of the switching period and T_0 in RAN4 specification

As we discussed in the last meeting, for Rel-18 Tx switching across 3 or 4 bands, we are fine to define the exact location of switching period. However, if any new agreement on the exact switching period location will be reached, it should not conflict with the following RAN1 agreement on the configuration of switching period location. With these, we are ok with option 2.

- RAN1 agreement: The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, define the time relation of the switching period location and T0 for two cases with the switching period located on the switch-from carrier and switch-to carrier respectively, and the definition of exact switching period location should not conflict with RAN1 agreement on the configuration of switching period location.

- 2.2	 Advanced optional UE ability for switching case across three bands, i.e., switch between {1T, 1T, 0T} and {0T, 0T, 2T}

The following was captured in the WF:

- Further discuss the following options in the next meeting

  - Option 1: For the switching scenario without ambiguity, i.e., the two Tx chains are on the same band before or after switching (switch between {1T, 1T, 0T} and {0T, 0T, 2T}), introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1).

    - Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2.

    - Option 1a: apply the same UE capability for this scenario and for the following scenario agreed in the previous meetings. (MTK, China Telecom, ZTE)

RAN4 LS approved in R4-2303507:

When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged during UL switching as follows:

- Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination.

  - Option 2: Not introduce advanced optional UE ability

For Tx switching between {1T, 1T, 0T} and {0T, 0T, 2T}, we support to introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1), where Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively and Tswitch_1 < Tswitch_2.

In addition, we support option 1a to apply the same UE capability for this scenario and for the scenario approved in LS R4-2303507 (with details shown above). For the two scenarios, the UE implementations for the advanced UE ability are similar, so the same UE capability with the same granularity can be used.

Proposal 2: For Tx switching between {1T, 1T, 0T} and {0T, 0T, 2T}, introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1).

- Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively and Tswitch_1 < Tswitch_2.

- Apply the same UE capability with the same granularity for this scenario and for the scenario approved in RAN4 LS in R4-2303507.

- 2.3  2-layer UL-MIMO support for carrier(s) capable of 2Tx

The following was captured in the WF:

- Further discuss in the next meeting:

  - Option 1: Mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx

    - DCM: RAN2 agreed that UL MIMO capability is used for UE to indicate 2Tx support. It means that UE need to indicate UL MIMO capability if UE wants to indicate 2Tx capability.

  - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

  - Option 2: Not mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx

We support to mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx, considering the following reasons:

- The support of 2-layer UL-MIMO is beneficial for UL throughput performance.

- The additional complexity of supporting 2-layer MIMO at transmitter side is quite moderate.

- In Rel-16/17 Tx switching, 2-layer UL-MIMO is mandatory supported for the carrier(s) capable of 2Tx.

- As commented by NTT DOCOMO in the last meeting, for RAN2, it was already agreed that UL MIMO capability is used for UE to indicate 2Tx support, and it means that UE need to indicate UL MIMO capability if UE wants to indicate 2Tx capability. So, we need to keep consistency between RAN2 and RAN4 specifications.

Proposal 3: Mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx, considering: 1) UL throughput benefit, 2) moderate complexity increase, 3) specification consistency with the Rel-16/17 Tx switching, 4) specification consistency with RAN2 for Rel-18 Tx switching.

- 2.4  Band configurations for Rel-18 Tx switching

The following was captured in the WF:

- Further discuss the necessity of introducing the band configurations for UL Tx switching across 3 and 4 bands in the specification in the next meeting.

Firstly, as we commented in the last meeting, there is no BC specific requirement for the support of Rel-18 Tx switching. For the use of Tx switching across 3/4 bands, the generic Rel-18 Tx switching requirements (i.e., CR endorsed in RAN4 #106) and the existing CA/SUL BC specific requirements are sufficient. In the endorsed CR for Rel-18 Tx switching, it clearly says:

For each uplink band pair, according to the capability [uplinkTxSwitching-OptionSupport],

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with one UL band on band X or band Y shall be satisfied;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with two uplink bands shall be satisfied.

Let us take 3-band CA configuration of band A+B+C as example. Assuming switchedUL is supported for band pair A+B, dualUL is supported for band pairs of A+C and B+C, in addition to generic Rel-18 Tx switching requirements, UE shall apply the following existing BC specific requirements:

–	DL A+B+C with UL A, DL A+B+C with UL B, and the fallback CA configurations (for switchedUL band pair A+B)

–	DL A+B+C with UL A+C, and the fallback CA configurations (for dualUL band pair A+C)

–	DL A+B+C with UL B+C, and the fallback CA configurations (for dualUL band pair B+C)

Secondly, the band combinations supporting Rel-18 Tx switching are up to UE implementation and reporting, which is the same as Rel-16/17 Tx switching. We see no issue to follow this Rel-16/17 approach for Rel-18 Tx switching. In contrast, adding band combinations for Rel-18 Tx switching in the specification will obviously increase unnecessary RAN4 workload for every release.

Thirdly, when there are necessary BC specific RF requirements to be discussed and specified, we support to discuss them per BC basis and capture the agreement in the specification. For example, for DL interruption, we need to discuss the feasibility of mandating no DL interruption per BC basis, and so it is necessary and meaningful to introduce the band configurations in the specification from DL interruption perspective.

Proposal 4: The band combinations supporting Rel-18 Tx switching across 3/4 bands are up to UE implementation and reporting, and there is no need to introduce the band configurations for Rel-18 UL Tx switching in RAN4 specification, considering that:

- No BC specific requirement for Rel-18 Tx switching is needed. For the use of Tx switching across 3/4 bands, the generic Rel-18 Tx switching requirement (i.e., CR endorsed in RAN4 #106) and the existing CA/SUL BC specific requirements are sufficient.

- The band combinations supporting Rel-16/17 Tx switching are up to UE implementation and reporting.

Proposal 5: When there are necessary BC specific RF requirements to be discussed and specified, e.g., DL interruption due to Tx switching, we support to discuss the requirement per BC basis and capture the agreement in the RAN4 specification.

- Conclusion

The contribution discussed the remaining open issues for UL Tx switching across 3/4 bands, with the following proposals:

Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, define the time relation of the switching period location and T0 for two cases with the switching period located on the switch-from carrier and switch-to carrier respectively, and the definition of exact switching period location should not conflict with RAN1 agreement on the configuration of switching period location.

Proposal 2: For Tx switching between {1T, 1T, 0T} and {0T, 0T, 2T}, introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1).

- Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively and Tswitch_1 < Tswitch_2.

- Apply the same UE capability with the same granularity for this scenario and for the scenario approved in RAN4 LS in R4-2303507.

Proposal 3: Mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx, considering: 1) UL throughput benefit, 2) moderate complexity increase, 3) specification consistency with the Rel-16/17 Tx switching, 4) specification consistency with RAN2 for Rel-18 Tx switching.

Proposal 4: The band combinations supporting Rel-18 Tx switching across 3/4 bands are up to UE implementation and reporting, and there is no need to introduce the band configurations for Rel-18 UL Tx switching in RAN4 specification, considering that:

- No BC specific requirement for Rel-18 Tx switching is needed. For the use of Tx switching across 3/4 bands, the generic Rel-18 Tx switching requirement (i.e., CR endorsed in RAN4 #106) and the existing CA/SUL BC specific requirements are sufficient.

- The band combinations supporting Rel-16/17 Tx switching are up to UE implementation and reporting.

Proposal 5: When there are necessary BC specific RF requirements to be discussed and specified, e.g., DL interruption due to Tx switching, we support to discuss the requirement per BC basis and capture the agreement in the RAN4 specification.

- References

- R4-2306621, WF on Rel-18 Tx switching across 3/4 bands, China Telecom, RAN4 #106-bis-e, Apr 2023.

- R4-2306623, LS on Rel-18 Tx switching across 3/4 bands, China Telecom, RAN4 #106-bis-e, Apr 2023.