3GPP TSG-RAN WG4 Meeting # 107		R4-2307279

Incheon, KR, May 22 – May 26, 2023

Source: 		Qualcomm Incorporated

Title: 	Conditions of UE multiple Rx-beam based DL reception

Agenda item:	8.8.3.1

Document for:		Approval

# Introduction

In this paper, we present our view on conditions of UE multiple Rx-beam based DL reception.

# Discussion

There should be no question that UE capable of simultaneous beams will not always form dual beams. Depending on radio channel condition and NW configurations, UE may opportunistically enable dual Rx beams. For instance, there are some specific NW configurations from which UE can infer it is already in or in the course of dual-Rx based mTRP mode, e.g. configurations of TCI list for PDSCH reception, CORESET, Group-based L1-RSRP, etc. If NW configurations and radio channel condition are such that UE does not expect an mTRP operation mode anytime soon, RAN4 minimum requirements should not force the UE to enable dual Rx beams all the time. On the other hand, we do not think the status of UE beam needs to be reported to NW explicitly. And we do not see the need of dictating UE beam management via a separate signaling. As to the agreement of “enhanced RRM requirements based on faster beam sweeping with multi-Rx chains,” the enhancement, if introduced and feasible, should be considered in a case by case manner and the conditions for the enhancement should be precisely defined without a separate signaling.

### Indication of multi-Rx operation Way forward: - Proposals - Option 1: (Nokia, Huawei, LGE, Intel, ZTE, vivo, Samsung, Apple) - Introduce mechanism/condition for indication of multi-Rx operation, including on/off indication of multi-Rx operation. - FFS on mechanism/condition for indication of multi-Rx operation - Option 2: (MTK, OPPO) - No new mechanism is needed for UE to fallback from multi-Rx to single Rx. ### Beam sweeping/Sharing/Scaling factor reduction Agreement from GTW: - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities Way forward: - Please bring further analysis on the FFS parts of the above GTW agreement to the next meeting. Agreements: - How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage.

Proposal 1: RAN4 to not introduce signaling/indication based UE multi-Rx beam status report/control.

Proposal 2: Any enhanced RRM requirements by using multi-Rx chain based faster beam sweeping, if feasible and introduced, should be considered in a case by case manner, and the conditions for the enhancement should be precisely defined without a separate new signaling.

Proposal 3: RAN4 to not assume UE receives signals/channels from mTRP using simultaneously formed multiple Rx beams when any of the following conditions are not met:

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB from each TRP, e.g. SNR regime where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

- Note that the number of UE cell search engines should remain the same, irrespective of the number of active UE Rx beams.

# Conclusions

In this paper, we proposed the following.

Proposal 1: RAN4 to not introduce signaling/indication based UE multi-Rx beam status report/control.

Proposal 2: Any enhanced RRM requirements by using multi-Rx chain based faster beam sweeping, if feasible and introduced, should be considered in a case by case manner, and the conditions for the enhancement should be precisely defined without a separate new signaling.

Proposal 3: RAN4 to not assume UE receives signals/channels from mTRP using simultaneously formed multiple Rx beams when any of the following conditions are not met:

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB from each TRP, e.g. SNR regime where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

- Note that the number of UE cell search engines should remain the same, irrespective of the number of active UE Rx beams.