3GPP TSG-RAN WG4 Meeting # 107		R4-2307326

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.32.3

Source:	Apple

Title:	RRM for eRedCap

Document for:	Discussion

# Introduction

In RAN4#106-bis-e meeting, the WF on eRedCap was approved in [1], and there are open issues for further discussion. In this contribution we continue the discussion on the RRM impact for eRedCap UE.

# Discussion on Inactive eDRX cycle > 10.24 sec

When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements

The agreement and open issue was:

- Wait for the RAN2 decisions on the T value and PTW length design before defining serving cell measurements requirements

for the serving cell measurement requirement in Rel-17 RedCap, there is one difference between IDLE and INACTIVE states that the “T according to clause 7.1 in TS38.304” was only used for INACTIVE requirement, as shown below. We think the serving cell measurement requirement in Inactive mode with PTW shall also follow the same principle to use the T as the step size instead of DRX or eDRX cycle since the serving cell measurement behavior inside PTW window can also align with the PO reception.

In last RAN2 meeting, it was agreed that,

In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

Thus, when configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements, the requirement shall be based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304.

Proposal 1: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements, the requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304.

When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements

The agreement and open issue was:

The detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of: - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec - FFS if eDRX_Inactive 20.48sec

When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements, the measurement interval can be defined like for R17 IDLE case. In R17 the measurement/detection/evaluation delay is defined in the step of DRX cycle or eDRX cycle depending on whether the eDRX cycle length is above 10.24s or not, e.g., as shown in the note3 of following example. However, in Inactive mode, UE shall follow the Inactive eDRX configuration from RAN rather than IDLE eDRX configuration from CN.

Proposal 2: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements, the detection/measurement/evaluation delay requirement shall be specified in the step of:

- INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec

- DRX cycle, if eDRX_Inactive  20.48sec

When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements

The agreement and open issue was:

- Measurements/evaluation period shall be constrained to be within a single PTW irrespective of whether IDLE or INACTIVE eDRX PTW is being used. - Note: Wait for the RAN2 decisions on the T value and PTW.

RAN2 agreed in last meeting that,

Following cases are invalid: - Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle. - Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle. RAN PTW length is mandatorily present within Rel-18 INACTIVE eDRX’s configuration. Use the same UE_ID_H as IDLE eDRX for calculating the PH for RAN paging when INACTIVE eDRX is longer than 10.24s. Use TeDRX_RAN instead of TeDRX_CN to calculate the PH for RAN paging when TeDRX_RAN is longer than 10.24s.

Regarding the measurement inside PTW window, we think the legacy principle shall still be kept for Inactive long eDRX case, i.e., measurement and evolution period must be contained in one single PTW window (measurement/evaluation cross PTW windows is not expected).

Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, measurements/evaluation period shall be constrained to be within a single PTW irrespective of whether IDLE or INACTIVE eDRX PTW is being used.

When to measure when IDLE and INACTIVE eDRX PTW do not coincide

The agreement and open issue was:

RAN4 to wait for RAN2 progress to discuss following: - Whether to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding.

RAN2 agreed in last meeting that,

Following cases are invalid: - Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle. - Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle. In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

RAN2 concluded that within the INACTIVE eDRX PTW and outside IDLE eDRX PTW, UE shall follow the RAN configured DRX cycle to monitor the paging, however, RAN2 didn’t decide the validity of the case of “Within CN PTW and outside RAN PTW”, and RAN4 still need to wait for RAN2 progress.

Proposal 4: RAN4 still needs to wait for more RAN2 progress to decide the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding.

CG-SDT requirements with PTW

The agreement and open issue was:

CG-SDT requirements with PTW - FFS whether to define requirements for CG-SDT for Rel-18 eRedCap UE with PTW CG-SDT requirements with PTW: RSRP2 time range Following option is to be discussed after RAN4 has identified whether new requirements are to be specified for Rel-18 eRedCap with PTW: - RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2.

The requirements for Rel-17 RedCap CG-SDT did not consider PTW scenario either, and the new CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. In our view, some potential issues need to be addressed for CG-SDT with PTW, e.g., whether TA validation shall be contained in single PTW or can be cross PTWs; and whether SDT occasion can be used outside PTW window or not.

We have further clarified the details in last meeting:

- What type of new requirements are needed or why the legacy requirements can’t be reused?

For instance, the section 5.2B.2.1 in current TS38.133 is a dedicated section for SDT with RedCap, and the eDRX based SDT requirement is also in this section (Table 5.2B.2.1-3). But the current eDRX assumption in Table 5.2B.2.1-3 is based on eDRX<=10.24s, the long eDRX case has not been discussed.

Regarding ‘why legacy requirement can’t be used’, we would like to share one potential issue, but more details may still need FFS: In addition to the current condition for CG-SDT transmission (e.g., TA validation, RSRP threshold, buffer threshold, etc), do we allow initial SDT transmission or sub-sequent SDT transmission outside PTW window? At least we need to have some discussion to decide if some new clarification need to be added to the existing section.

- If eDRX and CG-SDT are independent features and can be configured independently, is there a need to define new requirements?

Yes, it’s needed, and general requirement was already defined in section 5.2B.2.1, but it only considered eDRX<=10.24s.

Proposal 5: new CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. Details are FFS.

# Conclusion

In this contribution we discuss the impact on RRM for the eRedCap UE.

Proposal 1: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements, the requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304.

Proposal 2: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements, the detection/measurement/evaluation delay requirement shall be specified in the step of:

- INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec

- DRX cycle, if eDRX_Inactive  20.48sec

Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, measurements/evaluation period shall be constrained to be within a single PTW irrespective of whether IDLE or INACTIVE eDRX PTW is being used.

Proposal 4: RAN4 still needs to wait for more RAN2 progress to decide the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding.

Proposal 5: new CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. Details are FFS.

# References

[1] R4-2306365, WF on R18 RedCap RRM requirements, Ericsson, RAN4 #106-bis-e