3GPP TSG-RAN WG4 Meeting # 107	R4-2307332

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.8.4.1

Source:	Apple

Title:	On General aspects for Multi-RX in FR2 demod requirements

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#106bis demodulation requirements for multi-RX in FR2 were discussed and way forward [1] was agreed.  In this contribution we present our views on general aspects of demodulation requirements for multi-RX reception on the DL.

# 2. Discussion

On the general aspects of Multi RX FR2 demod the following agreements were made:

Issue 1-1-2: whether to assume correlation matrix approach for simulation and requirements derivation only. - Agreement: - Assume correlation matrix approach for simulation and requirements derivation only. Issue 1-1-3: Whether to place the test probes at offset associated with the assumed correlation for OTA setup. - Agreement: - Place the test probes at offset associated with the assumed correlation for OTA setup. Issue 1-1-4: Whether to Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup. - Agreement: - Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup Issue 1-1-5: Best beam pair selection for demodulation performance requirements. - Agreement: - Wait for agreement in RF and/or RRM session on metric for best beam pair selection and discuss if applicable and feasible for demod setup. - UE must maintain the beam from each panel fixed during each multi-Rx performance test Issue 1-1-6: Whether to study cross-talk between TRPs after Rx beamforming is achieved. - Agreement: - Consider cross-talk between TRPs captured via the correlation matrix as being discussed under Issue 1-1-1 Issue 1-1-7: Assumption on UE Antenna module. - Agreement: - Don’t explicitly include antenna module placement in demod discussion; can be considered through correlation modelling under Issue 1-1-1 (if needed). Issue 1-1-8: Whether to study applicability rule for UE panel deployment. - Agreement: - Don’t consider applicability rule for UE panel deployment. Issue 1-1-9: Whether to consider MCS reduction for IFF method (if agreed for demodulation requirement). - Agreement: - Discuss later once achievable MCS is agreed. Issue 1-1-10: Delta AoA assumption for single DCI - Agreement: - Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies. Issue 1-1-11: Delta AoA assumption for multi- DCI fully overlapping. - Agreement: - Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies. Issue 1-1-12: Whether to introduce requirements for both single-DCI and multi-DCI scenarios. - Agreement: - Keep the current agreement of transmissions schemes and layer combinations for evaluation for companies to provide initial simulation results. Consider both sDCI with SDM and mDCI with fully overlapping resources, prioritize mDCI with fully overlapping. Issue 1-1-13: Performance requirement for multi- DCI non-overlapping. - Agreement: - Define at least one set of performance requirements for multi-DCI non-overlapping (if agreed to introduce requirements). Issue 1-1-15: Whether to study isolation level between two parallel TRP-to-RX links for 2x2 independent UE processing. - Agreement: - No need to consider isolation level separately. The isolation level between two parallel TRP-to-Rx links are inherently considered as part of the correlation model discussed under Issue 1-1-1. Issue 1-1-16: PN model and PN compensation. - Agreement: - Leave PN impact and modelling to companies’ implementation, the impact can be considered into results submitted by companies and the impact on test parameters i.e., MCS selection subject to companies’ results. Issue 1-1-17: Assumption on FFT window. - Agreement: - Consider single FFT window as baseline assumption for FR2 multi-Rx demodulation requirements definition.

On Correlation Matrix

For demodulation requirements TDL channel is used. The performance would be impacted by the TRP separation and UE implementations. Examples of different TRP separations are shown in Figure 1 from [2].

Figure 1: Example TRP offsets 60o, 120 o, 150 o

Examples of different UE implementations are shown in Figure 2.

Figure 2: Example UE implementations

The following proposals were captured in [1] for correlation matrix:

Issue 1-1-1: On  new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements.

<Way forward >

- Proposals:

- Option 1:

  - Use RH approach to generate OTA chamber internal H matrix for M-Rx 2 layer LLS simulations defined as:
RH= 1ɳ1*ɳ1|ɳ1|2ɳ2*1ɳ1ɳ2*ɳ1ɳ2ɳ2*ɳ11ɳ1*|ɳ2|2ɳ2ɳ2*1 ⊙RH, where ⊙ is the elementwise product

  - Define requirements using the following configurations of AoA in the OTA chamber for 2-layer (1+1) case, with corresponding relative sidelobe antenna gains of:

- AoA30: ɳ_1= ɳ_2= 0.1

- AoA90: ɳ_1= ɳ_2= 0.01

- AoA150: ɳ_1= ɳ_2= 0.03

- Option 2:

    - Reuse the current MIMO correlation matrix calculation methods defined in RAN4 spec to generate correlation matrix for each TRP-to-RX MIMO channel matrix Hi,j. Channel matrices from different TRPs to a Rx chain, i.e., Hi,1 vs Hi,2, are assumed to be statistically independent.

  - MIMO channel matrices from a TRP to two Rx chains, i.e., Hi,1 vs Hi,2, can be correlated, and the correlation coefficient αB between two Rx chains can be categorized as low (independent), medium and high which may depend on the AoA offset between two beam pairs. FFS: exact value of αB.

As we agreed in previous meetings, we will use correlation matrix approach to model aspects of spatial separation between the TRP, UE panel placements, analog beam forming in link level simulation using TDL channel model for demod performance evaluation and requirements definition.

The final correlation matrix R is applied to H to generate Hcorr as follows:

Hcorr=R12*vec(H)										         (1)

H is the composite channel:

H=H1 H2= H11H12H21H22 									         (2)

Figure 3: Multi-TRP Multi-RX channel modeling

The correlation matrix R has 2 components - the Polarization correlation matrix and the Beam forming correlation matrix.

R=Rpol⊙RBF											     (3)

Observation #1: The correlation matrix R has 2 components - the Polarization correlation matrix and the Beam forming correlation matrix. R=Rpol⊙RBF

The Polarization correlation matrix considers the cross-polarization and the inter-panel correlation. Let’s consider the cross-pol antenna model across single TRP and UE.

Observation #2: The polarization correlation matrix considers the cross-polarization and the inter-panel correlation.

Figure 4: Single TRP to UE Model

The UE correlation matrix across panels is –

RUE= 1ββ*1											         (4)

Where b is the cross-panel coupling factor. The coupling factor b should be based on antenna radiation pattern and the array factor of the panel in addition to UE implementation/panel placement.

Observation #3: The inter-panel correlation matrix is  RUE= 1ββ*1

Observation #4: The cross-panel coupling factor b is based on the antenna radiation pattern, array factor of the panel and the UE panel implementation/placement.

The channel matrix for single TRP-UE is H=h11h12h21h22h31h32h41h42

The correlation matrix for single TRP1 is -

R1=EvecH*vecHH=Γ1⊗RUE⊙Γ2	     		       				         (5)

Where G1 is the cross-pol correlation matrix for TRP1, Panel1, represented by

Γ1=10-γ10010γ1-γ10100γ101									         (6)

G2 is introduced to accommodate for different cross-pol correlation per UE panel, and defined as:

Γ2=111111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ11111					         (7)

Now, R1 can be defined as:

R1=1β00-γ1-βγ100β*100-β*γ2-γ200001β00γ1γ1β00β*100βγ2γ2-γ1-β001β00-β*γ2-γ200β*10000γ1βγ1001β00β*γ2βγ200β*1			     		         (8)

Observation #5: For a single TRP the corelation matrix is defined as R1=Γ1⊗RUE⊙Γ2
	G1 is the cross-pol correlation matrix for TRP1, Panel1
	G2 is introduced to accommodate for different cross-pol correlation per UE panel

Observation #6: The cross-pol matrices G1 G2 are defined as- 	
	Γ1=10-γ10010γ1-γ10100γ101    and    Γ2=111111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ11111

The terms g1, g2 depend on UE panel implementation/ orientation.

Observation #7: The terms g1, g2 depend on UE panel implementation/ orientation.

The polarization correlation matrix for 2TRPs can be extended as:

Rpol=PΓ1⊗RUE⊙Γ2P00PΓ3⊗RUE⊙Γ4P 		     		       		       (9)

Where P is a permutation matrix to reorder the channel coefficients to group the co-polarized elements together as below:

H=h11h12h13h14h31h32h33h34h21h22h23h24h41h42h43h44			     		       		      		     (10)

Observation #8: The polarization correlation matrix for 2TRPs can be extended as:

Rpol=PΓ1⊗RUE⊙Γ2P00PΓ3⊗RUE⊙Γ4P

Where Rpol=EvecH*vecHH and

vecH=[h11h31h21h41h12h32h22h42h13h33h23h43h14h34h24h44]

The above channel matrix is sufficient for non-beamformed channels.

In order to account for the beamforming at the UE, we can assume the beamformed channels to be described in the terms of the non-beam formed channels as:

vecHBF=h11 h31 h21 h41 h12 h32 h22 h42 h13 h33 h23 h43 h14 h34 h24 h44				(11)

vecHBF=ph11 ph31 qh21 qh41 ph12 ph32 qh22 qh42 rh13 rh33 sh23 sh43 rh14 rh34 sh24 sh44	(12)

Where p, q, r, s are the terms to model the gains from beam forming from the main lobe and leakage terms from side lobes.

Figure 5: 2 TRP to UE model

Observation #9: The terms p,q,r,s model the gains from beam forming from the main lobe and leakage from side lobes.

HBF=ph11ph12rh13rh14ph31ph32rh33rh34qh21qh22sh23sh24qh41qh42sh43sh44 								(13)

RBF=EvecHBF*vecHBFH									(14)

(15)

The coefficients p, q, r, s are dependent on the TRP separation (AoA offset).

Observation #10: The terms p,q,r,s model depend on the TRP separation.

RBF=kron(RTRP1-UE00RTRP2-UE,1111)							(16)

Observation #11: The beamforming correlation matrix s defined as RBF=kron(RTRP1-UE00RTRP2-UE,1111), where RTRP1-UE and RTRP2-UE are defined based on p,q,r,s in eqn 15.

Further study is needed to determine the coefficients described to define the correlation matrix. We propose to use this model to determine the coefficients to define correlation model.

Proposal #1: Use the proposed model and further study how to determine the coefficients to define the correlation matrix.

On Receiver assumption

The baseline receiver for multi-TRP in FR2 is agreed as MMSE-IRC. But the UE processing is still FFS.

Issue 1-1-14: Receiver assumption.

<Way forward >

- MMSE-IRC receiver

  - Option 1: 2x2 Per TRP

  - Option 2: Jointly processing as 4x4

  - Companies are encouraged to further evaluate the performance, UE processing complexity impact on above options.

For the UE processing assumption, we think more study and evaluation is needed before we decide on the baseline assumption. The multi-TRP transmission scheme and conditions could be used to determine whether to use per TRP link processing or joint 4x4 processing.

Observation #12: The multi-TRP transmission scheme and conditions could be used to determine whether per TRP link processing or joint 4x4 processing is used.

We should keep both options open and evaluate performance for different AoA offset for different mTRP schemes. Evaluate the following with both processing options:

- mDCI fully overlapping

- sDCI SDM

For mDCI non overlapping we don’t think we need to evaluate both and can only use per TRP link processing.

Proposal #2: Evaluate both per TRP link and joint processing for mDCI fully overlapping and sDCI SDM transmission schemes.

Proposal #3: Use per TRP link processing for mDCI non-overlapping case.

# 3. Conclusion

In this paper, we provide our views on general aspects of demodulation requirements for multi-RX reception on the DL. Our observations and proposals are captured below:

On Correlation Matrix

Observation #1: The correlation matrix R has 2 components - the Polarization correlation matrix and the Beam forming correlation matrix. R=Rpol⊙RBF

Observation #2: The polarization correlation matrix considers the cross-polarization and the inter-panel correlation.

Observation #3: The inter-panel correlation matrix is  RUE= 1ββ*1

Observation #4: The cross-panel coupling factor b is based on the antenna radiation pattern, array factor of the panel and the UE panel implementation/placement.

Observation #5: For a single TRP the corelation matrix is defined as R1=Γ1⊗RUE⊙Γ2
	G1 is the cross-pol correlation matrix for TRP1, Panel1
	G2 is introduced to accommodate for different cross-pol correlation per UE panel

Observation #6: The cross-pol matrices G1 G2 are defined as- 	
	Γ1=10-γ10010γ1-γ10100γ101    and    Γ2=111111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ111111111γ2/γ1γ2/γ11111111111111111γ2/γ1γ2/γ11111

Observation #7: The terms g1, g2 depend on UE panel implementation/ orientation.

Observation #8: The polarization correlation matrix for 2TRPs can be extended as:

Rpol=PΓ1⊗RUE⊙Γ2P00PΓ3⊗RUE⊙Γ4P

Observation #9: The terms p,q,r,s model the gains from beam forming from the main lobe and leakage from side lobes.

Observation #10: The terms p,q,r,s model depend on the TRP separation.

Observation #11: The beamforming correlation matrix s defined as RBF=kron(RTRP1-UE00RTRP2-UE,1111), where RTRP1-UE and RTRP2-UE are defined based on p,q,r,s in eqn 15.

Proposal #1: Use the proposed model and further study how to determine the coefficients to define the correlation matrix.

On Receiver Assumption

Observation #12: The multi-TRP transmission scheme and conditions could be used to determine whether per TRP link processing or joint 4x4 processing is used.

Proposal #2: Evaluate both per TRP link and joint processing for mDCI fully overlapping and sDCI SDM transmission schemes.

Proposal #3: Use per TRP link processing for mDCI non-overlapping case.

# Reference

- R4-2306000, “WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception”, Qualcomm.

- R4-2219852, “System Parameter Assumptions for Multi AoA Rx Testing”, Keysight Technologies.