3GPP TSG-RAN4 Meeting #107	R4-2307350

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.8.3.6

Source:	Apple

Title:   	On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#10bis, receive timing difference between different directions was discussed [1], with the following agreement.

Issue 3-1-1: Whether to consider RTD larger than CP in multi-RX WI

Agreed to postpone the discussion of MRTD > CP until RAN4#107.

Issue 3-1-2: Others

Way forward: Please bring further analysis on the below proposal.

- RAN4 to study impact of receive timing differences from mTRPs on beam pair selection for group-based beam reporting

Issue 3-1-3: MRTD for UE not supporting RTD>CP

Agreed to postpone the discussion of MRTD > CP until RAN4#107.

In this contribution, we share our views.

# 2	Discussion

MRTD > CP

As stated previously in [2], there are multiple reasons to not consider supporting MRTD > CP in this WI, particularly considerable impact on UE implementation. Nevertheless, if RAN4 decides to consider MRTD > CP, we would like to highlight the standards impact besides impact on UE implementation, as shown below:

- The value of MRTD in the network that the UE has to support. Will there be just one MRTD (>CP) value or multiple values from the network deployment point of view?

- UE capability: this is related to the possible MRTD values in the network

- Scheduling restriction: due to the MRTD impact, the scheduling restriction may affect the current symbol the UE needs to measure.

- Support of 4 layer MIMO: with MRTD > CP, is it possible to support 4 layer MIMO in the DL? What is the impact on group-based beam reporting?

- How can the UE know the actual MRTD in the network? Will the network configure some RSs for the UE to measure the arrival time difference?

Proposal 1: To consider MRTD larger than CP, there are many issues related to standards impact to consider, besides the UE implementation impact:

- The value of MRTD in the network

- UE capability

- Scheduling restriction

- Support of 4-layer MIMO

- How can the UE know the actual MRTD in the network?

Impact of MRTD on beam pair selection for group-based beam reporting

For the UE to report a suitable pair of beams based on group-based beam reporting, the UE needs to measure both RSRP and RS-SINR. In the case where MRTD > CP, the UE measurement is expected to be affected due to the changing interference estimation. For example, when the UE measures RS from one TRP, due to MRTD, it may have to evaluate the interference from the other TRP for the symbols before and/or after RS symbols. However, detailed analysis should only be carried out after the decision that MRTD > CP needs to be supported.

Proposal 2: There is expected to be impact on beam pair selection due to different MRTD. Detailed analysis should be discussed when RAN4 makes a decision on whether to support MRTD > CP case.

Finally, due to the considerable impact on UE implementation, together with the large standards impact of introducing support of MRTD > CP, we propose:

Proposal 3: Whether UE should support receive timing difference larger than CP as an optional capability can be part of R19 scope discussion when RAN starts to discuss the R19 RAN4 package.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: To consider MRTD larger than CP, there are many issues related to standards impact to consider, besides the UE implementation impact:

- The value of MRTD in the network

- UE capability

- Scheduling restriction

- Support of 4-layer MIMO

- How can the UE know the actual MRTD in the network?

Proposal 2: There is expected to be impact on beam pair selection due to different MRTD. Detailed analysis should be discussed when RAN4 makes a decision on whether to support MRTD > CP case.

Proposal 3: Whether UE should support receive timing difference larger than CP as an optional capability can be part of R19 scope discussion when RAN starts to discuss the R19 RAN4 package.

# 4	References

R4-2306319, "WF on FR2_multiRx_part2," Ericsson

R4-2300273, "On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception," Apple