3GPP TSG-RAN WG4 Meeting #107	R4-2307356

Incheon, South Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	L3 part enhancement for FR2 SCell activation

Agenda item:	8.9.2.1

Document for:	Discussion

Introduction

At RAN4#106bis-e meeting, the report of L3 measurement result was discussed and LS is sent to RAN2 for the detailed signaling design. The WF for FR2 unknown SCell activation enhancement is captured in [1]. In this contribution, we will continue the discussion on the issues remaining to be discussed in RAN4.

Discussion

When the details of the report after SCell activation command is up to RAN2 decision, RAN4 need continue the discussion on the cases if the UE does not have a valid measurement result to report. And the following activation step after the L3 report needs to be clarified.  The timeline for SCell activation procedure with such UE reporting is illustrated in Fig.1.

Fig.1 Timeline for FR2 unknown SCell activation

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available?

Regarding to the L3 report after SCell activation command, it is agreed that L3 measurement result with SSB index is reported to network. We believe it is reasonable to report a valid L3 measurement result of the SCell, which is equivalent to the L3 measurement report for activating a known SCell. Although “valid” has been used in existing spec, it is unclear what “valid” means in terms of measurement performance. In our understanding, the L3 measurement is considered valid only if it fulfills the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-4.

Proposal 1: The report of L3 measurement result after SCell activation command needs to be valid.

Proposal 2: The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2).

Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results?

FFS:

- Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

- Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation.

- Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network.

- Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’.

- Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell.

It is also possible the UE may not have a valid L3 measurement result at the time of SCell activation command. This may happen e.g. when the SCell is blindly added to the CA list or the UE has not measured the SCell sufficiently. The UE behaviour is FFS in this case.

As the measurement status (e.g. if there is a valid result at UE) is unknown to the network, once the reporting after SCell activation command is configured, the UE is expected to acquire the UL reporting resources to send the valid L3 result. This allows the UE to continue measurements after receiving SCell activation command if the UE has some un-complete measurements. Depending on the number of samples the UE is required to obtain in order to provide a valid measurement result, the UE may derive a valid result hence avoid falling back to legacy unknown SCell activation.

Proposal 3: The UE is allowed to continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of SCell activation.

Issue 1-1-8: FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported.

This valid L3 measurement report after SCell activation command provides network the DL beam information of the SCell which is equivalent to the latest L3 measurement reporting in a known SCell. Therefore, the following activation steps can be the same as activating a known SCell hence AGC, cell search and L1-RSRP measurement are not needed. The TCI activation command and SP/P-CSI-RS activation command are based on the valid L3 measurement report after SCell activation command.

Observation #1: If the UE sends a valid L3 measurement report after SCell activation command, AGC, cell search and L1-RSRP measurement are not needed.

Proposal 4:  The unknown SCell shall be activated as if it is known and existing SCell activation delay requirement applies, if the UE sends a valid L3 measurement report after SCell activation command.

Proposal 5: The TCI and SP/P-CSI-RS activation commands are based on the valid L3 measurement report after SCell activation command, if the report is sent after SCell activation command.

In R17 fast SCell activation, A-TRS can be triggered if the UE has sent a valid measurement report for the SCell within a certain time period i.e. the SCell is known. Similarly, the network can also trigger A-TRS based on the received valid L3 measurement report after SCell activation to speed up the AGC and time/frequency synchronization. In this case, A-TRS for fast SCell activation in R17 can be activated in TCI activation command.

Proposal 6: A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command.

Beam sweeping factor reduction

In last meeting, it was agreed the beam sweeping factor capability X1/X2 are indicated independently. It is FFS if the value of X1 can be set to zero (The value of X2 is discussed in L1 part enhancement [2]).

Issue 1-2-2: can X1(Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation) be zero? (not related with WI of FR2 multi-Rx chain DL reception)

The discussion is related with the previous agreement: Agreement in last meeting R4-2303228

Issue 1-3-3: enhancement of “8*Trs” part of current FR2 unknown Scell activation delay Agreement: - To keep “X1*Trs” part of current FR2 unknown Scell activation delay in the delay requirement for FR2 Scell activation enhancment. - X1 can be less than 8 in the beam sweeping factor reduction discussion for cell detection stage in L3 part based on UE capability.

FFS:

- Option 1 (Apple, MTK, Xiaomi, HW (option 4b in issue 1-2-1 comments), Intel, QC, OPPO): no

- Option 2 (Ericsson, vivo, Nokia, HW (option 4a in issue 1-2-1 comments)): Yes

In our understanding, UEs may have different implementation during the overall AGC and cell detection phase. Some UEs may be able to detect the cell within the AGC period while some UEs need additional time for cell detection after AGC settling. As the UE is able to indicate X1/X2 separately, there seems no need to prevent the UE from indicating zero-value X1 if it is capable of doing so.

Proposal 7: X1 can be zero to allow for different UE implementation.

Others for L3 part

On the following open issue, we see it beneficial to use SSB periodicity instead of SMTC periodicity in terms of reducing the SCell activation delay. There was concern the SMTC periodicity is widely used in other procedures. To solve this issue, we can limit the applicability to FR2 SCell activation only.

Issue 1-5-1: whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation

Agreement (GTW, Tuesday, Apr 25, 2023)

- FFS whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation

  - Option 1:

    - Do not change the definition/usage of SMTC periodicity for FR2 unknown Scell activation requirements.

  - Option 2:

    - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity.

    - Such enhancement is only applied for FR2 unknown SCell activation.

Proposal 8: To use SSB periodicity instead of SMTC periodicity for FR2 unknown SCell activation.

Conclusion

This contribution discusses the L3 part enhancement for FR2 SCell activation delay reduction. The observations and proposals are summarized as below:

Proposal 1: The report of L3 measurement result after SCell activation command needs to be valid.

Proposal 2: The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2).

Proposal 3: The UE is allowed to continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of SCell activation.

Observation #1: If the UE sends a valid L3 measurement report after SCell activation command, AGC, cell search and L1-RSRP measurement are not needed.

Proposal 4:  The unknown SCell shall be activated as if it is known and existing SCell activation delay requirement applies, if the UE sends a valid L3 measurement report after SCell activation command.

Proposal 5: The TCI and SP/P-CSI-RS activation commands are based on the valid L3 measurement report after SCell activation command, if the report is sent after SCell activation command.

Proposal 6: A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command.

Proposal 7: X1 can be zero to allow for different UE implementation.

Proposal 8: To use SSB periodicity instead of SMTC periodicity for FR2 unknown SCell activation.

References

- R4-2306315, WF on R18 eFeRRM, Apple

- R4-2307357, L1 part enhancement for FR2 SCell activation, Nokia, Nokia Shanghai Bell