3GPP TSG-RAN WG4 Meeting #107	R4-2307359

Incheon, South Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on PUCCH SCell activation delay

Agenda item:	5.2.10.3

Document for:	Discussion

Introduction

At RAN4#106 meeting, the following was agreed on the PL-RS measurement delay [1]. In this contribution, we propose the number of samples for PL-RS measurement and some corrections to PUCCH SCell activation delay.

Issue 1-1-1: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)

- Agreement:

  - When PL-RS of target PUCCH SCell is known, the X=[4] sample measurement time is always considered

    - The above conclusion only applies to PL-RS evaluation in PUCCH SCell activation requirements

Discussion

According to the agreement, additional PL-RS measurement is considered when activating a PUCCH SCell. As the UE is required to perform intra-frequency measurements on the deactivated SCell, the UE is likely to have some measurement samples at the time of SCell activation. We are fine to allow additional measurement on the PL-RS possibly based on the refined beam, but it should not take long time to avoid unnecessarily extending the SCell activation delay. In our view, 1 sample is enough for PL-RS evaluation in PUCCH SCell activation delay requirement.

Proposal 1: 1 sample is enough for PL-RS evaluation in PUCCH SCell activation delay requirement, i.e. X=1.

In addition, the following was agreed in RAN4#103-e to allow parallel processing of PL-RS measurement, DL CSI-RS reception/processing and UL TA acquisation, and the proposed update is captured in TS38.133 clause 8.3.12. However, the principle of parallel processing is reflected only when the UE does not have a valid TA. This shall be applicable also to the valid TA case.

RAN4#103-e Agreements:

- Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”.

  - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after

  - Proposed update: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after

Proposal 2: The parallel processing of PL-RS measurement and DL CSI-RS reception/processing shall be also applied to the valid TA case.

The CR is provided to address above issues in [2]. It is proposed to agree on the CR.

Proposal 3: Agree on the changes in the CR R4-2307360 to address above issues.

Conclusion

This contribution discusses the other enhancements for FR2 SCell activation delay reduction. The observations and proposals are summarized as below:

Proposal 1: 1 sample is enough for PL-RS evaluation in PUCCH SCell activation delay requirement, i.e. X=1.

Proposal 2: The parallel processing of PL-RS measurement and DL CSI-RS reception/processing shall be also applied to the valid TA case.

Proposal 3: Agree on the changes in the CR R4-2307360 to address above issues.

References

- R4-2303157, WF on FeRRM PUCCH Scell activation requirements maintenance, Apple

- R4-2307360, 38.133 CR on PUCCH SCell activation delay requirement, Nokia, Nokia Shanghai Bell