3GPP TSG-RAN WG4 Meeting #107                                                                R4-2307393

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on NCR feature list

Source: 	CATT

Agenda item:	8.29.1.2

Document for:	Approval

- Background

This contribution provides our views on NCR feature list.

- Discussion

2.1 NCR-Fwd feature list

In last RAN4 meeting, there was some comment that feature list for NCR-Fwd may also need to be discussed. From our understanding, feature list is for UE and some NW node with UE function such as IAB-MT, so for NCR-Fwd which is a repeater with some control information, there’s no feature list because there’s no capability report. So we have the following proposal.

Proposal 1: There’s no feature list for NCR-Fwd.

2.2 NCR-MT feature list

For NCR-MT feature list, there was an agreement in the WF [1] to focus on the mandatory UE capability firstly. In [9], we had a proposal that IAB-MT R15 feature list agreement can be considered as the starting point. We still have the same consideration that IAB-MT R15 feature list can be reviewed for the reference for NCR-MT. The followings are the feature list agreements for IAB-MT [2] [3],

- 1-1: Optional

- 1-2, 1-3: Mandatory without capability signaling

- 1-4, 1-5 : Optional

- 1-6, 1-7, 2-14: Not applicable

- 1-8: Not applicable

- 2-10:[Optional]

- 2-13, 2-15: Not applicable.

- Band and/or band combination related features including EN-DC,CA and SUL : 1-9, 1-10, 2-2, 2-3, 2-4, 2-5, 2-6, 2-9, 2-16 and 2-17. and 7.5kHz UL raster shift (1-11): TBD

After review for NCR-MT, we have the following proposal for NCR-MT.

Proposal 2: Table 1 can be taken as a starting point for further discussion of NCR-MT feature list.

Table 1: Preliminary views on NCR-MT feature list

| Features | Index | Feature group | Mandatory/Optional for UE | Mandatory/Optional for R18 NCR-MT |
| --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | Optional with capability signalling | Optional |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | Mandatory without capability signalling | Mandatory without capability signalling |
|  | 1-3 | 64QAM for PUSCH | Mandatory without capability signalling | Mandatory without capability signalling |
|  | 1-4 | 256QAM for PDSCH | Mandatory with capability signalling for FR1 | Optional |
|  |  |  | Optional with capability signalling for FR2 | Optional |
|  | 1-5 | 256QAM for PUSCH | Optional with capability signalling (for both FR1 and FR2) | Optional |
|  | 1-6 | pi/2-BPSK for PUSCH | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Not applicable |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Not applicable |
|  | 1-8 | Active BWP switching delay | Mandatory to support either type 1 or type 2 with capability signalling | Not applicable |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Optional with capability signalling | FFS |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | FFS |
|  | 1-11 | 7.5kHz UL raster shift | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | FFS |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | Based on declaration, LS to other group may be needed |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. | FFS |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 | FFS |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 | FFS |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | FFS |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Optional with capability signalling | FFS |
|  | 2-7 | Almost contiguous UL CP-OFDM | Optional with capability signalling | Not applicable |
|  | 2-8 | UE power class | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class | Based on declaration, LS to other group may be needed |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 | FFS |
|  | 2-10 | Multiple frequency band indication | Mandatory without capability signalling | Optional |
|  | 2-11 | Modified MPR behaviour | Optional with capability signalling | Not applicable |
|  | 2-12 | Multiple NS/P-Max | Mandatory without capability signalling | Not applicable |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Optional with capability signalling. The capability signalling is absent if UE supports 50% | Not applicable |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Optional with capability signalling | Not applicable |
|  | 2-15 | Maximum uplink duty cycle for FR2 | Optional with capability signalling | Based on declaration, LS to other group may be needed |
|  | 2-16 | PA architectures for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. | Not applicable |
|  | 2-17 | PA architectures for intra-band UL CA | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture | Not applicable |

- Summary

This contribution provides our view for NCR feature list. We have the following proposals.

Proposal 1: There’s no feature list for NCR-Fwd.

Proposal 2: Table 1 can be taken as a starting point for further discussion of NCR-MT feature list.

Reference

[1] R4-2305391, “WF on system parameters and RF requirements”, RAN4#107, ZTE

[2] R4-2005606, “WF on IAB-MT Features”, RAN4#94 Bis-e, Qualcomm

[3] R4-2005608, “Draft LS on RAN4 IAB-MT feature list agreement”, RAN4#94 Bis-e

[4] R4-2009067, “WF on IAB-MT feature list”, RAN4#95e, CATT

[5] R4-2009051, “LS on RAN4 IAB-MT feature list agreement”, RAN4#95e, Qualcomm

[6] R4-2012563, “LS to RAN2 on IAB-MT feature list”, RAN4#96e

[7] R4-2100019, “Reply LS from RAN2 (R4-2100019)”, RAN2

[8] TS 38.822

[9] R4-2304438, “Discussion on NCR-MT feature list”, RAN4#104b, CATT