3GPP TSG-RAN WG4 Meeting # 107	            R4-2307400

# Incheon, KR, May 22th – May 26th, 2023

Agenda Item:	8.25.2.3

Source: 	CATT

Title: 	Discussion on L1/L2 inter-cell mobility delay requirements

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on L1/L2 inter-cell mobility was approved. In this contribution [1], some open issues on L1/L2 inter-cell mobility delay requirements are further discussed.

2. Discussion。

Detail of cell switch delay requirements for Pcell/PSCell

Issue 3-3-5: T/F fine tracking: TΔ and Tmargin

< Way forward >: FFS the following options

  - Option 1: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. FFS: whether TΔ and Tmargin can be 0 under certain conditions.

    - Option 1a: TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

    - Option 1b: TΔ and Tmargin can be 0 under the condition that DL synchronization for candidate cell(s) has been performed before cell switch command.

    - Option 1c: When TCI state to use for the target cell is already in the active TCI state list, it is also necessary for the UE to meet the Te requirement for an initial transmission, that is, at least one SSB is available at the UE during the last 160ms, then TΔ and Tmargin can be 0.

    - Option 1d: TΔ = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command

    - Option 1e: TΔ = 0 for the case that SSB based fine synchronization for candidate cell(s) is performed before cell switch command

  - Option 2: FFS

RAN1 already support synchronization before cell switch command. However, we believe that even if pre DL-sync is applied, it may be necessary to perform T/F fine tracking after receiving a cell switch command, which means that TΔ may not necessarily be 0.

For the certain conditions where TΔ and Tmargin can be 0, considering the purpose of TΔ, and we believe that it is necessary for the UE to meet the Te requirement for an initial transmission [2], that is, if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms, then the UE does not need to perform T/F fine tracking on subsequent initial transmission.

Proposal 1: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

Issue 3-3-8: Whether to define RACH-based cell switch delay requirements for unknown cell

< Way forward >:

  - Not define RACH-based cell switch delay requirements for unknown cell case.

  - FFS wait for RAN1 progress

Proposal 2: Not define RACH-based cell switch delay requirements for unknown cell case.

Issue 3-3-9: Whether to define PCell/PSCell switch delay requirements for unknown TCI state case when TCI state is indicated together with cell switch command

< Way forward >: FFS the following options

  - Option 1 (MTK, vivo, Huawei, CATT, OPPO): When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

  - Option 2 (Nokia): When TCI state is indicated together with cell switch command, define cell switch delay requirements for both known and unknown TCI state case

  - Option 3 (Ericsson): FFS

We prefer option 1, it is not reasonable for NW to indicate an unknown TCI state to UE. Though the TCI state is unknown at UE due to poor channel state, it is better to perform RACH-based cell switch [2].

Proposal 3: When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

Issue 3-3-10: Whether to define RACH-less cell switch delay requirements for unknown cell

< Way forward >:

  - Not define RACH-less cell switch delay requirements for unknown cell case.

  - FFS wait for RAN1 progress

Proposal 4: Not define RACH-less cell switch delay requirements for unknown cell case.

Issue 3-3-14: Tinterruption

< Way forward >: FFS the following options

  - Option 1: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd

  - Option 2: There is almost no interruption during cell switch procedure when target Pcell/SCell is current SCell/PCell.

  - Option 3:

    - For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU.

    - For RACH-less cell switch, T_interruption at least include T_processing,2

We support option 1. For option 3, considering Tsearch, TΔ , Tmargin can be 0 under certain conditions, so for RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU, and for RACH-less cell switch, Tinterruption at least include Tprocessing,2.

Proposal 5: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 6:

- For RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU.

- For RACH-less cell switch, Tinterruption at least include Tprocessing,2

3. Conclusion

In this paper, we provide our views on L1/L2 inter-cell mobility delay requirements. From this discussion we have derived the following proposals:

Proposal 1: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

Proposal 2: Not define RACH-based cell switch delay requirements for unknown cell case.

Proposal 3: When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

Proposal 4: Not define RACH-less cell switch delay requirements for unknown cell case.

Proposal 5: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 6:

- For RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU.

- For RACH-less cell switch, Tinterruption at least include Tprocessing,2

4. Reference

- R4-2306395 WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2306249 Topic summary for [106-bis-e][218] NR_Mob_enh2_part1, MediaTek Inc.