3GPP TSG-RAN4 Meeting #107	                                                            R4-2307411

Incheon, KR, May 22 – May 26, 2023

Title:	Reply LS on measurement definitions for positioning with bandwidth aggregation

Source:	CATT

Agenda Item:	8.23.3.1

Document for:	Approval

1 Introduction

In RAN1#112bis-e meeting, RAN1 discussed the PRS/SRS bandwidth aggregation and reached the following conclusion.

Conclusion The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same. - Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers, - Note: no RAN1 spec impact - Send an LS to RAN4 to confirm RAN1’s understanding

A LS was sent to RAN4 to confirm the understanding. In this paper, we discussed the issue and give our proposals. Based on the discussing, the draft response LS is provided in the Annex.

2 Discussion

Based on the objective on PRS/SRS bandwidth aggregation in the WID, R18 will define the support on aggregation of PRS/SRS resources across PFLs/carriers based on the following assumption:

- Up to three intra-band contiguous carriers

- The signals are transmitted/received using a single RF chain (same antenna)

- Apply to only timing related measurements

- Specify the requirements for measurement with gap in connected mode

- Specify the requirements for the measurement in inactive mode

We understand the intention of PRS/SRS bandwidth aggregation is to perform one measurement based on the aggregated PRS/SRS across multiple PFLs to improve the performance. With that, only one timing measurement result is reported for the aggregated resources, so the timing for multiple PFLs should be assumed as the same. That’s why we studied the feasibility in the SI and define the conditions (e.g. using single RF chain) to guarantee the timing alignment among PFLs. So we think the PRS/SRS aggregation doesn’t change the legacy definition but to perform the legacy measurement on aggregated resources, i.e. RAN1’s understanding is correct.

Proposal 1: RAN1’s understandings on measurement definitions for positioning with bandwidth aggregation are correct.

3 Summary

In this paper, we discuss the RRM aspects on PRS/SRS bandwidth aggregation and the following proposals are given:

Proposal 1: RAN1’s understandings on measurement definitions for positioning with bandwidth aggregation are correct.

4 References

- R4-2307005 (R1-2304082) LS on measurement definitions for positioning with bandwidth aggregation, ZTE (RAN1), RAN4#107

Annex Draft reply LS to RAN1

3GPP TSG-RAN4 Meeting #107	                                                            R4-23XXXX

Incheon, KR, May 22 – May 26, 2023

Title:	Reply LS on measurement definitions for positioning with bandwidth aggregation

Response to:	R4-2307005 (R1-2304082)

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:                  Qiuge Guo

E-mail Address:   guoqiuge@CATT.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks RAN1 for the LS (R1-2304082) on measurement definitions for positioning with bandwidth aggregation. RAN4 has discussed the issues and confirms that RAN1’s understandings are correct.

2. Actions:

To RAN WG1:

RAN4 kindly asks RAN1 to take the above information into account.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #108		August 21 – August 25, 2023		Toulouse	, FR

TSG RAN WG4 Meeting #108bis		October 9 – October 13, 2023		Xiamen, CN