3GPP TSG-RAN WG4 Meeting#107					R4-2307434

22nd May, 2023 – 26th May, 2023

Source: 	Qualcomm Incorporated

Title: 	8Rx for CPE/FWA/vehicle/industrial devices: Demodulation requirements

Agenda item:	 8.5.3.1

Document for:	Approval

# 1 Introduction

In this paper, our proposals on the open items mentioned in WF of #106_bis_online [1] are shared.

# 2 PDSCH Demodulation Requirements

## 2.1 Propagation conditions

Issue 2-3: Propagation conditions for Rank 2 test

- Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B

- Option 2: Only TDLC300-100 ULA Medium B

We prefer a single requirement for Rank2 case, and therefore propose only for the vehicular scenario.

Proposal 1: RAN4 to use only TDLC300-100 ULA Medium B for Rank 2 case.

## 2.2 MCS and antenna correlation for Rank 2 test

Issue 2-4: MCS and antenna correlation for Rank 2 test

- Proposals for TDLC300-100 ULA Medium B

  - Option 1: MCS 2, 7 (Table 2)

  - Option 2: MCS 19 (Table 1)

We think 64QAM table, used for fading conditions, is suitable for the vehicular case. Based on our studies, we propose MCS 19 of Table1

Proposal 2: RAN4 to use MCS 19 for 2 layers, 8Rx with TDLC 300-100, medium B PDSCH demod requirements.

2.3 MCS for Rank 4 test

Issue 2-5: MCS for Rank 4 test

- Option 1: MCS 13 and MCS26 (Table 1)

- Option 2: Only MCS17 (Table 1)

Based on simulation studies, MCS 17 of Table 1 is a suitable candidate.

Proposal 3: RAN4 to use MCS 17 for 4 layers, 8Rx PDSCH demod requirements

## 2.4  Single MCS configuration for Rank 8 test

Issue 2-7: Single MCS configuration for Rank 8 test

- Option 1:MCS13

- Option 2: MCS17

From simulation studies, MCS 13 achieves 70% of maximum throughput close to 20dB threshold.

Proposal 4: RAN4 to use MCS13 for 8 layers, 8Rx PDSCH demod requirements

# 3 SDR Requirements

Issue 3-4: Maximum MCS and MIMO layers for 256QAM

- 2 and 4 layers: reuse the requirements defined in Table 5.5A-5 of TS 38.101-4

- Candidate options for maximum achievable MCS for 8 layers

  - Option 1: MCS22

  - Option 2: MCS24

3.1 Maximum MCS with 8L 256 QAM

From simulation studies, MCS 26 achieves maximum throughput close to 30dB SNR threshold. Hence, MCS 24, a candidate option from WF, is proposed.

Proposal 5: RAN4 to use maximum MCS 24 (scaling factor =1) for 256 QAM, 8 layers, 8Rx SDR case

Issue 3-5: Maximum MCS and MIMO layers for 1024QAM

- Introduce SDR requirements for 1024QAM at least for 2 MIMO layers

- Candidate options

  - Option 1: Only 2 layers:

- Option 1a: MCS23

- Option 1b: MCS24

  - Option 2: Both 2 and 4 layers:

- Option 2a: MCS23

- Option 2b: MCS24

3.2 Maximum MCS with 2L and 4L 1024 QAM

From 2L simulation studies, MCS 25 achieves maximum throughput close to 30dB which is near 32 dB threshold for 2.5% Tx EVM. Therefore, we propose MCS24 for 2L 1024 QAM SDR case.

Proposal 6: RAN4 to use maximum MCS 24 (scaling factor =1) for 1024 QAM, 2 layers, 8Rx SDR case

From 4L simulation studies, MCS 24 achieves maximum throughput close to 32dB where TxEVM impacts the performance. Moreover, from spec. 38.101-4, for 2L, 4Rx case, MCS 23 is the requirement. Hence, we propose MCS 23 for 1024 QAM 4L case.

Proposal 7: RAN4 to use maximum MCS 23 (scaling factor =1) for 1024 QAM, 4 layers, 8Rx SDR case

# 4 CSI requirements

- Following options for i2 will be discussed for next RAN4 meeting:

  - Option 1: TE sets random i2 (1 out of 2 possibilities) during the test

  - Option 2: TE uses fixed value i2 = 0 or 1 during the test

  - Option 3: Leave TE implementation to use random i2 or fixed i2 = 0 or 1

4.1 i2 for CQI reporting

Although, the value of i2 does not affect the performance, we prefer to have a fixed value during the CQI reporting test.

4.2 SNR points

- Option 1: [4,5] dB and [10,11] dB

- Option 2: [7,8] dB for 64QAM, [1,2] dB for 16QAM

- Other options are not precluded

From simulation studies with i2=1, we support Option1.

Proposal 8: RAN4 to use (4,5) and (10,11) as SNR points in dB for 4L, 8Rx static channel CQI reporting.

# 5. Conclusions

Following is the summary of the proposals.

Proposal 1: RAN4 to use only TDLC300-100 ULA Medium B for Rank 2 case.

Proposal 2: RAN4 to use MCS 19 for 2 layers, 8Rx with TDLC 300-100, medium B PDSCH demod requirements.

Proposal 3: RAN4 to use MCS 17 for 4 layers, 8Rx PDSCH demod requirements

Proposal 4: RAN4 to use MCS13 for 8 layers, 8Rx PDSCH demod requirements

Proposal 5: RAN4 to use maximum MCS 24 (scaling factor =1) for 256 QAM, 8 layers, 8Rx SDR case

Proposal 6: RAN4 to use maximum MCS 24 (scaling factor =1) for 1024 QAM, 2 layers, 8Rx SDR case

Proposal 7: RAN4 to use maximum MCS 23 (scaling factor =1) for 1024 QAM, 4 layers, 8Rx SDR case

Proposal 8: RAN4 to use (4,5) and (10,11) as SNR points in dB for 4L, 8Rx static channel CQI reporting.

# References

[1] WF R4-2305888, “WF for 8Rx UE performance requirements_final”, 3GPP TSG RAN WG4 meeting, #106_bis_online, Apr. 17 – Aug 26, 2023.

[2] WF R4-2302942, “WF for 8Rx UE performance requirements_v8”, #106, 3GPP TSG RAN WG4 meeting, #106, Feb. 27 – Mar. 3, 2023.