3GPP TSG-RAN WG4 Meeting #107 	R4-2307472

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - General aspects

Agenda item:	8.8.4.1

Document for:	Discussion

Introduction

In RAN4#106-bis-e good progress was made on the general topic with several agreements, see WF [1] .

In the following we will provide our view on the remaining open issues and make new observations and proposals where needed.

We see the most important open issues to enable link level simulations being:

- New correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements

- Receiver assumption

- Provide with a generic approach that covers minimum requirements for different UE architectures, module configurations and module placement on a device without severe effective SNR range restrictions

Discussion

New correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements

In RAN4#106-bis-e, several proposals for the new correlation matrix were discussed. After the discussion, the two remaining proposed options were captured in the WF [1]

Issue 1-1-1: On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements. <Way forward > - Proposals: - Option 1: - Use RH approach to generate OTA chamber internal H matrix for M-Rx 2 layer LLS simulations defined as: RH= 1ɳ1*ɳ1¦ɳ1¦2ɳ2*1ɳ1ɳ2*ɳ1ɳ2ɳ2*ɳ11ɳ1*¦ɳ2¦2ɳ2ɳ2*1 ⊙RH, where ⊙ is the elementwise product - Define requirements using the following configurations of AoA in the OTA chamber for 2-layer (1+1) case, with corresponding relative sidelobe antenna gains of: - AoA30: ɳ_1= ɳ_2= 0.1 - AoA90: ɳ_1= ɳ_2= 0.01 - AoA150: ɳ_1= ɳ_2= 0.03 - Option 2: - Reuse the current MIMO correlation matrix calculation methods defined in RAN4 spec to generate correlation matrix for each TRP-to-RX MIMO channel matrix Hi,j. Channel matrices from different TRPs to a Rx chain, i.e., Hi,1 vs Hi,2, are assumed to be statistically independent. - MIMO channel matrices from a TRP to two Rx chains, i.e., Hi,1 vs Hi,2, can be correlated, and the correlation coefficient αB between two Rx chains can be categorized as low (independent), medium and high which may depend on the AoA offset between two beam pairs. FFS: exact value of αB.

In the following, we will further explain the Nokia proposal (option 1) on how to define a new correlation matrix to represent the OTA chamber with two antennae panels as well as how to incorporate such model into the existing RAN4 simulation model used for FR2 single TRP simulations. It is worth mentioning here that in our previous proposal we were modelling the OTA and beam alignment part, and it has since become evident that most companies would like to directly include the legacy TDL model. Consequently, to merge both the Legacy TDL model (controlled by external fading emulator) and the OTA and UE beam alignment part, the formulation of our derivation has been expanded and thus might look different to the previous one.

Legacy conductive test system

Initially, we represent the legacy conductive test system for FR1 requirements as illustrated in Figure 1, where each UE antenna port is connected to the test system via 50 Ohm cables.

The received signal can be modelled as:

Equation 1

y=HL x ,

where x is the transmitted signals from the TRPS, HL is the channel matrix (e.g. 2x2 or 4x4) and y is the received signal at the UE, (see also [2]) as this is legacy conductive methodology

Figure 1: legacy conductive system model block diagram for a 4x4 system

New Multi TRP radiated test system proposal

Hereafter, we represent the Multi TRP, Multi Rx test system for FR2 requirements as shown in Figure 2, where the UE is placed inside of the anechoic chamber and the test system is connected to two probes at different positions inside of that chamber.

In this case, as depicted in Figure 2, the received signal can be modelled as:

Equation 2

y=HOTA HL x ,

where x is the transmitted signals from the TRPs, HL is the same channel matrix (e.g. 2x2 or 4x4) as in the previous conductive setup, HOTA comprises the OTA chamber and UE (including spatial filters) and y is the received signal at the UE.

When considering legacy fading emulation, the received signal can be modelled as: y=HOTA HL x, where x is the transmitted signals from the TRPS, HL is the same channel matrix (e.g. 2x2 or 4x4) as in the previous legacy conductive setup, HOTA comprises the OTA chamber and UE (including spatial filters) and y is the received signal at the UE.

Figure 2: M-Rx demod OTA test system conceptual representation

Figure 3 translates the conceptual representation of the anechoic chamber depicted in Figure 2 to the mathematical notation used throughout this contribution.

Figure 3: Generalized system model based on the legacy full test system assumption with fader, chamber, and UE beams.

Thus, the generalized system model for the 2TRP-2RX (chains) configuration with up-to 4-layers can now be written as:

Equation 3

y1R1y2R1y1R2y2R2y=HOTAHLx1T1x2T1x1T2x2T2x,

where the superscript of x and y, i.e., Ti and Rj, respectively, refer to TX i, i∈1,2, and RX j, j∈1,2; and the subscripts of x and y, i.e, k, k∈1,2 and l,l∈1,2, denote the layer index from the corresponding transmitter TRP Ti and the antenna index at RX Rj, respectively.

The two H matrices in Equation 3 are the legacy channel matrix HL, which corresponds to the stochastic channel model (e.g., by using TDLA, TDLB, and etc.); and HOTA, which represents the channel inside the OTA chamber (after the beam-lock, i.e. the TX beam-RX beam pairing). Hence, HOTA will have deterministic values.

We can define HL to represent the stochastic channel model (e.g., TDLA, TDLB, etc.) and HOTA, to represent the deterministic channel inside the OTA chamber after the TX beam-RX beam pairing and consequent beam-lock.

In the following, we will detail both HL and HOTA by taking into account relevant factors impacting the proposed model, such as the well-known 3GPP transmit and receive correlation matrices.

HL derivation

In general, we can write the 4x4 channel matrix HL as follows:

Equation 4

HL=h11T1R1h12T1R1h21T1R1h22T1R1h11T2R1h12T2R1h21T2R1h22T2R1h11T1R2h12T1R2h21T1R2h22T1R2h11T2R2h12T2R2h21T2R2h22T2R2

where hmnToRp yields the channel coefficient of the link between transmitter n, n∈1,2, to receiver m,m∈1,2, for the corresponding transmit-receive pair ToRp,o,p∈1,2.

In the current 3GPP specification, the correlation matrix is only defined for one such transmit-receive pair. Therefore, to introduce the correlation matrix for the multiTRP, multiRX case, we need to decouple HL into submatrices which correspond to different links of TRP-RX pairs.

In the 2TRP 2RX case, there are 4 pairs of TRP-RX, such that o=p identifies the links of TRP-RX pairs for the intended signals, whereas o≠p gives the cross links for the interference signal. Hence, HL can be decoupled into 4 blocks of 2x2 dimension, wherein each such 2x2 submatrix corresponds to one of the 4 Tx-Rx pairs. As a result, Equation 4 can now be rewritten as

Equation 5

HL=HT1R1HT2R1HT1R2HT2R2, with

- HT1R1=h11T1R1h12T1R1h21T1R1h22T1R1,

- HT2R1=h11T2R1h12T2R1h21T2R1h22T2R1,

- HT1R2=h11T1R2h12T1R2h21T1R2h22T1R2,

- HT2R2=h11T2R2h12T2R2h21T2R2h22T2R2.

In RAN4 simulations, it is a common practice to use different correlation levels by employing distinct correlation matrices for the transmit and receive antennas.

Figure 4: Generalized system model based on the full test system assumption with fader, chamber, and UE beams with the correlated channel matrix HLC where γ represents the Inter-TRP relative contribution controlled outside of the OTA chamber.

Similar to the 3GPP specification, by employing distinct correlation matrices for the transmitter and receiver of each TRP-RX pair, as depicted in Figure 4 we now define the correlated channel matrix HLc as:

Equation 6

HLc=HcT1R1γHcT2R1γHcT1R2HcT2R2with the correlated submatrices (per TRP-RX pair):

- HcT1R1= RRX112HT1R1RTX112, and Rspat11= RTX1⨂RRX1;

- HcT2R1= RRX112HT2R1RTX212, and Rspat21= RTX2⨂RRX1;

- HcT1R2= RRX212HT1R2RTX112, and Rspat12= RTX1⨂RRX2;

- HcT2R2= RRX212HT2R2RTX212, and Rspat22= RTX2⨂RRX2.

where RTX1,RTX2and RRx1, RRx2 are the transmit correlation matrices of TX1 and of TX2 and the receive correlation matrices of RX1 and of RX2, respectively, which are tabulated in the 3GPP specifications. The values of RTXi,i∈1,2, can take any values of RgNB as defined in 3GPP and RRXj,j1,2, the corresponding values of RUE. The variable inter-TRP relative contribution (outside of the OTA chamber) is then controlled by γ.

We can define HLc=HcT1R1γHcT2R1γHcT1R2HcT2R2 as the correlated channel matrix where the coefficients of each realization of HLc are derived using “classical” TDL modelling on each of the for subblocks { HcT1R1, …, HcT2R2} and γ represents the Inter-TRP relative contribution controlled outside of the OTA chamber.

Equation 6 can be then rewritten in a more explicit form for the 2+2 case as:

Equation 7

HLc,2+2=hc11T1R1hc12T1R1hc21T1R1hc22T1R1γhc11T2R1γhc12T2R1γhc21T2R1γhc22T2R1γhc11T1R2γhc12T1R2γhc21T1R2γhc22T1R2hc11T2R2hc12T2R2hc21T2R2hc22T2R2,

For the 1+1 case, HLc,1+1 simply becomes h11T1R1γh11T1R2γh11T1R2h11T1R2 and the TDL correlation is scalar “1” for each of the four coefficients. This assumes the same polarization is used for each TRP/RX in the 1+1 case.

We propose to model the received signal as: y= HOTAHLcx, where HLc=HcT1R1γHcT2R1γHcT1R2HcT2R2, and γ represents the Inter-TRP relative contribution controlled outside of the OTA chamber.

We note that when using the classical covariance matrix notation for these channel coefficients under the system model from the proposal above, the spatial covariance matrix can be derived as follows, where the γ parameter is multiplicatively included in HLc(γ) on top of the coefficients derived from the classical TDL model. The (γ) is used here to emphasize the inclusion of γ and does not change the matrix from before.

Hall=HOTARRX100RRX2RRX-mTRPmRXHcT1R1γHcT2R1γHcT1R2HcT2R2RTX100RTX2RTX-mTRPmRX

vecHall=Rspat-mTRPmRX vecHLc(γ)

where 
Rspat-mTRPmRX=I4x4⨂HOTARTX-mTRPmRX⨂RRX-mTRPmRX

HOTA derivation

Equally important, HOTA is a deterministic matrix (for a given AoA probe set, a fixed UE orientation and fixed given UE beam selection), which is given as follows:

Equation 8

HOTA,2+2,generic=η11T1R1η21T1R1η12T1R1η22T1R1η11T2R1η21T2R1η12T2R1η22T2R1η11T1R2η12T1R2η12T1R2η22T1R2η11T2R2η12T2R2η21T2R2η22T2R2,

where each element represents the gain η of the corresponding links inside the chamber after the beam pairing process (see Figure 2). The superscript of η, namely, Ta, a∈1,2, and Rb,b∈1,2, denote the OTA chamber pair of transmit probe Ta and receive probe Rb. The subscript of η, namely, c, c∈1,2, and d,d∈1,2, denote the polarization of transmit and receive probe; in which if c=d they are the same polarization and if c≠d they are different polarization.

Moreover, it is worthy noticing the general formulation in Equation 8 also encompasses our previous option 1 proposal introduced in RAN4#106-bis-e and captured in the WF [1]. In fact, by adjusting the 2x2 block matrices to the 2-layer (1+1) OTA chamber configuration, we obtain 1x1 blocks as follows,

Equation 9

HOTA,1+1,generic=η11T1R1η21T2R1η12T1R2η22T2R2,

From previous simulation results for the 1-layer case; thereby, HOTA,1+1,generic can be rewritten as

Equation 10

HOTA,1+1=1ɳ2ɳ11,

Where in this 1+1 case, ɳ1 is the relative magnitude loss (dual polarisation transmission per probe and dual polarisation reception in our simulations) on second Rx panel as the first probe’s signal hits the sidelobe of second RX panel, and, similarly, ɳ2 is the loss when the second probe’s signal hits the sidelobe of the first RX panel.

Without loss of generality, it still possible to further particularize the general formulation in Equation 8 by using the actual correlation values collected in the anechoic chamber test setup of Figure 2 (or gathering simulation results under the same assumptions as detailed in Appendix A).

Based on similarities between ɳ values observed in our simulation results, the generalized HOTA,1+1 can be simplified by introducing the ɳ1, ɳ2 parametrization as HOTA,1+1=1ɳ2ɳ11, which can be further simplified by setting ɳ1=ɳ2 so that HOTA,1+1=1ɳ1ɳ11.

We propose a simplified parameterized version of HOTA,1+1 to be: HOTA,1+1=1ɳ1,ɳ11

Based on similarities between ɳ values observed in our simulation results, the generalized HOTA,2+2 can be simplified by introducing the ɳ1, ɳ2,ɳ3 parametrization as:
HOTA,2+2=1ɳ2ɳ21ɳ1ɳ3ɳ3ɳ1ɳ1ɳ3ɳ3ɳ11ɳ2ɳ21.

We propose a simplified parameterized version of HOTA,2+2 to be: HOTA,2+2=1ɳ2ɳ21ɳ1ɳ3ɳ3ɳ1ɳ1ɳ3ɳ3ɳ11ɳ2ɳ21.

Combining HLc and HOTA to obtain channel coefficients for one realization

By combining Equation 7 and Equation 8, we can now rewrite our general 2+2-case system model in Equation 3 as follows:

Equation 11

y1R1y2R1y1R2y2R2y=1ɳ2ɳ21ɳ1ɳ3ɳ3ɳ1ɳ1ɳ3ɳ3ɳ11ɳ2ɳ21.HcT1R1γHcT2R1γHcT1R2HcT2R2x1T1x2T1x1T2x2T2x

y= HOTA,2+2HLcx.

Equation 11 provides a general system model which already captures all components that is sufficient to model the simulator for multiTRP multiRX..

In the 3GPP standards, what is currently specified and available is the correlation matrix model for single-TRP single-RX case, which cannot be directly used in the multiTRP multi RX case.

Coefficients of deterministic HOTA

In order to propose numerical values for HOTA for the different probe placement and UE orientation in the chamber we have conducted a series of electromagnetic CST simulations shown in the appendix A. Based on those simulations we are proposing values for ɳ1,ɳ2 and ɳ3 elements as shown in Table 1and Table 2.

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ1 | 0.1 | 0.01 | 0.01 | 0.02 | 0.03 |

Table 1: proposed values for HOTA in the 1+1 case

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ2 | 0.04 | 0.025 | 0.04 | 0.06 | 0.09 |
| ɳ1 | 0.040 | 0.02 | 0.004 | 0.02 | 0.01 |
| ɳ3 | 0.005 | 0.004 | 0.004 | 0.01 | 0.005 |

Table 2: proposed values for HOTA in the 2+2 case.

It is worth noticing that the main contributor to the Inter-probe interference will be the n1 value, which is the strongest for AoA 30, the lowest for AoA 90 (due to quasi nulling effect) and the other AoA (60, 120, 150) have a similar n1. Also, the inter polarization coupling on desired TRP n2, tends to increase for larger AoA (120 and 150).

When deriving values for 1+1 we assume that both polarizations are used in both probes and both rx-chains, therefore there is approximately a factor related to the gain difference between the n1 values in 1+1 vs 2+2 which is fully as expected.

We propose the deterministic values of HOTA,1+1 for OTA probe AoA delta of 30, 60, 90, 120, 150 to be as in table:

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ1 | 0.1 | 0.01 | 0.01 | 0.02 | 0.03 |

We propose the deterministic values of HOTA,2+2 for OTA probe 30, 60, 90, 120, 150 to be as in table:

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ2 | 0.04 | 0.025 | 0.04 | 0.06 | 0.09 |
| ɳ1 | 0.040 | 0.02 | 0.004 | 0.02 | 0.01 |
| ɳ3 | 0.005 | 0.004 | 0.004 | 0.01 | 0.005 |

Best beam pair selection for demodulation performance requirements

In RAM4#106bis it was discussed how to select the best beam pair for demodulation performance requirements (see [1]):

Issue 1-1-5: Best beam pair selection for demodulation performance requirements. - Agreement: - Wait for agreement in RF and/or RRM session on metric for best beam pair selection and discuss if applicable and feasible for demod setup. - UE must maintain the beam from each panel fixed during each multi-Rx performance test

According to RF WF R4-2306604, the TRP to module pairing should use baseline Option1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all TRPs. The best of the other modules is assigned to track the other TRPs.

The agreement on TRP to module paring from WF R4-2306604 using “Option1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all TRPs. The best of the other modules is assigned to track the other TRPs.” as baseline is a good approach and can be modelled using HOTA

Receiver assumption

In RAN4#106bis the receiver assumption was discussed but no agreement made if 2x2 per TRP or joint processing as 4x4 should be used (see [1]):

Issue 1-1-14: Receiver assumption. <Way forward > - Proposals - MMSE-IRC receiver - Option 1: 2x2 Per TRP - Option 2: Jointly processing as 4x4 - Companies are encouraged to further evaluate the performance, UE processing complexity impact on above options.

The arguments for choosing one of the available options will mostly be based on the performance and the processing complexity. Joint processing allows the UE to process the received signals from the two panels jointly; and hence, the signal from the other TRP will not be treated as interference. How the performance of the two options differ, will strongly be defined by the level of signals from the other TRP received by the panels. The stronger the other TRP signal, the bigger the performance difference between joint detection (4x4) and per TRP (2x2) processing.

As shown in the value of eta from our simulation, it can be seen that there will be other TRP signal received by the panels, in which for 30 degrees of angular offset, the value is the highest. This implies that in real world, for smaller values of angular offset, the other TRP signal can be expected to become stronger. Hence, it is better to treat them as part of the useful signal by using joint detection, as compared to treating them as interference which needs to be cancelled.

Joint processing 4x4 shall be considered especially in scenarios where the angular offset is small and the inter-TRP interference is sufficiently high.

Conclusion

This paper presents Nokia's view on the open issues with relation to the general aspects for MultiRx Demodulation performance.

In the paper, the following Observations and Proposals were made:

New correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements

- When considering legacy fading emulation, the received signal can be modelled as: y=HOTA HL x, where x is the transmitted signals from the TRPS, HL is the same channel matrix (e.g. 2x2 or 4x4) as in the previous legacy conductive setup, HOTA comprises the OTA chamber and UE (including spatial filters) and y is the received signal at the UE.

- We can define HL to represent the stochastic channel model (e.g., TDLA, TDLB, etc.) and HOTA, to represent the deterministic channel inside the OTA chamber after the TX beam-RX beam pairing and consequent beam-lock.

- We can define HLc=HcT1R1γHcT2R1γHcT1R2HcT2R2 as the correlated channel matrix where the coefficients of each realization of HLc are derived using “classical” TDL modelling on each of the for subblocks { HcT1R1, …, HcT2R2} and γ represents the Inter-TRP relative contribution controlled outside of the OTA chamber.

- We propose to model the received signal as: y= HOTAHLcx, where HLc=HcT1R1γHcT2R1γHcT1R2HcT2R2, and γ represents the Inter-TRP relative contribution controlled outside of the OTA chamber.

We note that when using the classical covariance matrix notation for these channel coefficients under the system model from the proposal above, the spatial covariance matrix can be derived as follows, where the γ parameter is multiplicatively included in HLc(γ) on top of the coefficients derived from the classical TDL model. The (γ) is used here to emphasize the inclusion of γ and does not change the matrix from before.

Hall=HOTARRX100RRX2RRX-mTRPmRXHcT1R1γHcT2R1γHcT1R2HcT2R2RTX100RTX2RTX-mTRPmRX

vecHall=Rspat-mTRPmRX vecHLc(γ)

where 
Rspat-mTRPmRX=I4x4⨂HOTARTX-mTRPmRX⨂RRX-mTRPmRX

Based on similarities between ɳ values observed in our simulation results, the generalized HOTA,1+1 can be simplified by introducing the ɳ1, ɳ2 parametrization as HOTA,1+1=1ɳ2ɳ11, which can be further simplified by setting ɳ1=ɳ2 so that HOTA,1+1=1ɳ1ɳ11.

We propose a simplified parameterized version of HOTA,1+1 to be: HOTA,1+1=1ɳ1,ɳ11

Based on similarities between ɳ values observed in our simulation results, the generalized HOTA, 2+2 can be simplified by introducing the ɳ1, ɳ2,ɳ3 parametrization as:
HOTA,2+2=1ɳ2ɳ21ɳ1ɳ3ɳ3ɳ1ɳ1ɳ3ɳ3ɳ11ɳ2ɳ21.

We propose a simplified parameterized HOTA,2+2 to be: HOTA,2+2=1ɳ2ɳ21ɳ1ɳ3ɳ3ɳ1ɳ1ɳ3ɳ3ɳ11ɳ2ɳ21.

We propose the deterministic values of HOTA,1+1 for OTA probe AoA delta of 30, 60, 90, 120, 150 to be as in table:

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ1 | 0.1 | 0.01 | 0.01 | 0.02 | 0.03 |

We propose the deterministic values of HOTA,2+2 for OTA probe 30, 60, 90, 120, 150 to be as in table:

|  | AoA 30 | AoA 60 | AoA 90 | AoA 120 | AoA 150 |
| --- | --- | --- | --- | --- | --- |
| ɳ2 | 0.04 | 0.025 | 0.04 | 0.06 | 0.09 |
| ɳ1 | 0.040 | 0.02 | 0.004 | 0.02 | 0.01 |
| ɳ3 | 0.005 | 0.004 | 0.004 | 0.01 | 0.005 |

Best beam pair selection for demodulation performance requirements

The agreement on TRP to module paring from WF R4-2306604 using “Option1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all TRPs. The best of the other modules is assigned to track the other TRPs.” as baseline is a good approach and can be modelled using HOTA

Receiver assumption

Joint processing 4x4 shall be considered especially in scenarios where the angular offset is small and the inter-TRP interference is sufficiently high.

- References

- R4-2306000 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

- TS 38.508-1 V17.8.0 User Equipment (UE) conformance specification; Part 1: Common test environment

- Appendix A

This appendix contains our simulation results for the OTA chamber with different AoA and UE orientations. The configuration used are as follows:

The simulations are electromagnetic simulations based on a realistic smartphone Mechanical Computer Aided Design (M-CAD) with Front and Rear (glass material), chassis (metal material) and a frame (plastic material) as detailed in Figure 5 and Table 3. Four patches form a linear array for each antenna module. The two arrays considered in our simulations are Array#1 on the top and Array#2 on the right side of the device.

Figure 5: details on the MCAD details for the second type of simulations

Table 3: material parameters chosen for the second type of simulations

We have run simulations based on the AoA discussed in the RF OTA session (30, 60, 90, 120 and 150 degrees). Figure 6 shows the results of our simulations:

Figure 6 Simulation results for different AoA

The results have been colour-coded using the following legend where it is assumed that beam from Array#1 (H/V) is directed to probe1 (H/V) and beam from Array#2 (H/V) is directed to probe2 (H/V):

- Green:	Gain on incoming signal to the main lobe.

- Light blue:	Cross coupling of incoming signal.

- Orange:	Coupling from a probe to the sidelobe of opposite receiving array with the same polarisation (for example: probe1(H) to receiving array 2(H))

- Yellow:	Coupling from a probe to the sidelobe of opposite receiving array with the opposite polarisation (for example probe1(H) to receiving array 2(V)