3GPP TSG RAN WG4 Meeting #107	R4-2307487

# Incheon, KR, 22 - 26 May, 2023

Agenda item:	4.1 (UE maint. R16 / NR_n41_BW-Core)

Source:	SoftBank Corp.

Title:	Updates on PC1.5/NS_47 support of n41 for Japan

Document for:	Approval

1.  Introduction

Due to expected change of regulation in Japan to allow 29dBm in n41 in Japan, investigation was proposed on feasibility of NS_47 in [1]. This paper is to update the situations around the issue.

2.  Issues

2.1　Applicable versions

Since NS_47 is relevant to a regulatory requirement in Japan, ambiguity should be removed as much as possible. We asked chipset vendors whether this can be supported from Rel-16, the first release to introduce NS_47. So far, the responses are positive in general but, at the time of writing of this, we are still waiting for some responses. Then, we’d like to come back in August with a solid proposal/CRs.

[Proposal-1] Final CR(s) are postponed to August meeting.

2.2 　A-MPR

As far as the author recognizes, one result has already been presented [2] as a starting point. The result is copied below for reference. In addition, [3] mentioned a rough estimation and urged to complete this task ASAP for finalizing n3-n41 HP-UE.

Table 1: A-MPR for modulation and waveform type including PC1.5 (from [2])

| Modulation/Waveform | A1(dB) |  |  | A2(dB) |  |  | A3(dB) |  |  | A4(dB) |  |  | A5(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC3 | PC2 | PC1.5 | PC1.5 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 5.5 | ≤ 8.5 | ≤ 11.0 | ≤ 2 | ≤ 5 | ≤ 8.5 | ≤ 3 | ≤ 6 | ≤ 8.5 | ≤ 3.0 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 5.5 | ≤ 8.5 | ≤ 11.0 | ≤ 2 | ≤ 5 | ≤ 8.5 | ≤ 3 | ≤ 6 | ≤ 8.5 | ≤ 3.0 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 5.5 | ≤ 8.5 | ≤ 11.0 |  | ≤ 5 | ≤ 8.5 | ≤ 3 | ≤ 6 | ≤ 8.5 | ≤ 3.0 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 6 | ≤ 8.5 | ≤ 11.0 |  | ≤ 5 | ≤ 8.5 | ≤ 3 | ≤ 6 | ≤ 8.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 6 | ≤ 8.5 | ≤ 11.0 |  | ≤ 5 | ≤ 8.5 |  | ≤ 6 | ≤ 8.5 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 7 | ≤ 10 | ≤ 12.5 |  | ≤ 5 | ≤ 8.5 | ≤ 4 | ≤ 7 | ≤ 9.5 | ≤ 4.0 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 7 | ≤ 10 | ≤ 12.5 |  | ≤ 5 | ≤ 8.5 | ≤ 4 | ≤ 7 | ≤ 9.5 | ≤ 4.0 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 7 | ≤ 10 | ≤ 12.5 |  | ≤ 5 | ≤ 8.5 |  | ≤ 7 | ≤ 9.5 |  |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 13.0 | ≤ 7 | ≤ 10 | ≤ 12.5 |  |  | ≤ 8.5 |  | ≤ 7 | ≤ 9.5 |  |

While we expect further inputs in this meeting, we also expect the deviation from the final result would be minor since Table 1 has a similar tendency to other PC1.5 MPR/A-MPR: some values smaller/bigger but around 3dB relative to PC2 in general. Then, if HP-UE task is urgent, we’d like to propose to agree the values in this meeting. And if necessary and OK from HP-UE standpoint, we are fine to put the vales in [] for possible further adjustments.

[Proposal-2] If HP-UE of n3-n41 urgently needs, we try to agree the A-MPR values in this meeting.

2.3 　Domestic process

The public consultation process is underway on the modification of MOP, in addition to various aspects of “5G usage expansion” as in [4]. At present, the deadline of public comments is set to the 5th of June. It still seems unclear when the regulation is to be published.

3. Conclusion

This paper is intended for providing updates on the PC1.5 support in NS_47 context. Proposals are:

[Proposal-1] Final CR(s) are postponed to August meeting.

[Proposal-2] If HP-UE of n3-n41 urgently needs, we try to agree the A-MPR values in this meeting.

Reference

R4-2218527		On the support of n41 NS_47 with PC1.5, SoftBank, RAN4#105

R4-2300346		Consideration on PC1.5 for NS_47, Apple, RAN4#106

R4-2304579		NS_47 work needed for n41 PC1.5 1UL configuration in HPUE CA_n3-n41, Skyworks,

- RAN4#106b-e

https://www.soumu.go.jp/main_content/000879070.pdf or https://www.soumu.go.jp/menu_news/s-news/01kiban14_02000584.html (in Japanese only)