3GPP TSG-RAN WG4 Meeting #107	R4-2307495

# Incheon, South Korea, 22 – 26 May, 2023

Source: 	NEC

Title: 	TP to TS 36.181: Characteristics of the interfering signals

Agenda Item:	9.7.3.5

Document for:	Approval

Introduction

This contribution provides the text proposal to TS 36.181 annex for characteristics of the interfering signals.

Discussion

In the last meeting, RAN4 had a good progress to create TS 36.181 for satellite access node conformance testing. Text proposal for most clauses were provided and approved. However, during the discussion in the last meeting, it was realized that the annex for characteristics of the interfering signals, which is referred in the document, was still missing. In the Way Forward [1], it was agreed to introduce the annex.

It is proposed the characteristics of the interfering signals are the same as those defined in E-UTRA requirements. Therefore, corresponding annex in TS 36.141[2] was referenced but text on in-band/guard-band NB-IoT was removed, because in-band/guard-band NB-IoT is not supported..

Conclusions

Based on the above discussion, the following proposal was formulated:

Proposal 1: Approve the attached TP to TS 36.181.

References

[1] R4-2305936, WF on remaining issues for the TS 36.181 drafting, Huawei

[2] TS 36.141

Text proposal

--------------Start of text proposal-------------

Annex E (normative):
Characteristics of the interfering signals

For E-UTRA SAN, the interfering signal shall be a PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211[7]. Mapping of PUSCH modulation to receiver requirement are specified in table E-1.

Table E-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| In-channel selectivity | 16QAM |
| Adjacent channel selectivity and narrow-band blocking | QPSK |
| Blocking | QPSK |
| Receiver intermodulation | QPSK |

For NB-IoT standalone SAN, the interfering signal shall be a NPUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 10.1 of TS36.211[7]. Mapping of NPUSCH modulation to receiver requirement are specified in table E-2.

Table E-2: Modulation of the interfering signal – NB-IoT

| Receiver requirement | Modulation |
| --- | --- |
| Adjacent channel selectivity and narrow-band blocking | π/4 QPSK |
| Blocking | π/4 QPSK |
| Receiver intermodulation | π/4 QPSK |

--------------End of text proposal-------------