3GPP TSG-RAN WG4 Meeting #107	R4-2307497

# Incheon, South Korea, 22 – 26 May, 2023

Source: 	NEC

Title: 	Discussion on NCR class declaration

Agenda Item:	8.29.1.1

Document for:	Approval

Introduction

This contribution provides the text proposal to TS 36.181 for reference measurement channels.

Discussion

In the last meeting, RAN4 discussed on the NCR class and achieved the following agreement in the Way Forward [1].

- For NCR-fwd, following NCR classes introduced

  - DL/UE side: wide-area, medium range and local area

  - UL/BS side: Wide-area and local area

  - Above NCR classes applicable for all NCR types

- NCR classes for NCR-MT

  - [Wide-area and local area]

- FFS whether NCR class can be declared separately for NCR-fwd, and NCR-MT

In this contribution, we focus on the issue,

“FFS whether NCR class can be declared separately for NCR-fwd, and NCR-MT”

For NCR-fwd has BS side and UE side, before discussing whether NCR class can be declared separately for NCR-fwd, and NCR-MT, we want to confirm that NCR class can be declared separately for NCR-fwd BS side and NCR-fwd UE side. It comes from the above agreement that,

- For NCR-fwd, following NCR classes introduced

  - DL/UE side: wide-area, medium range and local area

  - UL/BS side: Wide-area and local area

  - Above NCR classes applicable for all NCR types

Applicable classes are different between NCR-fwd BS side and UE side. When NCR class of NCR-fwd UE side is medium range, there is no way to declare the same class for BS side. Above agreement on the NCR-fwd classes implicitly tells that NCR class can be declared separately for NCR-fwd BS side and NCR-fwd UE side. If it is not true, we will never be able to declare the medium range class for UE side.

Proposal 1:

Separate NCR class declaration for NCR-fwd BS side and NCR-fwd UE side shall be allowed.

NCR-MT has only BS side. Same discussion applies for NCR-fwd UE side and NCR-MT. It is straightforward that NCR class can be declared separately for NCR-MT and NCR-fwd UE side.

Proposal 2:

Separate NCR class declaration for NCR-MT and NCR-fwd UE side shall be allowed.

Remaining issue is,

Whether NCR class can be declared separately for NCR-fwd BS side and NCR-MT

To discuss this issue, we should see how the repeater classes are defined. Repeater classes for uplink are defined in [2] as below.

For repeater type 1-C and type 2-O, repeater uplink classes are defined as indicated below:

- Wide Area repeaters are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

- Local Area repeaters are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

From the definitions above, it is clear the repeater classes are defined based on the cell scenarios. Although the definitions are for Rel-17 NR repeaters, it is expected to have the same repeater uplink class definitions for NCR including type 1-H and type 1-O.

If we consider NCR-fwd BS side and NCR-MT are communicating with the same BS, cell scenarios for NCR-fwd BS side and NCR-MT shall be same. This leads to the conclusion that NCR classes for NCR-fwd BS side and NCR-MT shall have the same NCR class. Unless otherwise NCR-fwd BS side and NCR-MT are communicating with the different BSs, we do not think different NCR class declarations are required. As we do not think there are cases NCR-fwd BS side and NCR-MT are communicating with the different BSs, we do not think diffe

Because NCR-MT and NCR-fwd have links with the same BS, uplink cell scenarios for NCR-MT and NCR-fwd shall be always same and, as a result, repeater classes for NCR-MT and NCR-fwd uplink

Proposal 3:

Separate NCR class declaration for NCR-fwd BS side and NCR-MT is not required.

Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1:

Separate NCR class declaration for NCR-fwd BS side and NCR-fwd UE side shall be allowed.

Proposal 2:

Separate NCR class declaration for NCR-MT and NCR-fwd UE side shall be allowed.

Proposal 3:

Separate NCR class declaration for NCR-fwd BS side and NCR-MT is not required.

References

[1] R4-2305931, WF on system parameters and RF requirements, ZTE

[2] TS 38.106