3GPP TSG-WG RAN4 Meeting #107	R4-2307503

Incheon, Republic of Korea, 22-26 May, 2023

Source: 				Nokia, Nokia Shanghai Bell

Title: 				LTE based 5G UE RF open issues

Agenda Item: 	9.3.3

Document for:	Approval

# Introduction

In this contribution, open issues on UE Refsens and ACS requirements are discussed according to the agreed WF [1] in RAN4#106-bis-e as shown below.

# 2. Refsens - The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with Option 1. 6, 7, and 8 MHz PMCH for reference sensitivity Option 2. 10 MHz PDSCH for reference sensitivity Way forward: FFS these options # 3. ACS Option 1: 33 dB (for all 6, 7, and 8 MHz bandwidths) Option 2: 29 dB (for all 6, 7, and 8 MHz bandwidths) Option 3: Channelization; Power in transmission bandwidth configuration; PInterferer; ACS. 10 MHz (baseline); REFSENS + 14 dB; REFSENS + 45.5; 33 dB. 6 MHz; REFSENS + 14 dB; REFSENS + 28.5; [16] dB. 7 MHz; REFSENS + 14 dB; REFSENS + 30.0; [17.5] dB. 8 MHz; REFSENS + 14 dB; REFSENS + 35.0; [22.5] dB. Way forward: FFS these options

# Discussion

Physical channel and bandwidth

The channel bandwidth 6, 7, or 8 MHz depends on the countries and regions. For the UE certification purpose in each region, it would be necessary that each channel bandwidth is testable with the channel used in each region. Even though 5G broadcast may be supported by smartphones that also support LTE channels like PDSCH, a standalone receiver only device cannot be excluded. It is not possible to use the existing PDSCH requirement and conformance test for such device, as it requires uplink for testing. Thus, for each channel bandwidth a PMCH reference measurement channel would be required for testing.

It is proposed that all the channel bandwidths, 6, 7 and 8 MHz, are specified with downlink only PMCH for all the receiver requirement for 5G broadcast. However, we may not need to test all channel bandwidths. The support of all channel bandwidths may not be mandatory if 5G broadcast receiver product only targets a specific region.

Proposal 1: For all receiver requirement, all the channel bandwidths, 6, 7 and 8 MHz, are specified with downlink only PMCH.

Observation 1: Conformance test may not be required for all the three channel bandwidths depending on the region.

ACS

So far ACS level [-16 to -33] dB has been discussed for 6/7/8 MHz channel. With 16 dB ACS, at least uncoordinated SFN deployment scenario would not be possible as cellular deployment usually assumes 33 dB ACS. Due to lack of coexistence study for HPHT scenario, it is not clear if 16 dB ACS is tolerable for uncoordinated deployment of TV channels. The ACS level 16 dB likely discouraging the adjacent channel deployment but only the 2nd adjacent channel, which is not optimum from spectrum efficiency point of view. Therefore, we propose that RAN4 agree on at least 29 dB ACS.

Proposal 3: RAN4 agrees at least ACS = 29 dB for 6, 7 and 8 MHz channel bandwidth.

# Conclusion

Proposal 1: For all receiver requirement, all the channel bandwidths, 6, 7 and 8 MHz, are specified with downlink only PMCH.

Observation 1: Conformance test may not be required for all the three channel bandwidths depending on the region.

Proposal 3: RAN4 agrees at least ACS = 29 dB for 6, 7 and 8 MHz channel bandwidth.

# Reference

[1] R4-2306566 WF on RF requirements for 5G Broadcast, Qualcomm, RAN4#106-bis-e