3GPP TSG-RAN WG4 Meeting # 107		R4-2307597

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.14.5.3

Source: 	CMCC

Title: 			 Simulation results for ATG PUSCH demodulation

Document for:	Discussion

- Introduction

After RAN4#106bis-e meeting, companies are encouraged to bring evaluation results [1]. In this contribution, we provide our simulation results and simulation assumption.

- Simulation results

Simulation results

Our simulation results of FR1 FDD cases are provided in the table below

| FDD 15kHz AWGN+200Hz doppler typeA |  |  |  |
| --- | --- | --- | --- |
| Antenna | Bandwidth (MHz) | MCS | SNR（70%TP） |
| 1T2R | 5 | 10 | 5.18 |
|  |  | 13 | 8.40 |
|  |  | 20 | 14.86 |
| 1T4R | 5 | 10 | 2.16 |
|  |  | 13 | 5.39 |
|  |  | 20 | 11.84 |
| 2T2R | 5 | 10 | 5.17 |
|  |  | 13 | 8.40 |
|  |  | 20 | 14.85 |
| 1T2R | 40 | 10 | 5.24 |
|  |  | 13 | 8.34 |
|  |  | 20 | 14.93 |

Our simulation results of FR1 TDD cases are provided in the table below

| TDD 30kHz AWGN+500Hz doppler typeA |  |  |  |
| --- | --- | --- | --- |
| Antenna | Bandwidth (MHz) | MCS | SNR（70%TP） |
| 1T2R | 10 | 10 | 5.32 |
|  |  | 13 | 8.41 |
|  |  | 20 | 15.05 |
| 1T4R | 10 | 10 | 2.32 |
|  |  | 13 | 5.40 |
|  |  | 20 | 12.04 |
| 2T2R | 10 | 10 | 5.33 |
|  |  | 13 | 8.41 |
|  |  | 20 | 15.05 |
| 1T2R | 100 | 10 | 5.17 |
|  |  | 13 | 8.42 |
|  |  | 20 | 15.06 |

- Simulation assumption

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | AWGN+200Hz for FDD AWGN+500Hz for TDD |
| Transform precoding |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

- Conclusion

In this contribution, we provide our initial simulation results about NR UE ATG demodulation requirements.

- Reference

[1] R4-2305899, WF on absolute physical layer throughput requirement, Intel Corporation.