3GPP TSG-RAN4 Meeting #107	R4-2307626

# Incheon, Korea, 22th– 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on ATP requirements for TDD 4Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the work plan, the ATP requirements for TDD 4Rx PDSCH should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding ATP requirements for TDD 4Rx. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI NR_demod_enh3 cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.2.3.2.X	Minimum requirements PDSCH absolute physical layer throughput with link adaptation

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput with link adaptation performance under 4 receive antenna conditions.

For the parameters specified in Table 5.2.3.2.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

The required SNR to achieve T% of maximum throughput as specified in Table 5.2.3.2.X-2, where the maximum throughput is defined as the TBS corresponding to CQI index 15 with rank 2.

Table 5.2.3.2.X-1: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 2×4 |
| Correlation configuration |  |  |  | ULA Low |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3, (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 16 |
| Csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| aperiodicTriggeringOffset |  |  |  | 9 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  |  | CodebookSubsetRestriction |  | Not configured |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 5.5 |
| Maximum number of HARQ transmission (Note 1) |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Measurement channel |  |  |  | As specified in Table A.4-2, Rank 1: TBS.2-3 Rank 2: TBS.2-4 |
| Note 1: For retransmission including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission. |  |  |  |  |

Table 5.2.3.2.X-2: Minimum requirements

|  | T% of maximum throughput | Required SNR (dB) |
| --- | --- | --- |
| Test 1 | TBD | TBD |
| Test 2 | TBD | TBD |

<End of change>