3GPP TSG-RAN WG4 Meeting # 107												R4-2307634

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			9.4.3

Source:	Anterix

Title:	Test and Simulation Results for LTE US 900

Document for:	Discussion

# Introduction

Based on the R4-2306569 WF on UE Requirements of the US 900 MHz band recommendation for UE coexistence testing, this contribution presents independent lab testing and live network testing the for LTE US 900 band 106.  We evaluated three different devices operating in band 106 with the intent of looking at meeting a -30 dBm/MHz coexistence.  We also looked at spectrum occupancy in 6 different US markets to what impacts this would have on existing networks.

# Lab testing assumption and setup

Testing was done on FCC certified devices in a conducted setup utilizing Band 8 devices operating with network assigned duplexing so as to emulate band 106 as band 106 devices and test equipment are not available.  Testing was done per 3GPP TS 36.101 section 6.6.3 with a measurement bandwidth of 1 MHz for all bands.  Channel Bandwidths of 3 MHz and 1 MHz were tested with a center frequency set at 899 MHz for the 3 MHz CBW.

The following devices were evaluated:

- Gateway MTR5-LEU2-B04 (MultiTech rCell 500 series)  FCC ID : RI7LE910CXWWX

- Gateway RV50X (Sierra Wireless) , FCC ID : N7NMC7455

- AirPrime EM7455 Module (Sierra Wireless) : FCC ID - N7NEM7455

## Test Results

All test results are worst-case coexistnece measurements withing the LTE band 5 downlink range 869-894 MHz.  The devices tested all pass the proposed -30 dBm coexistence level, with significant margin, without the use of A-MPR.

Figure 1: MultiTech rCell 500

Figure 2:  Sierra Wireless RV50X

Figure 3:  Sierra Wireless EM7455

# Spectrum Utilization

The band 5 spectrum allocation in the US (869 -894 MHz) is primarily utilized by three network operators.  In the networks evaluated the spectrum utilized on the edge ends around 890 MHz with the last 4 MHz of spectrum not typically occupied.

Each market was driven for over 2 hours and data averaged over that time.  A Rohde & Schwarz TSMA 6 scanner was used with an external antenna for all tests.

## Band 5 Spectrum usage across the US

The following are spectrum scans from each market, taken during Q1 2023.

Figure 4: Boulder, CO

Figure 5: Cleveland, OH

Figure 6: Detroit, MI

Figure 7: Houston, TX

Figure 8: Minneapolis, MN

Figure 10: Pittsburgh, PA

# Conclusions

Our data collected independently in the lab with devices typically used in our networks show that a -30 dBm coexistence requirement would allow for existing band 5 networks to operate with the same performance and allow band 106 networks to coexist.  No A-MPR is necessary to achieve this and no changes to Ue or eNodeB filter designs is necessary with the proposed specification.

For several years Anterix has deployed multiple LTE networks operating in band 106 spectrum in close proximity to existing band 5 networks.  These legacy LTE networks are band 8 Ue and eNodeB operating with the optional network assigned duplexing – nearly identical to what is proposed by band 106 and there have not been any operational issues with coexistence with band 5 during that time.

Our conclusion is that the -30 dBm coexistence specification proposed and supported by multiple companies is the best solution to enable the band 106 market.

# Appendix: Test Data

…