3GPP TSG-RAN WG4 Meeting #107									              R4-2307643

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.11.2.3

Source:	Apple

Title:	Discussion on RRM core requirements for option C

Document for:	Discussion

# Introduction

In RANP#99 a new work item on BWP without restriction was approved [1]. Objectives are duplicated here for information:

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

RAN4 TU starts from April meeting. RAN4 had initial discussion and reached some agreement in RAN4#106-bis-e [1]. In this contribution, we focus on the remaining issues of option C.

# Discussion

Scope of option C is to specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs. CD-SSB and NCD-SSB are different logic concepts.

The first issue we would like to discuss is about clarification on existing timing requirements for supporting Option C:

## Issue 4-3: Clarification on existing timing requirements for supporting Option C

<Way forward >:

- Proposals

  - Option 1: (Ericsson, Nokia, Intel, CATT, MediaTek, OPPO)

    - The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

  - Option 2: (vivo, Huawei, Spreadtrum)

    - It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

According to discussion in previous RAN4 meetings, it is agreeable that existing timing requirements can still apply. Only some clarification is needed. The above two options do not contradict each other.

Proposal 1: The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

Proposal 2: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

Regarding inter-frequency measurement:

## Issue 4-5: Impact to inter-frequency measurement requirements for supporting option C

<Way forward >:

- The issue is postponed.

This issue can be revisit after RAN4 reaches consensus on intra-frequency measurement.

As for other requirement in addition to L1/L3 measurements for supporting option C:

## Issue 4-7: Whether to define requirements in addition to L1/L3 measurements for supporting option C

<Way forward >:

- The issue is postponed.

There were some proposals w.r.t. handover requirements in the last RAN4 meeting. However, we think this has dependency on how we handle L3 measurement, which is not in the scope yet. We suggest RAN4 revisits this once L3 measurement related issue is concluded.

# Conclusion

In this contribution, we provide discussion on BWP without restriction – option C. After discussion, the following conclusions are provided:

Proposal 1: The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

Proposal 2: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

# References

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo

- R4-2303313, WF on BWP_withoutRestriction, vivo