3GPP TSG-RAN WG4 Meeting #107									              R4-2307652

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.25.5

Source:	Apple

Title:	Discussion on Enhanced CHO configurations

Document for:	Discussion

# Introduction

Enhanced CHO configurations were preliminarily discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues. In this contribution, we continue discussing the open issues.

# Discussion

# CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

The first issue in [1] is about definition of Tprocessing in PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

Issue 3-2-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Agreements:

  - PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution.

  - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2.

  - Definition of Tprocessing:

    - Option 1: same as that defined in requirements of handover with PSCell.

    - Option 2: same as that defined in requirements of CHO.

The key different between legacy handover with PSCell and procedure in objective 3 is how the handover is triggered. Once conditions in CHO for PCell is met, UE behavior is exactly same as handover with PSCell. Requirement this case has been extensively discussed in FeRRM WI. We don’t need to reopen the discussion. Therefore, reusing same definition in requirements of handover with PSCell, as in option 1, is quite straightforward to us. Regarding CHO, the reason it cannot be reused is it doesn’t involve unknown PSCell.

Proposal 1: in PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3), Tprocessing is same as that defined in requirements of handover with PSCell, which is:

- Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

The second issue is about PSCell handover delay:

Issue 3-2-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Candidate solutions:

  - Companies are encouraged to further discuss the following issues:

    - When does UE execute PSCell addition/change

      - Option 1: after PCell CHO is executed but before it is completed.

      - Option 2: after PCell CHO is completed.

    - Whether Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell?

For the first question, i.e., when does UE execute PSCell addition/change, methodology of handover with PSCell can be reused. Specifically, PSCell addition/change is executed after PCell CHO is executed but before it is completed.

Regarding Tsearch_PCell_Conditional, since the overall procedure is triggered by meeting the corresponding events, most likely target PCell is already a known cell. However, as mentioned by some company in RAN4#106-bis-e that ‘target PCell is known’ doesn’t always mean SFN is known. The example is: SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. In this case, addition TΔ + 2ms is needed for UE to acquire SFN of the target PCell.

Proposal 2: in PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3), PSCell addition/change is executed after PCell CHO is executed but before it is completed.

Proposal 3: Tsearch_PCell_Conditional = TΔ + 2ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tsearch_PCell_Conditional = 0.

Proposal 4: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3):

- DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms.

# CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

The first issue is about PCell handover delay:

Issue 3-3-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: same as that in obj. 3.

  - Option 2: DCHOwithCPAC = Max [DCHO,  DCPAC]

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 3: same as CHO

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

    - Tprocessing = 25 ms

    - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Issue 3-3-2: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1:

    - if conditions for CPC are met before conditions for CHO are met, follow objective 3 requirements.

    - If conditions for CPC are met after CHO is complete, reuse existing CPC requirements.

  - Option 2: DCHOwithCPAC = Max [DCHO,  DCPAC]

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 3:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

It was agreed in RAN2 in previous meeting that

For the CHO+CPC case:  When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.  Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).  Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

As it can be observed, the baseline is UE needs to wait until both CHO and CPC conditions are met. FFS whether CHO can be executed before CPC condition is met. At current stage RAN4 shall focus on the baseline procedure.

Proposal 5: RAN4 shall focus on baseline procedure in objective 4, i.e., When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. The UE waits until both CHO and CPC conditions are met.

With this proposal, we can derive PCell and PSCell handover delay requirement in objective 4 by reusing framework of objective 3 with some modification on TEvent_DU and Tmeasure since the procedure won’t be executed unless both event conditions are met.

Proposal 6: in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4), both PCell and PSCell delay can reuse framework of obj.3, with modification on TEvent_DU and Tmeasure since the procedure won’t be executed unless both event conditions are met.

Proposal 7: TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

The next issue is about interruption from multiple scenarios due to performing CHO and CPC simultaneously.

Issue 3-3-3: possible interruption from multiple scenarios due to performing CHO and CPC simultaneously

- Candidate solutions:

  - Option 1:

    - Before defining requirements, RAN4 shall discuss possible interruptions from multiple scenarios due to performing CHO and CPC simultaneously.

      - By assuming Tevent_DU for CHO and CPC are started at the same time but done in different time, there can be potential interruptions such as

      - RF tune during CHO or CPC can interrupt to perform the measurement for the other process of CHO or CPC.

      - RF tune may not start at the same time. During Tprocessing, it can be interrupted due to RF tune for the other process of CHO or CPC.

  - Option 2: FFS

As mentioned in proposal 5, RAN4 shall start from baseline scenario at current stage. In baseline scenario, UE waits until both CHO and CPC conditions are met and then triggers handover with PSCell. Therefore the interruption mentioned in this issue would not exist.

Observation 1: interruption from multiple scenarios due to performing CHO and CPC simultaneously does not exist in baseline scenario.

According to the approved work plan, RAN4 needs to work on CR splitting in this meeting.

- 3GPP RAN4 #107 meeting (May 2023, Core Part) - Discuss RRM core part requirements for L1/L2 triggered mobility - Discuss RRM core part requirements for NR-DC with selective activation of cell groups via L3 enhancements - Discuss RRM core part requirements for Enhanced CHO configurations - Conclude approach(es) for improvement on SCell/SCG setup delay - LS to RAN2 for definition of corresponding signalling support, if needed - Work split on RRM core part CRs - Reply RAN1/2 LS, if any

Considering the progress of this objective is quite good, we think RAN4 can start working the CR. Thus we propose the following splitting:

Proposal 8: CR splitting for enhanced CHO configurations.

| Title | Section | Responsible company |
| --- | --- | --- |
| Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC | (new) 6.1.6.1.1 |  |
| Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC | (new) 6.1.6.1.2 | Apple |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC | (new) 6.1.7.1.1 |  |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC | (new) 6.1.7.1.2 |  |

# Conclusion

In this contribution, we provide further discussion on improvement on enhanced CHO configurations. After discussion, the following conclusions are provided:

Proposal 1: in PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3), Tprocessing is same as that defined in requirements of handover with PSCell, which is:

- Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

Proposal 2: in PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3), PSCell addition/change is executed after PCell CHO is executed but before it is completed.

Proposal 3: Tsearch_PCell_Conditional = TΔ + 2ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tsearch_PCell_Conditional = 0.

Proposal 4: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3):

- DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms.

Proposal 5: RAN4 shall focus on baseline procedure in objective 4, i.e., When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. The UE waits until both CHO and CPC conditions are met.

Proposal 6: in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4), both PCell and PSCell delay can reuse framework of obj.3, with modification on TEvent_DU and Tmeasure since the procedure won’t be executed unless both event conditions are met.

Proposal 7: TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

Observation 1: interruption from multiple scenarios due to performing CHO and CPC simultaneously does not exist in baseline scenario.

Proposal 8: CR splitting for enhanced CHO configurations.

| Title | Section | Responsible company |
| --- | --- | --- |
| Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC | (new) 6.1.6.1.1 |  |
| Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC | (new) 6.1.6.1.2 | Apple |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC | (new) 6.1.7.1.1 |  |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC | (new) 6.1.7.1.2 |  |

# References

- R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2303309, WF on NR Mobility Enhancements RRM requirements (part 2), Apple