3GPP TSG-RAN WG4 Meeting #107									              R4-2307654

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.26.2.2

Source:	Apple

Title:	On R18 MUSIM - collisions between gaps and priority rules

Document for:	Discussion

# Introduction

Collisions between gaps and priority rules were discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues.

In this contribution, we continue discussing the open issues.

# Discussion

The first issue is about constraints on MUSIM gap priority configuration from NW A:

Issue 2-1-4-1: Constraints on MUSIM gap priority configuration from NW A

- Proposals

  - P1: NW A maintains the same relative priorities requested by the UE; The configured priority level may or may not be the same as that requested by UE. (vivo Apple MTK Xiaomi Huawei Qualcomm Nokia Charter ZTE)

    - P1-a: Based on P1, NW A is not required to keep the relative priority order for a particular MUSIM gap when the MGRP of that particular MUSIM gap is less than a threshold, in this scenario NW A will still keep the same relative order of the other MUSIM gaps except for that particular MUSIM gap (vivo)

  - P2: When MUSIM gaps with equal priority is allowed, if UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (vivo Apple Qualcomm oppo)

  - P4: If the network cannot fulfill the UE priority requests the network may chose not to assign the requested MUSIM gaps (Nokia Qualcomm)

  - P6: In the special case when both one MUSIM gap and one other MG gap has set the highest priority level in gapPriority-r17 IE, then we propose that MUSIM gap has the ability to signal with an extra 1-bit to indicate higher priority than the highest level in gapPriority-r17 IE (Charter)

Agreements: No

Candidate options:

Recommendations: Continue discussion

We would like to point out that RAN4 already agreed it is up to NW A on how to use the priority information from UE. Therefore, NW doesn’t need to strictly follow the priority information provided by UE. However, it is beneficial for NW to maintain the same relative priorities requested by the UE since UE shall have better understanding than NW on how to use the MUSIM gaps. This doesn’t conflict with previous agreements. Another thing is we see some benefit of allowing same priority if merging two gaps with equal priority is allowed.

Proposal 1: NW A maintaining the same relative priorities requested by the UE. The exact priority may or may not be the same as that requested by UE.

Regarding aperiodic gaps:

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps

Agreements from GTW

  - Option 1 (QC, Nokia, vivo, Charter, Xiaomi)

    - The priority level of aperiodic MUSIM gap can be configured by NW A

    - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level

    - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A

  - Option 2 (MTK, Huawei, Apple, Ericsson, OPPO):

    - Aperiodic MUSIM gap by default has the highest priority level.

    - The gap priority level is not explicitly configured by the NW

Agreements: No

Candidate options:

Recommendations: try to down select between option 1 and 2

Considering aperiodic MUSIM is still triggered by network and it is expected not to happen quite often, we have no problem with no priority for it. If network thinks it should be with lower priority than another overlapping gap and shall be dropped, network shall not trigger this aperiodic at all. In other word, network won’t trigger aperiodic gap unless the aperiodic gap can always override other overlapping gaps.

One use case mentioned in the last RAN4 meeting is that, if RAN4 agrees on ‘Keep’ solution, then it is possible that UE can keep both aperiodic gap and the overlapped periodic gap if they are with equal priority. This might be a valid use case. However, assigning priority for aperiodic gap is not the only choice. We can also consider ‘Keep’ solution for aperiodic MUSIM gap.

Observation 1: if an aperiodic MUSIM gap will be dropped due to collision with other gaps, network shall not configure this aperiodic gap at all. In other word, there is no benefit for network to configure aperiodic MUSIM gap unless the aperiodic gap can always override other gaps.

Proposal 2: no need to assign priority of aperiodic MUSIM gap. In case of collision,

- Option 1: aperiodic MUSIM gap shall override other gaps.

- Option 2: keep both overlapping gap occasions.

Next issue is about order for applying the priority when number of colliding MGs is larger than 2:

Issue 2-1-6: Order for applying the priority when number of colliding MGs is larger than 2

- Proposals:

  - P1: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority (vivo xiaomi Qualcomm MTK oppo Huawei Apple)

  - P2: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority. (Ericsson Nokia Chapter ZTE)

Agreements: No

Candidate options:

Recommendations: Discuss at the next meeting

P1 is straightforward to us. It can make both network and UE very clear about which gap to use in case of collisions.

Proposal 3: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

Regarding definition of collision:

Issue 2-2-1: Definition of the collision between different MUSIM gaps

- Proposals

  - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (Qualcomm Apple MTK xiaomi Ericsson CMCC ZTE oppo vivo)

  - Option 2: Postpone the discussion till issue 2-2-2 is stable (Huawei Qualcomm ZTE)

  - Option 3: A collision between MUSIM gaps means a physical overlap in time domain between two MUSIM gaps and RAN4 does not define ‘proximity’ for collisions between MUSIM gaps. (Nokia)

Agreements: No

Candidate options:

Recommendations: Continue discussion.

Option 1 is the simplest to move forward.

Proposal 4: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps.

As for solutions for collision between different MUSIM gaps:

Issue 2-2-2: Solutions for collision between different MUSIM gaps

- Proposals

  - Option 1: Priority based solution is used for collision between different MUSIM gaps when these collided MUSIM gaps are assigned with different priority levels (vivo Apple Qualcomm)

  - Option 2: Priority based solution is used for collision between different MUSIM gaps (MTK xiaomi oppo Charter)

  - Option 2-1: The priority handling rule shall be used if it collides with the periodic gaps (except the paging gap) (ZTE)

  - Option 3: Keep solution (keep all collided MUSIM gap) is used when MUSIM gaps have same priority (Apple Qualcomm oppo vivo)

    - Option 3a: The overlapping MUSIM gap occasions under scenario of option 3 shall be merged. (Apple)

    - Option 3b: whether multiple MUSIM gaps can be assigned the same priority level or not subject to UE capability (Qualcomm vivo)

    - Option 3c: Up two periodic MUSIM gaps can be configured with the same priority and inform such the configuration to RAN2 (oppo)

  - Option 4: MUSIM gap ‘keep rule’ will be applied in some certain scenarios, such as Paging monitoring and AGC (Ericsson)

  - Option 4-1: The kept/merged solution is used if the second gap in collision is paging gap (ZTE)

  - Option 5: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide (Huawei)

  - Option 6: For keep solution, UE shall under defined conditions not drop a colliding MUSIM gap of lower priority, provided the UE perform all actions related to the colliding MUSIM gaps of higher priority or priorities. RAN4 shall define these conditions. (Nokia)

Agreements: No

Recommendations: Consider and down-select from the following options at the next meeting:

- Option 1: Priority based solution is used for collision handling between different MUSIM gaps

- Option 2: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps

- Option 3a: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps

  - Priority based solution is used when collided MUSIM gaps have different priority levels

  - Conditions when Keep solution is used are FFS.

- Option 3b: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps

  - Conditions when priority based solution is used and conditions when Keep solution is used are FFS.

In general we support option 3a in the moderator’s summary. The simplest way is to apply the same collision handling in concurrent gaps design. We understand that sometimes UE may need two gaps the achieve one purpose. For instance, UE may need to perform AGC, T/F tracking before paging reception. In this case UE may request a gap with longer MGL instead of requesting two gaps. Alternatively, UE can request two gaps and RAN4 allows UE to merge the two gaps. This can be achieved by requesting two gaps with same priority.

Proposal 5: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps

- Priority based solution is used when collided MUSIM gaps have different priority levels

- Keep solution is used when MUSIM gaps have equal priority level.

Next issue is about solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority:

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority

- Proposals

  - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (vivo Apple xiaomi oppo)

  - Against: (Nokia)

  - P2: No requirements apply if the two gaps are configured with same MGRP (Huawei Ericsson)

  - Against: (Nokia)

  - P3: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios: Any of the collision gaps is Type-1 MG; NW-A doesn’t configure a priority associated with any of the collision gaps. (Ericsson Huawei ZTE MTK)

  - P4: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm)

  - Against: (Huawei xiaomi)

  - P5: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated. (Nokia)

  - Against: (Qualcomm Huawei xiaomi )

  - P6: The sharing rule solution could be considered. (xiaomi)

  - Against: (Qualcomm Huawei)

Agreements: No

Candidate options:

Recommendations: Suggest to focus on P1, P2 and P3 since they have relative more supporters and less objections.

We think this issue only applies for the case that NW has been upgraded to support priority configuration for MUSIM gap or NW A gaps. Otherwise, we see no point for NW not to provide priority information for MUSIM gaps and NW A gaps. We have no problem with leaving no requirements for that since the scenario would exist temporarily.

Observation 2: collision between MUSIM gap and Type-1 MG or gap configured without priority shall only happens when NW hasn’t been upgraded to support priority configuration of MUSIM gaps and NW A gaps.

Proposal 6: considering the scenario would only exist temporarily, requirements shall not apply if any one of the collided gaps is not assigned a priority.

The last issue is about collision between SMTC and MUSIM gaps for handover and Scell activation

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation

- Proposals

  - P1: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no special handling solution is defined. (Qualcomm MTK xiaomi Huawei Nokia vivo oppo)

  - P2: RAN4 to define requirements for the collision between MUSIM gaps with Handover, Scell activation and SI update. When NW-A’s RS resources for one-shot RRM procedure (Handover, Scell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority; Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading have higher priority than Measurements procedures for both NW-A and NW-B (Ericsson)

  - P3: When NW-A’s uplink signals for one-shot RRM procedure(Handover, Scell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for Scell activation should be prioritized. (Ericsson)

  - P4: Add a high-level clarification in RAN4 spec that during one-shot procedure such as Scell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met. (Apple Ericsson)

Agreements: No

Candidate options:

Recommendations: Discuss at next meeting

Too avoid complicated design, one possible compromise could be that: from requirement point of view, RAN4 confirms that the scope of collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements. Besides, RAN4 clarifies in high level in our spec that during some one-shot procedure, MUSIM gaps are not expected to be enabled unless the one-shot procedure is not interrupted and can be completed in time. For instance, during SCell activation, UE is not expected to enable MUSIM gaps unless the existing SCell activation delay can still be met.

Proposal 7: from requirement point of view, RAN4 confirms collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG.

Proposal 8: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

# Conclusion

In this contribution, we provide further discussion on improvement on collisions between gaps and priority rules. After discussion, the following conclusions are provided:

Proposal 1: NW A maintaining the same relative priorities requested by the UE. The exact priority may or may not be the same as that requested by UE.

Observation 1: if an aperiodic MUSIM gap will be dropped due to collision with other gaps, network shall not configure this aperiodic gap at all. In other word, there is no benefit for network to configure aperiodic MUSIM gap unless the aperiodic gap can always override other gaps.

Proposal 2: no need to assign priority of aperiodic MUSIM gap. In case of collision,

- Option 1: aperiodic MUSIM gap shall override other gaps.

- Option 2: keep both overlapping gap occasions.

Proposal 3: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

Proposal 4: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps.

Proposal 5: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps

- Priority based solution is used when collided MUSIM gaps have different priority levels

- Keep solution is used when MUSIM gaps have equal priority level.

Observation 2: collision between MUSIM gap and Type-1 MG or gap configured without priority shall only happens when NW hasn’t been upgraded to support priority configuration of MUSIM gaps and NW A gaps.

Proposal 6: considering the scenario would only exist temporarily, requirements shall not apply if any one of the collided gaps is not assigned a priority.

Proposal 7: from requirement point of view, RAN4 confirms collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG.

Proposal 8: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

# References

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303249, LS on priority levels for MUSIM gaps, vivo