3GPP TSG RAN WG4 Meeting #107	R4-2307717

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.8.3.1

Source:	vivo

Title:	On general aspects for FR2 multi-Rx

Document for:	Discussion

- Introduction

In the RAN4#106-bis meeting, there were continued discussions on the RRM impacts and general aspects of FR2 multi-Rx chain DL reception. The agreements below and other open issues are captured in the WF [1].

- Sub-topic 1-1: Scope and scenarios ### Issue 1-1-2: Single (component) carrier for defining RRM requirements <Agreement >: - Deprioritize discussions on RRM requirements for CA/DC scenarios till single carrier requirements are completed. ### Issue 1-1-5: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel <Agreement >: - Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to receive two independent signals from the same or nearly the same direction is not further discussed in RRM session. - Note: It can be revisited in case any RRM impact is identified. ### Issue 1-1-9: Scenarios/assumption for “simultaneous reception” <Agreement >: - Scenarios for mTRP simultaneous reception is to be covered as part of scheduling restrictions/measurement restrictions, L1 measurement, and TCI state switching agenda items - Sub-topic 1-2: RRM requirements impact ### Issue 1-2-5a: Whether to define power saving related requirements <Agreement >: - No power saving specific requirements, e.g., L1 measurements relaxation for multi-Rx operation, are considered in the WI. ### Issue 1-2-2: UE architectures <Agreement >: - Multi-Rx architecture impact to RRM requirements can be considered on a case-by-case basis, if needed - Sub-topic 1-3: Applicability and conditions - Sub-topic 1-4: UE capability Issue 1-4-1: Clarification/understanding on R16 UE capabilitiy simultaneousReceptionDiffTypeD <Agreement >: - It is RAN4 understanding that the R16 UE capability simultaneousReceptionDiffTypeD is only applicable for simultaneous PDSCH reception. ### Issue 1-4-2: UE capability for simultaneous reception in Rel-18 <Agreement >: - The issue is postponed until there is sufficient progress on requirements. ### Issue 1-4-3a: UE capability of simultaneousRxDataSSB-DiffNumerology-Inter-r16 <Agreement >: - No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI

In this contribution, we further provide our views on general aspects of FR2 multi-Rx chain DL reception. According to agenda for this RAN4 meeting, general aspects include scope, scenarios, capabilities, RRM impacts and topics not covered by other AIs.

- Discussion

- 2.1 RRM requirements impact

### Issue 1-2-1: Definition of “simultaneous reception” <Way forward >: - Proposals - Option 1: - Definition of simultaneous reception from a baseband receiver perspective should be covered in Demod discussions, and from antenna panel perspective it should be covered in RF discussions. - Option 2: - Two signals are considered to be received simultaneously, if their instances are received in the same or overlapping OFDM symbols, which may occur in one, some, or all signal occasions during their measurement or evaluation period. - NOTE 1: Only that two measurements are performed in parallel does not directly imply that the RSs are received simultaneously, e.g., two parallel measurements can be based on two RSs whose occasions do not even overlap and which therefore are not received simultaneously. - Minimum overlap of RS occasions: The minimum amount of overlap needed for multi-rx requirements to apply is TBD (e.g., full overlap, partial overlap, at least X% of overlapping RS occasions during the measurement period, etc.). - Option 3: - No need to discuss definition of ‘simultaneous reception’. The condition can be discussed during CR phase

It is not necessary to have such ‘definition’ as option 2 for simultaneous reception. The spec impact of simultaneous reception is on requirements for measurement restriction and scheduling restriction, which are defined based on single occasion. Thus, there is no need to discuss the definition for simultaneous reception. If any conditions are needed for simultaneous reception, it can be discussed in CR.

Proposal 1: No need to discuss definition of ‘simultaneous reception’. The condition can be discussed during CR phase.

### Issue 1-2-3: Beam management related - Proposals - Option 1: (Nokia, ZTE) - • RAN4 to define requirements assuming Independent Beam Management framework as baseline across multiple Rx chains on the same carrier following agreements from RF on AoA limitation. - Option 2: (Huawei, LGE, Intel, vivo, Apple, Samsung, Ericsson, MTK, OPPO) - Not to strive for agreement on the issue, but focus on detailed requirements.

It is up to UE implementation how multi-Rx capable UE receives two beams from different directions on a single carrier. This can be indicated by new UE capability for simultaneous reception to be introduced in Rel-18. It is not necessary to link requirements to IBM/CBM capability which are used for CA/DC.

Proposal 2: Not to strive for agreement on defining requirements assuming IBM framework, but to focus on detailed requirements.

### Issue 1-2-5: Indication of multi-Rx operation < Way forward >: - Proposals - Option 1: (Nokia, Huawei, LGE, Intel, ZTE, vivo, Samsung, Apple) - Introduce mechanism/condition for indication of multi-Rx operation, including on/off indication of multi-Rx operation. - FFS on mechanism/condition for indication of multi-Rx operation - Option 2: (MTK, OPPO) - No new mechanism is needed for UE to fallback from multi-Rx to single Rx.

Then power saving gain may be achieved by allowing UE to turn on/off panel(s) when it is possible or when it is necessary. For supporting 4-layer MIMO with multi-Rx, it is mainly about tradeoff between throughput and power saving. UE assistance information may be considered for this purpose. In addition, multi-Rx could also be used for enhanced L1 measurements with simultaneous reception. Independent UE capability for enhanced L1 measurements should be introduced. Therefore, different assistance information than for 4-layer MIMO operation may be considered for power saving purpose for enhanced L1 measurements.

Proposal 3: Power saving mechanism by indication of multi-Rx operation may be considered for multi-Rx UE operation in the WI.

### Issue 1-2-7a: RRM impact of group-based beam reporting for simultaneous reception <Way forward >: - Proposals - Option 1: - RAN4 to discuss the conditions under which a group should be reported using Rel-17 GBBR, as part of L1 measurement requirements. - Option 2: - RAN4 to discuss the Rx beam assumptions (beam in beam pairs for simultaneous reception or best beam for each TRP) when UE turns on multi panels. - Option 3: - No specific new requirements for group-based beam reporting are necessary to be defined and existing L1-RSRP measurement requirements are applicable for group-based beam reporting. - Option 4: - RAN4 to clarify whether simultaneous reception for single TRP is considered. - When group based reporting is configured, RAN4 to further discuss how to define reporting format for the case that no good beam pair is available. - Option 5: - Group-based beam reporting for simultaneous reception has no impact to RRM measurement accuracy requirements - Group-based beam reporting for simultaneous reception has impact to RRM L1-measurement period requirements. - For UEs support group-based beam reporting capability, the L1-RSRP measurement delay requirement in procedure 6 (Rel-17 group-based reporting procedure) is somehow related to NW configuration - If groupBasedBeamReporting-r17 = ‘ disabled ‘ , the existing L1-RSRP measurement delay requirements can be reused - If groupBasedBeamReporting-r17 = ‘enabled’, whether it is necessary to enhance the requirements need further discussion. - RAN4 need to clarify the applicability of the enhanced L1-RSRP measurement period requirement.

UE is not required to measure CSI-RS and/or SSB resources simultaneously for group-based beam reporting. NW does not know which beam pair can be received by UE simultaneously before group-based beam reporting. Thus, it is not possible for NW to configure CSI-RS and/or SSB on the same resources for UE to receive simultaneously. It is up to UE implementation to figure out the beam pair for group-based beam reporting by measuring L1-RSRP of configured CSI-RS and/or SSB resources. The UE measurement is performed in TDM manner. Existing L1-RSRP measurement requirements are applicable for group-based beam reporting. No new requirements for group-based beam reporting are necessary to be defined.

Even if under certain cases, it might be possible for NW to configure CSI-RS and/or SSB resources on the same OFDM symbol, existing L1-RSRP measurement requirements can still be still applicable. There are already measurement restrictions on colliding symbols. Defining enhanced requirements for only some potential cases, it will require UE to turn of multiple panels for group-based beam reporting all the time, which is not friendly for power saving.

Proposal 4: No specific new requirements for group-based beam reporting are necessary to be defined and existing L1-RSRP measurement requirements are applicable for group-based beam reporting.

### Issue 1-2-10: Frequent switching - Proposal - Option 1: (Ericsson) - Frequent reconfiguration of the UE between multi-rx and non-multi-rx operation should be avoided, e.g., the important measurements and procedures may need to be completed before the switching. - Option 2: (Nokia, Huawei, ZTE, vivo, Samsung, MTK) - No further discussion on frequent switching as it is up to NW implementation.

Frequent reconfiguration of the UE between multi-Rx and non-multi-Rx operation is totally up to network implementation. There is no spec impact.

Proposal 5: No further discussion on frequent switching as it is up to NW implementation.

- 2.5 Applicability and conditions

### Issue 1-3-1: Applicability of new requirements to different QCL types <Way forward >: - Proposals - Option 1: - Requirements defined for QCL type-D only are also applicable when QCL type D is configured together with QCL type A/C. - Option 2: - Scenarios where QCL type D is configured in combination with QCL type A/C should be also covered by the requirements for simultaneous RS reception. - FFS: The same RRM requirements for simultaneous RS reception can apply for RS configured with: - QCL type D, - QCL type D + QCL type A, or - QCL type D + QCL type C.

Firstly, for TCI state configuration, two QCL types may be configured, e.g., the UE is configured with QCL type A or QCL type C in addition to QCL type D in FR2.

In FR2, QCL type D should always be configured. The reference signal for QCL type A/C may be the same as for QCL type D. Different reference signals for QCL type1 and QCL type2 may also be configured.

In existing requirements for BFD, UE is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the UE active BWP

In existing RLM requirements, the requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

In our understanding, the issue means that the requirements specified for QCL Type-D explicitly should also be applicable when QCL type-A or QCL type-C are configured together. However, it seems not necessary to have such high-level conclusion unless there are clear requirements for this being identified.

Proposal 6: No further discussion the issue unless impact to specific requirements is identified.

- 2.6 UE capability

### Issue 1-4-3: UE capability of simultaneousRxDataSSB-DiffNumerology <Way forward >: - Proposals - Option 1: - The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI. - Option 2: - The existing UE capability simultaneousRxDataSSB-DiffNumerology is feasible if the supporting of concurrent SSB and data are consistent between FR1 and FR2-1. But it should be ultimately determined after the decision of single or multiple UE capabilities are necessary. - Option 3: - No need to specify new mix-numerology capability for multi-RX chain in FR2 when consider simultaneous SSB and data reception. - Option 4: - A new UE capability is needed to support simultaneous reception with mixed numerologies. - Option 5: - The existing UE capability to support mixed numerologies is not directly applicable for multi-RX chain operation with mixed numerologies, hence a new UE capability is needed to support simultaneous reception with mixed numerologies.

The definition of simultaneousRxDataSSB-DiffNumerology is as follows.

simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5].

The purpose of the UE capability is to indicate concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology. It is used for defining measurement restriction and scheduling restriction requirements for L1 and L3 measurements in FR1.

For multi-Rx operation, when measurement restriction and scheduling restriction requirements are defined for simultaneous reception of RS+RS and RS+data from two directions, UE capability of simultaneous reception with a different numerology may be needed.

Proposal 7: New UE capability of supporting simultaneous reception with mixed numerology from different directions with different QCL type D RS +RS or RS + data for enhanced L1 measurements is needed depending on progress of measurement restriction and scheduling restriction requirements.

- Summary

In this contribution, we further provided views on general aspects for NR FR2 multi-Rx chain DL reception WI. Following proposals and observations are present.

Proposal 1: No need to discuss definition of ‘simultaneous reception’. The condition can be discussed during CR phase.

Proposal 2: Not to strive for agreement on defining requirements assuming IBM framework, but to focus on detailed requirements.

Proposal 3: Power saving mechanism by indication of multi-Rx operation may be considered for multi-Rx UE operation in the WI.

Proposal 4: No specific new requirements for group-based beam reporting are necessary to be defined and existing L1-RSRP measurement requirements are applicable for group-based beam reporting.

Proposal 5: No further discussion on frequent switching as it is up to NW implementation.

Proposal 6: No further discussion the issue unless impact to specific requirements is identified.

Proposal 7: New UE capability of supporting simultaneous reception with mixed numerology from different directions with different QCL type D RS +RS or RS + data for enhanced L1 measurements is needed depending on progress of measurement restriction and scheduling restriction requirements.

- References

- R4-2306318	WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo