3GPP TSG RAN WG4 Meeting #107	R4-2307721

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.8.3.5

Source:	vivo

Title:	On TCI state switch delay for multi-Rx

Document for:	Discussion

- Introduction

In the RAN4#106-bis meeting, there were discussions on dual TCI states switching for FR2 multi-Rx chain DL reception. The following agreements on general aspects relevant to dual TCI states switching were achieved and captured in the WF [1].

Issue 2-1-1: General conditions Agreements: RAN4 to define TCI state switch delay requirements only for the case where to-be-activated/switched TCI state upon the TCI switch command(s) reception includes two different QCL-TypeD reference resources and the reference resources of the two TCI states are included in the group-based L1-RSRP measurement report. - Note: This is one of the condition and other conditions are not precluded - Note: Only for single TCI to dual TCI and dual TCI to dual TCI. Issue 2-1-5: Other issues or proposals for further discussion Agreements: RAN4 not to discuss TCI state switching requirements for following cases in this WI - Dual TCI switching in SFN - FFS: PDCCH with repetition. Issue 2-3-3: Single DCI SFN (if it is agreed to discuss in multi-RX WI) Agreement: No discussion in this WI. Issue 2-3-4: Multi-DCI SFN (if it is agreed to discuss in multi-RX WI) Agreement: No discussion in this WI. Issue 2-5-1: Requirements to be consider Agreements: - Requirements for DCI based dual TCI states switch delay for PDSCH reception are defined for known case only. - Requirements for MAC CE based dual TCI states switch delay for PDCCH reception are defined for known case. FFS if it is to be limited to known case only. Issue 2-6-1: Active TCI state list update Agreements: For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs.

There were also many other issues of TCI state switching, which were also captured in the WF [1]. In this contribution, we further provide our views on TCI state switching requirements for FR2 multi-Rx chain DL reception.

- Discussion

- 2.1 TCI state switch delay requirements for PDCCH

In the RAN#106 meeting, following TCI state switching triggering mechanism were agreed.

- TCI Switch triggering method: - MAC-CE based dual TCI state switch for PDCCH reception - DCI based dual TCI state switch for PDCSH reception - FFS RRC triggered TCI state configuration.

In legacy NR requirements, RRC triggered TCI state switch/configuration is for PDCCH reception. The requirements for RRC based TCI state switch delay apply when only 1 TCI state is configured in RRC TCI state list. In our understanding, if multiple TCI states are configured in RRC command, it needs further triggering by MAC CE command. Thus, typically RRC command cannot trigger dual TCI state switching.

Proposal 1: RRC triggered TCI state configuration is only considered after feasibility is confirmed.

MAC-CE based dual TCI states switching for PDCCH

For PDCCH with single-DCI, two cases need to be considered.

- For SFN PDCCH, two TCI states should be activated and indicated by one single MAC CE. Then, the UE can receive from different directions based on the configured dual TCI states. SFN PDCCH is for single-DCI scheduling.

- For PDCCH non-SFN (PDCCH repetition), the TCI states are configured for different CORESETs independently

It was agreed that SFN PDCCH is not considered in the WI.

PDCCH repetition can only be supported for UE supporting mTRP-PDCCH-TwoQCL-TypeD-r17. With this UE capability, there could be the case that UE can receive time-domain overlapping CORESETs with two QCL type-D. PDCCH repetition may also be used for 4-layer MIMO, e.g., by scheduling SDM PDSCHs.

For PDCCH with multi-DCI, two TCI states are configured for PDCCH, and each TCI state is activated by one MAC CE. Multi-DCI PDCCH is also typical use case for supporting 4-layer MIMO in FR2.

Observation 1: MAC-CE based Dual TCI states switching requirements for PDCCH repetition and PDCCH with multi-DCI is relevant to 4-layer MIMO.

Proposal 2: For MAC CE based dual TCI states switch, requirements are specified for both single-DCI PDCCH repetition and multi-DCI PDCCH.

The legacy MAC-CE based TCI state switch delay requirements are specified for both known and unknown cases. For known cases, one SSB is needed for timing/frequency tracking if the target TCI state is not within the active TCI state list.

For MAC-CE based dual TCI states switch delay requirements, we need to decide whether both known and unknown cases should be considered. At least, the requirements for known cases should be specified. It needs further consideration if requirements should be specified for unknown case.

Since the target dual TCI states is for simultaneous PDCCH/PDSCH reception, it should be based on group-based beam reporting. Therefore, both the dual TCI states should be either known or unknown. It should be typical use case that both the dual TCI states are known. For unknown case, it may also be considered for the use case that dual TCI states are triggered by MAC-CE later than, e.g., 1280ms, after group-based beam reporting. In this case, beam pair information is still known to NW. NW may still configure the UE to use this beam pair for simultaneous PDCCH/PDSCH reception.

Proposal 3: Requirements for MAC-CE based dual TCI states switch delay for PDCCH reception are also defined for unknown + unknown case.

- 2.3 TCI state switch delay requirements for PDSCH

TCI states switching for PDSCH is triggered by DCI. For PDSCH with single-DCI, two TCI states are indicated by a TCI code point. Same threshold is used for the dual TCI states. The dual TCI state switch delay requirements could reuse legacy requirements as baseline.

For PDSCH with multiple-DCI, two TCI states are indicated by two TCI code points in two DCIs. It is considered as two independent DCI based TCI state switch. Legacy requirements could be reused as baseline. Further clarification on applicable conditions may be considered.

Regarding dual TCI states switch for PDSCH, it was agreed that the TCI state switch delay requirements are specified when both TCI states are known as in legacy requirements. Since unified TCI framework is not considered in the WI, the threshold of time offset between the reception of the PDCCH and the corresponding PDSCH is timeDurationForQCL.

Proposal 4: Requirements for single-DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

Proposal 5: Requirements for multiple-DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

- 2.4 Known/unknown condition

Issue 2-5-2: Definition of known condition: Way forward: GBBR is agreed as prerequisite to enable simultaneous reception. Does that mean dual TCI states are configured based on beams reported in GBBR or can other RS which are QCLed to beams reported in GBBR can be configured. Please consider following to provide your further analysis. - Option 1: dual TCI states are only configured based on beams reported in GBBR. - Option2: dual TCI states which are QCLed to beam pair reported in GBBR can be configured

In general, the known/unknown condition can be defined based on legacy known/unknown conditions for TCI state switch when possible. Additional conditions specific to dual TCI states can be further considered. In our view, the L1-RSRP measurement reporting should be group-based beam reporting that which beam pair(s) UE can receive simultaneously is reported and one of the reported beam pairs is configured as target dual TCI states. In addition, dual TCI states which are QCL-ed to beam pair reported in group-based beam reporting can also be configured since the spatial domain filter would be the same.

Therefore, following option would be reasonable for definition of known/unknown condition for dual TCI states switch.

Proposal 6: Known/unknown conditions for dual TCI states switch can be defined as follows.

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least one group-based L1-RSRP report configured with groupBasedBeamReporting or groupBasedBeamReporting-r17 and

-	the target dual TCI states are based on reported RS resources pair within one group, or

-	the target dual TCI states are QCL-ed to reported RS resources pair within one group

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

- 2.5 TCI state list update requirements

For enhanced TCI states activation/deactivation for PDSCH, addition of dual TCI states may be considered. It was agreed that for active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline.

The T/F tracking for the dual TCI states could still be based on single SSB. In this case the legacy requirements can also be reused. If the T/F tracking for the dual TCI states is based on different SSBs, then the active TCI state list update delay needs further discussion. Since T/F tracking for the dual TCI states are independent, the total delay is specified based on SSB with longer delay.

Proposal 7: For active TCI state list update for addition of a new dual TCI states, legacy requirements is reused if T/F tracking for the dual TCI states is based on single SSB.

Proposal 8: For active TCI state list update for addition of a new dual TCI states, the total update delay is based on SSB with longer delay if T/F tracking for the dual TCI states is based on different SSB.

- Summary

In this contribution, we provided views on dual TCI states switching requirements for NR FR2 multi-Rx chain DL reception. Following proposals are present.

Proposal 1: RRC triggered TCI state configuration is only considered after feasibility is confirmed.

Observation 1: MAC-CE based Dual TCI states switching requirements for PDCCH repetition and PDCCH with multi-DCI is relevant to 4-layer MIMO.

Proposal 2: For MAC CE based dual TCI states switch, requirements are specified for both single-DCI PDCCH repetition and multi-DCI PDCCH.

Proposal 3: Requirements for MAC-CE based dual TCI states switch delay for PDCCH reception are also defined for unknown + unknown case.

Proposal 4: Requirements for single-DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

Proposal 5: Requirements for multiple-DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

Proposal 6: Known/unknown conditions for dual TCI states switch can be defined as follows.

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least one group-based L1-RSRP report configured with groupBasedBeamReporting or groupBasedBeamReporting-r17 and

-	the target dual TCI states are based on reported RS resources pair within one group, or

-	the target dual TCI states are QCL-ed to reported RS resources pair within one group

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

Proposal 7: For active TCI state list update for addition of a new dual TCI states, legacy requirements is reused if T/F tracking for the dual TCI states is based on single SSB.

Proposal 8: For active TCI state list update for addition of a new dual TCI states, the total update delay is based on SSB with longer delay if T/F tracking for the dual TCI states is based on different SSB.

- References

- R4-2306319	WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2), Ericsson