3GPP TSG-RAN WG4 Meeting#107	R4-2307732

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1522 | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to configured maximum power for inter-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For inter-band UL CA band combinations (BC), the PHR for serving cells are not correct in case the power class of the band combination is lower than the NR-band power class |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2A.4: the Pcmax,f,c for each serving cell of the band combination is modified with the limits appliable when the UE is configured with inter-band UL CA. Isolated impact analysis: the change does not affect UEs implemented according to earlier versions of the specification (no new signaling). However, these UEs may not report the correct PH for serving cells when configured with UL CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PHR is overestimated when the power class of the band combination is lower that of the NR-band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

For a UE configured with uplink inter-band CA, the configured maximum output power PCMAX,c for each transmission occasion on serving cell c shall be set as specified in clause 6.2.4 modified by PEMAX,CA, and PPowerClass,CA as follows,

PCMAX_L,f,c = MIN {MIN(PEMAX,c, PEMAX,CA) – ∆TC,c,  MIN(PPowerClass – ΔPPowerClass, PPowerClass,CA) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass, PEMAX,CA, PPowerClass,CA}

where

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present;

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 for the band combination without taking into account the tolerance specified in the Table 6.2A.1.3-1.

The MPRc and A-MPRc apply per serving cell c and transmission occasion as specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,f,c  is calculated under the assumption that the transmit power is increased independently on all serving cells c.

The total configured maximum output power PCMAX shall be set within the following bounds:

< end of changes >