3GPP TSG-RAN WG4 Meeting #107                                           R4-2307762

Incheon, Korea, May 22 – 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On the co-existence study for NR duplex operation

Agenda Item:	8.20.2.1

Document for:	Approval

Introduction

In the last couple of RAN4 meetings, co-existence simulation assumptions have been discussed, related agreements can be found in [1~6]. Also, good progress for calibration have been arranged among companies as recorded in [7]. In this contribution, we will provide some simulation results for FR1 legacy TDD UL scenario.

Discussion

Preliminary simulation results

According to the latest discussion outcome, Urban Macro -> Urban Macro is the high priority scenario for both FR1 and FR2.

Table 1: Scenarios for SBFD co-ex study

| FR | Scenario No. | Deployment Scenario (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | TBD |
|  | 3 | Indoor -> Indoor | Low |
| FR2 (30GHz) | 4 | Urban Macro -> Urban Macro | High |
|  | 5 | Urban Hotspot -> Urban Hotspot | TBD |
|  | 6 | Urban Micro -> Urban Micro | Low |
|  | 7 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. |  |  |  |

In the following table, other information including victim/aggressor and SBFD configuration are provided.

Table 2: More details on the scenarios for SBFD co-ex study

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 | NR TDD DL | High |
|  |  | Case 3 | NR TDD DL | Low |
| NR TDD UL | SBFD(DUD) | Case 4 | NR TDD UL | Low |
|  | SBFD(DU) | Case 5 | NR TDD UL | Low |
|  |  | Case 6 | NR TDD UL | Low |
| SBFD (DUD) | NR TDD DL | Case 1 | No system in adjacent channel | High |
| SBFD (DU) | NR TDD DL | Case 2 |  | High |
|  |  | Case 3 |  | Low |
| SBFD(DUD) | NR TDD UL | Case 4 |  | Low |
| SBFD(DU) | NR TDD UL | Case 5 |  | Low |
|  |  | Case 6 |  | Low |
| Note: The above combination sets may be down-scaled if some sets are equivalent in SLS study perspective after agreed on other assumptions. |  |  |  |  |

We would like to provide simulation results for the following cases considering that UE ACLR model has been updated:

- FR1 TDD DL/UL as victim and SBFD as aggressor

- FR1 SBFD DL/UL as victim and TDD DL as aggressor

Figure 1. FR1 Uma SBFD (DU) co-ex with TDD: performance comparison from TDD DL perspective

Table 1. Updated results for FR1 Uma SBFD (DU) co-ex with TDD from TDD DL perspective

| Aggressor | Victim | Observation Point | SINR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- |
| NR SBFD {DU} 80MHz DL + 20MHz UL | NR TDD 100MHz DL | 5% | 0.08 | 0.1 |
|  |  | 50% | 0.01 | - |
|  |  | 95% | - | - |

Figure 2. FR1 Uma SBFD (DU) co-ex with TDD: performance comparison from TDD UL perspective

Table 2. Updated results for FR1 Uma SBFD (DU) co-ex with TDD from TDD UL perspective

| Aggressor | Victim | Observation Point | SINR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- |
| NR SBFD {DU} 80MHz DL + 20MHz UL | NR TDD 100MHz UL | 5% | 8.6 | - |
|  |  | 50% | 4 | 27 |
|  |  | 95% | 0.12 | 0.8 |

Figure 3. FR1 Uma SBFD (DU) co-ex with TDD: performance comparison from SBFD DL perspective

Table 3. Updated results for FR1 Uma SBFD (DU) co-ex with TDD from SBFD DL perspective

| Aggressor | Victim | Observation Point | SINR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- |
| NR TDD 100MHz DL | DL of NR SBFD {DU} 80MHz DL + 20MHz UL | 5% | 0.5 | 5.6 |
|  |  | 50% | 0.39 | 1.6 |
|  |  | 95% | 0.44 | 0 |

Figure 4. FR1 Uma SBFD (DU) co-ex with TDD: performance comparison from SBFD UL perspective

Table 4. Updated results for FR1 Uma SBFD (DU) co-ex with TDD from SBFD UL perspective

| Aggressor | Victim | Observation Point | SINR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- |
| NR TDD 100MHz DL | UL of NR SBFD {DU} 80MHz DL + 20MHz UL | 5% | - | - |
|  |  | 50% | 5 | - |
|  |  | 95% | 0.5 | 11 |

Based on the simulation results from the above tables, degradation can be found especially for UL. Thus we have the following observation.

Observation: For FR1 Uma scenario, Co-existence between SBFD with ‘DU’ configuration and legacy TDD system brings non-negligible degradation to the UL performance.

Conclusion

In this contribution we discussed on co-existence study for NR duplex operation. According to the contribution, we have the following observation:

Observation: For FR1 Uma scenario, Co-existence between SBFD with ‘DU’ configuration and legacy TDD system brings non-negligible degradation to the UL performance.

- References

- R4-2214378, “WF on adjacent channel co-existence study”, Samsung, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2217466, “WF on adjacent channel co-existence evaluation of SBFD operation”, Samsung, CMCC, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic meeting, October 10 – 19, 2022.

- R4-2220246, “WF for co-existence study”, CMCC, Samsung, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, 14 – 18, November, 2022.

- R4-2302888, “WF for SBFD adjacent channel co-existence study”, CMCC, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27 February – 03 March, 2023.

- R4-2305921, “WF on SBFD co-existence simulation”, CMCC, 3GPP TSG-RAN WG4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.

- R4-2302889, “Offline calibration collection and summary”, Samsung, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27 February – 03 March, 2023.