3GPP TSG-RAN WG4 Meeting # 107	R4-2307801

# Incheon, Korea (Republic of), May 22 – 26, 2023

Source:	LG Electronics

Title:	Discussion on co-channel coexistence for LTE SL and NR SL

Agenda item:	8.31.3.3

Document for:	Discussion

Introduction

In [1], RAN4 agreed to wait for RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL. In this contribution, we provide our views on this case based on RAN1 progress.

Discussion

Based on the RAN1 discussion, some agreements have been made in RAN1

<RAN1#112bis-e> Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module Agreement In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission Agreement For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain, ....

Based on overall RAN1 agreements, for dynamic resource pool sharing, no additional RRM impact is anticipated since RAN1 is trying to resolve the overlapping related issues in resource election between LTE SL and NR SL.

- Proposal: Do not introduce additional RRM requirements for co-channel coexistence for LTE SL and NR SL.

Conclusion

In this contribution, we provide views on co-channel coexistence for LTE SL and NR SL, and, we propose

- Proposal: Do not introduce additional RRM requirements for co-channel coexistence for LTE SL and NR SL.

Reference

[1] R4-2306364, “WF on NR_SL_enh2_part2,” OPPO.