3GPP TSG-RAN WG4 Meeting #107		R4- 2307805

Incheon, Korea, 22nd – 26th May, 2023

Agenda item:	8.10.3.1

Source:	Intel Corporation

Title:	Discussion on measurements without gaps for UE reporting NFG

Document for:	Discussion

# Introduction

Following the discussions on measurements without gaps for UE reporting NeedForGap, the uses cases are classified into two

- Case 1: without gap and no interruption (e.g. ’[TBD1]’ indicated in [TBD new signaling])

- Case 2: without gap but interruption allowed (e.g. ’[TBD1]’ indicated in [TBD new signaling])

In this paper we provide our views on how to specify the requirements for each of the cases.

# Discussion

## Interruptions

Interruption locations

In the last meeting, we have agreed on specifying the interruption length and ratio requirements for UE reporting NFG. But whether interruption locations are specified needs further discussions.

Firstly, given the difficulties to define the interruption locations as there is not any the specific measurement patterns in case of measurements without gap, we prefer to not to define this requirement. On the other hand, from NW and UE efficiency perspective, minimum interruption is preferred. With that said reasonable interruption ratios can avoid unacceptable outages of the system. Therefore, we propose to define the overall framework of interruption requirements with the interruption length and ratio but not to specify the interruption locations.

Proposal 1: Do not specify the interruption locations.

Interruption lengths and ratio

For the interruption length requirement itself, the methodology we had for NCSG in Rel17 can be reused. That is our proposal:

Proposal 2: As a starting point, the interruption length can be same as these defined for NCSG, e.g.

- When UE reporting “no-gap[TBD]” in [NeedForGapInfoNR]  the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

- When UE reporting “others[TBD]” in [NeedForGapInfoNR] no interruption allowed

For the requirements on the interruption ratio, it was agreed in the last meeting that

- Interruption ratio is defined as follows: - 80ms ≤ Tcycle < 160ms: up to [2.50%] probability of interruption - 160ms ≤ Tcycle < 320ms: up to [1.25%] probability of interruption - 320ms ≤ Tcycle: up to [0.625%] probability of interruption - FFS if the interruption rate can be captured in equation format - Do not define requirement for the case Tcycle < 80ms - FFS if interruption ratio applies to a single frequency layer or all frequency layers - Tcycle definition is FFS - Option 1: Tcycle = SMTC x CSSF x Kp - Other options are not precluded

For the definition of Tcycle, we prefer to consider all factors in the option 1: SMTC, CSSF and Kp. That is to say that the interruption upper bound is dependent on the target frequency carrier.

## Measurement reporting delay requirements

It is agreed that when RAN4 specifies the measurement requirements for intra/inter-freq measurement without gap when interruptions are allowed (case 2), the following key aspects need to be updated at least.

- Update the definition of intra/inter-frequency SSB based measurements without measurement gaps to include the cases when UE indicates ‘nogap-withinterruption[TBD]’ via ‘needForGap-r18[TBD]’

- Update the scaling factor because of the measurement gap overlapping (Kp)

- Updates on CSSFoutside_gap

- Updates on Klayer1_measurement

Now that RAN4 has agreed on the interruption requirements with the interruption ratio and length, in principle the measurement requirements can be specified as the similar way as those for deactivated SCell measurement without gap in TS38.133 Table 9.2.5.2-3/9.2.5.2-4.

For the measurement period requirements of intra/inter-freq measurements without gap when interruption allowed (case 2), the most essential difference with these of measurements without gaps on deactivated SCell is the “measCycleSCell” which is invalid any more. And the corresponding requirements for case 2 needs to update the measurement cycle.

Proposal 3: The measurement period requirements of intra-freq measurements without gap when interruption allowed (case 2) in Rel18 can be defined as

Table 9.x.y.z-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Max(200ms, Ceil(5 x Kp) x measCycleNFG) x CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, Ceil(5 x Kp) x max(measCycleNFG, 1.5xDRX cycle)) x CSSFintra |
| DRX cycle> 320ms | Max(200ms, Ceil(5 x Kp) x max(measCycleNFG, DRX cycle)) x CSSFintra |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Table 9.x.y.z-2: Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Max(400ms, Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleNFG ) x CSSFintra |
| DRX cycle≤ 320ms | Max(400ms, Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleNFG, 1.5xDRX cycle)) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleNFG, DRX cycle) x CSSFintra |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Proposal 4: The measurement period requirements of inter-freq measurements without gap when interruption allowed (case 2) in Rel18 can be defined as

Table 9. x.y.zz-1: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x measCycleNFG)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(measCycleNFG,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Table 9. x.y.zz-2: Measurement period for inter-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x measCycleNFG)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(measCycleNFG,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

There is no clear definitions on CSSFoutside_gap for the measurement without when UE reports ‘no-gap’ via ‘needForGapInfoNR’ in current spec [3]. Thus we propose to clarify the CSSFoutside_gap in 9.1.5.1 of TS38.133. Updates/Clarification on CSSFoutside_gap when defining the requirements for case 1 is needed.

Proposal 5: The measurement period requirements of intra/inter-freq measurements without gap and no interruption (case 1) in Rel18 can be defined by reusing the existing requirements in Section 9.2.5 / 9.3.9 of TS38.133 respectively with the necessary updates on CSSFoutside_gap in 9.1.5.1 of TS38.133.

## UE behaviours

Generally, we think that there was some overlap or ambiguity when UE support both NeedForGap and NCSG for the possible gap-less measurements. For an example, when there is a vacant RF chain UE can indicate either of the following options to perform the measurement outside of measurement gap.

Alt 1. ‘no-gap’ via “needForGapsInfoNR”

Alt 2. ‘ncsg or nogap-noncsg’ via “needForNCSG-InforNR’

Thus, whether UE needs to distinguish them for the same purpose can be FFS. More importantly, for the possible enhancements, RAN4 shall be investigate the potential impacts on legacy UE. To simplify RAN4’s works, Option 2/2a is more preferable.

Proposal 6: No need to establish the mapping between UE’s indication for NeedForGaps and NCSG.

# Conclusion

Proposal 1: Do not specify the interruption locations.

Proposal 2: As a starting point, the interruption length can be same as these defined for NCSG, e.g.

- When UE reporting “no-gap[TBD]” in [NeedForGapInfoNR]  the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

- When UE reporting “others[TBD]” in [NeedForGapInfoNR] no interruption allowed

Proposal 3: The measurement period requirements of intra-freq measurements without gap when interruption allowed (case 2) in Rel18 can be defined as

Table 9.x.y.z-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Max(200ms, Ceil(5 x Kp) x measCycleNFG) x CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, Ceil(5 x Kp) x max(measCycleNFG, 1.5xDRX cycle)) x CSSFintra |
| DRX cycle> 320ms | Max(200ms, Ceil(5 x Kp) x max(measCycleNFG, DRX cycle)) x CSSFintra |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Table 9.x.y.z-2: Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Max(400ms, Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleNFG ) x CSSFintra |
| DRX cycle≤ 320ms | Max(400ms, Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleNFG, 1.5xDRX cycle)) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleNFG, DRX cycle) x CSSFintra |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Proposal 4: The measurement period requirements of inter-freq measurements without gap when interruption allowed (case 2) in Rel18 can be defined as

Table 9. x.y.zz-1: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x measCycleNFG)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(measCycleNFG,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Table 9. x.y.zz-2: Measurement period for inter-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x measCycleNFG)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(measCycleNFG,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: measCycleNFG is the measurement cycle when UE supported [no-gap-with-interruption] |  |

Proposal 5: The measurement period requirements of intra/inter-freq measurements without gap and no interruption (case 1) in Rel18 can be defined by reusing the existing requirements in Section 9.2.5 / 9.3.9 of TS38.133 respectively with the necessary updates on CSSFoutside_gap in 9.1.5.1 of TS38.133.

Proposal 6: No need to establish the mapping between UE’s indication for NeedForGaps and NCSG.

# References