3GPP TSG-RAN WG4 Meeting # 107	R4-2307811

# Incheon, Korea (Republic of), May 22 – 26, 2023

Source:	LG Electronics

Title:	Discussion on L1-RSRP measurement for FR2 multi-Rx chains

Agenda item:	8.8.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on L1-RSRP measurement for multi-Rx chains based on [1].

Discussion

Beam sweeping/Sharing/Scaling factor reduction

In the last meeting, overall agreements for beam sweeping factor reduction for L1 RSRP measurements were derived as follows.

Agreement from GTW: - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities Way forward: Please bring further analysis on the FFS parts of the above GTW agreement to the next meeting. Agreements: How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage.

if group-based reporting is configured, a UE reports the best beam pairs based on each resource set in consideration of simultaneous reception. To find the best beam pair, a UE may need to perform full Rx beam sweeping to measure the signal from each TRP since each UE antenna panel is not paired to each TRP. If the multi-Rx chain is activated during the measurement or evaluation period, the UE can receive measurement resource signals using the multi-Rx chain depending on UE implementation. In that case, Rx beam sweeping factor for measurement or evaluation period can be reduced. Additionally, depending on Rx beam coverage for multi-panels, Rx beam sweeping factor could be reduced. However, it is difficult to define specific conditions to reduce Rx beam sweeping factor since most of them are related to UE implementation. Therefore, the capability for the number of Rx beam sweeping might be needed, and it should be applied while multi-Rx chains are active.

- Proposal 1: Introduce reduced Rx beam sweeping factor for L1 measurements along with the capability of supported Rx beam sweeping factor while multi-Rx chains are active.

Group based beam reporting (GBBR) requirements

For the requirements for GBBR, RAN4 has discussed for following issues.

Issue 1-3-2-1: How to perform measurement. That means the measurement delay required for GBBR (i.e., measurement delay for beam pair) - Option 1: Reuse legacy L1-RSRP delay as UE do not know which beams can be paired. - Option 2: Due to simultaneous reception measurement delay for beam pair can be enhanced for L1-RSRP Issue 1-3-2-2: How to choose beam pair that are going to be reported based on measurement results - Option 1: define conditions for selecting beam pair in the report - Please provide conditions that can be considered - Option 2: How to choose beam pair is UE implementation Issue 1-3-2-3: NW configures UE with GBBR, but UE did not find a good beam pair to report. What should be the UE behaviour - Option 1: NO behaviour is needed - Option 2: report NA or single beam - Option 3: Check with RAN1 - Note: if we agree for this, we may need to define what are the conditions for beam pair selection as we need to define what is not finding good beam pair. Issue 1-3-4: Which reference signals to be considered for GBBR configuration Agreements: SSB+SSB or CSI-RS + CSI-RS can be supported for GBBR. Way forward: FFS if SSB+CSI-RS is supported for GBBR. Issue 1-3-5: If CSR-RS is configured for GBBR FFS whether CSI-RS resource with repetition ON will be configured for group-based reporting. Issue 1-3-6: Type of reporting used for GBBR FFS if Periodic, semi-persistent and aperiodic report are supported

For Issue 1-3-2-1 above, no specific requirements for GBBR are required, but some updates could be considered based on the existing L1-RSRP delay. In the last meeting, RAN4 sent LS to RAN1 related to the above issue, so we prefer to wait for RAN1 response to finalize whether to enhance L1-RSRP requirements.

- Proposal 2: Wait for RAN1 response to finalize whether to enhance L1-RSRP requirements

Conclusion

In this contribution, we provide our views on L1-RSRP measurement for multi-Rx, and we propose

- Proposal 1: Introduce reduced Rx beam sweeping factor for L1 measurements along with the capability of supported Rx beam sweeping factor while multi-Rx chains are active.

- Proposal 2: Wait for RAN1 response to finalize whether to enhance L1-RSRP requirements

Reference

[1] R4- 2306319, “WF on FR2_multiRx_part2,” Ericsson.