3GPP TSG-RAN4 Meeting #107	R4-2307816

Incheon, KR, May 22 – May 26, 2023

Source:	Samsung

Title:	discussion and simulation results on 8Rx PDSCH requirements

Agenda item:	8.5.3.1.2

Document for:	Discussion

- Introduction

In last RAN4 #106bis-e meeting, general test scope, simulation assumptions for FR1 8Rx PDSCH requirements have some agreements in WF [1]. In this contribution, we share our views on below open issues of FR1 PDSCH requirements.

Issue 2-3: Propagation conditions for Rank 2 test - Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B - Option 2: Only TDLC300-100 ULA Medium B Issue 2-4: MCS and antenna correlation for Rank 2 test - Proposals for TDLC300-100 ULA Medium B - Option 1: MCS 2, 7 (Table 2) - Option 2: MCS 19 (Table 1) - Proposals for TDLA30-10 ULA Medium B (if agreed) - Option 1: MCS 13, 26 (Table 2) - Option 2: MCS 19 (Table 1) Issue 2-5: MCS for Rank 4 test - Option 1: MCS 13 and MCS26 (Table 1) - Option 2: Only MCS17 (Table 1) Issue 2-7: Single MCS configuration for Rank 8 test - Option 1:MCS13 - Option 2: MCS17

- Discussion

Propagation conditions for Rank 2 test

As specified by the objective in [2] as following

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include: - Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices - Specify the BS demodulation performance requirements to support UL 4-layer MIMO UE operation - Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify RLM test cases to support 8Rx - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers - Specify release independence requirements in TS 38.307 if needed.

Due to the support requirement of vehicle devices, companies propose to introduce TDLC300-100 propagation condition. But TDLA propagation condition is still needed for other devices, thus we prefer to introduce two test cases for rank 2.

Proposal 1: Introduce two test cases for rank 2, TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B.

MCS and antenna correlation for Rank 2 test

According to the simulation results in [3], considering 8Rx is supported, which means high SNR could be easily reached,  and current rank 2 test cases for 4Rx requirements is also based on MCS19 (Table 1). Therefore, we propose to use MCS19 (Table 1) for Rank 2 test.

Proposal 2: Use MCS19 (Table 1) for TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium Rank 2 tests.

MCS for Rank 4 test

According to the simulation results in [3], the SNR point @70% maximum throughput of MCS17 (Table 1) is more suitable than that of MCS13 (Table 1).

Proposal 3: option 2, use MCS17 (Table 1) for Rank 4 test.

Single MCS configuration for Rank 8 test

According to the simulation results in [3], the SNR point @70% maximum throughput of MCS17 (Table 1) seems too high to reach, thus we prefer to use MCS13 (Table 1) for rank 8 test.

Proposal 4: option 1, use MCS13 (Table 1) for Rank 8 test.

- Simulation results

Simulation assumptions and configuration details could refer [3] in RAN4 #106bis-e meeting. Here we list our simulation results in general.

Table 3-1 SNR point of 70% maximum throughput for rank2, 4, 8 cases

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 6.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 7.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.42 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

According to the simulation results in [3], considering the impairment margin, we propose to introduce PDSCH performance requirements for 8Rx.

Proposal 5: introduce PDSCH performance requirements for 8Rx as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 10.37 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.42 |
| 2-1 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 10.93 |
| 3-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 14.73 |

- Conclusion

In this contribution, we provide analysis and views on 8Rx UE demodulation requirements for remaining open issues. The proposals could be summarized as:

Proposal 1: Introduce two test cases for rank 2, TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B.

Proposal 2: Use MCS19 (Table 1) for TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium Rank 2 tests.

Proposal 3: option 2, use MCS17 (Table 1) for Rank 4 test.

Proposal 4: option 1, use MCS13 (Table 1) for Rank 8 test.

Proposal 5: introduce PDSCH performance requirements for 8Rx as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 10.37 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.42 |
| 2-1 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 10.93 |
| 3-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 14.73 |

- Reference

[1] R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106bis-e meeting

[2] RP-221496, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei, HiSilicon, 3GPP TSG-RAN #96 meeting

[3] R4-2304900, “simulation results on 8Rx UE demodulation requirements”, Samsung, 3GPP RAN4 #106bis-e meeting