3GPP TSG-RAN WG4 Meeting # 107	R4-2307878

# Incheon, Korea (Republic of), May 22 – 26, 2023

Source:	LG Electronics

Title:	Discussion on guard period in TDD cell for ATG operation

Agenda item:	8.14.4.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on guard period due to large TDD cell based on [1].

Discussion

Guard period issue due to large TDD cell and coexistence demand

In last meeting, following options for guard period due to large TDD cell in AGT was discussed.

Issue 3-2-1: Whether co-existence and overhead issues due to large GP need to be addressed in R18 ATG - Option 1: - Large TDD cells are inefficient due to large GP, if we switch often. - The largest cell radius will define the GP for all TDD cells and UL/DL configuration has to be the same for all coexisting (overlapping) TDD networks - Option 2: How to configure a suitable GP is up to network. There is no need to discuss it.

The most critical parameter to deciding the guard period is the propagation delay regarding cell size. In the ATG case, the cell radius can be larger than 200km for ISD 200km captured above, so a longer guard period would be required based on the principle to derive a guard period. However, if a fixed longer guard period is considered, it is very inefficient for system performance since depending on ATG UEs’ location, the required guard period would be different as shown in Fig1. Unlike the TN system, the following features and characteristics for ATG should be considered.

- ATG gNB

  - The distance between ATG gNBs can be larger than 200km

  - The antenna of ATG gNB is tilted up.

- ATG UE

  - Directional antenna is assumed for 4GHz frequency

  - The minimum distance between aircrafts (ATG UE) is about 9.2km from separation standards of aircraft

- The number of ATG UE in a ATG cell

  - Based on the minimum distance between ATG UE, the number of ATG UEs would be limited in a cell

Based on the listed above, the impact of ATG gNB and ATG gNB / ATG UE to ATG UE interference at the DL to UL switch could be less. Therefore, instead of a fixed longer guard period, it is more efficient that the ATG UE reports the propagation delay information to ATG gNB, so that DL or UL scheduling restrictions can be used by ATG gNB based on the information. And even if how to configure a suitable guard period is up to network as option 2 above, the information should be considered when setting up a guard period to avoid unnecessary long guard period.

Figure 1 Example of UE to UE interference due to propagation delay

- Proposal 1: The propagation delay information between UE and gNB should be considered when setting up a guard period to avoid unnecessary long guard period even if how to configure a suitable guard period is up to network. FFS for how to capture this in the specification.

Conclusion

In this contribution, we provide the views on guard period due to large TDD cell, and we propose

- Proposal 1: The propagation delay information between UE and gNB should be considered when setting up a guard period to avoid unnecessary long guard period even if how to configure a suitable guard period is up to network. FFS for how to capture this in the specification.

Reference

[1] R4-2306344, “WF on NR ATG RRM requirements,” CMCC