3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308016

# Incheon, May 22 – May 26, 2023

Agenda item:	8.9.2.1

Source:	China Telecom

Title:	Discussion on L3 part enhancement for FR2 SCell activation

Document for:	Discussion

- Introduction

In RAN4 #106-bis-e meeting, FR2 SCell activation delay reduction solutions were discussed further and the WF [1] was approved.

In this paper, we present our views about L3 part enhancement for FR2 SCell activation issues captured in the WF [1].

- Discussion

During the unknown FR2 SCell activation process, AGC tuning, cell search, L1-RSRP measurement and report, fine timing tracking process are required, which cause the large delay. L3 part of SCell activation mainly includes AGC tuning and cell search in order to obtain coarse timing and beam information. For SCell activation delay in L3 part, we discuss the potential delay reduction enhancement and present our views about these issues.

L3 reporting during SCell activation

Status in the WF [1]:

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available? FFS: - Option 1 (Apple, OPPO, Huawei, Xiaomi, vivo): - No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement. - Option 2 (Nokia, Ericsson): - The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). - Option 3 (Intel): - If UE already has available L3 measurement result when Scell activation command is triggered, the measurement result can be reported only if it satisfied the following conditions: - Measurement should satisfy the requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) - the time duration between measurement and Scell activation command is less than [X]s. - Option 4 (MTK): - RAN4 to define a new window [W]s to determine the freshness of the available L3-RSRP measurements at the UE prior to the reception time of Scell activation command. - Proposal 6: If L3-RSRP measurements are within [W], then the UE can send L3-RSRP report. - Option 6 (vivo): - The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network - Option 7 (CMCC, ZTE): - for L3 measurement report after Scell activation command, the reported SS-RSRP, SS-RSRQ, and SS-SINR measurements need to meet the accuracy requirements spedified in Clause 10. - Option 8 (ZTE): - To address the concern of the quick report is valid or not, the verification lies in two aspects: - 1)Whether the measurement report is fresh enough. - 2)Whether the quality of measurement result in the report is good or available enough. - The 1st aspect can be ignored since same issue exists in legacy L3 report. The 2nd aspect should be considered to guarantee the NW can obtain useful RSRP and SSB index info from the report. - Option 9 (Apple, CMCC, LGE, CTC, QC, OPPO): - Option 1 and the valid reporting must meet the existing measurement delay/accuracy requirement. Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results? FFS: - Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results. - Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation. - Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network. - Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’. - Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell. Issue 1-1-8: FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported. FFS: - Option 1 (Apple, Nokia, Intel, QC, MTK, LGE, Xiaomi, vivo, CMCC, ZTE, CTC, OPPO): - if UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - Proposal 1a (Apple, CTC, QC): after receiving SCell activation command, - if UE reports valid L3 measurement result before n+X slot (n is the slot when UE received SCell activation command), L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - otherwise, it shall be treated as legacy unknown case. - Proposal 1b(QC, CTC): - RAN4 shall define the reporting delay requirements from MAC-CE to trigger report and actual measurement report. If UE do not report measurement results during the reporting delay requirements, the new measurement configuration shall be expired. - Proposal 1c(vivo): - Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires. - Proposal 1d(OPPO): - FFS the side conditions. - Proposal 1e(Nokia): - A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command. - Proposal 1f(Xiaomi): - If no valid result to be reported, the UE needs to perform L3 measurement and L1 measurement with/without enhanced requirement according to UE capability. - Option 2 (HW, vivo, Apple (bullet #2)): (moderator: this option is also up to the conclusion in issue 1-1-1) - When the new triggered measurement reporting is reported before SCell activation command, it can be considered as known cases. - When the new triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative to the first valid L1-RSRP reporting and the new triggered L3 measurement report which occurs earlier. - Option 3 (Ericsson): - RAN4 to agree that L3 part can be skipped and whether L1 part can be skipped or not pending on whether NW sends TCI command or not. - Option 4 (moderator): - use option 1 as starting point, - FFS on the detailed requirement design, e.g. option 2.

For the issue on how to determine the measurement result is available, we think the valid reporting must meet the existing measurement delay/accuracy requirement and it’s not necessary to introduce additional criteria to determine whether the L3 measurement result is available or not for FR2 unknown SCell activation enhancement.

Proposal 1: The valid reporting must meet the existing measurement delay/accuracy requirement.

For the issue on necessity of L3 measurement reporting if UE has no valid measurement results, we think it’s not necessary to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results, and UE performs L3 part related procedure as legacy defined.

Proposal 2: It’s not necessary to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

If valid L3 measurement results are reported, in our views, the overall SCell activation delay can be reduced by skipping the L3 part and L1 part, and NW can perform TCI activation after valid L3 measurement results are reported. We think UE needs to report valid L3 measurement result within a time threshold after SCell activation command, e.g., UE needs to report valid L3 measurement result before n+X slot, and n is the slot when UE received SCell activation command.

Proposal 3: When the valid L3-RSRP measurement results are reported, L3 and L1 parts can be skipped.

Proposal 4: UE needs to report valid L3 measurement result within a time threshold after SCell activation command.

Beam related enhancement for L3 part

Status in the WF [1]:

Issue 1-2-1: Beam sweeping factor enhancement in L3 and L1 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) Agreement: - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Note: above enhancement only applies for FR2 unknown SCell activation enhancement - Note: How to capture in spec can be discussed in CR stage - The candidate values for X1/X2 are FFS Issue 1-2-2: can X1(Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation) be zero? (not related with WI of FR2 multi-Rx chain DL reception) The discussion is related with the previous agreement: Agreement in last meeting R4-2303228 Issue 1-3-3: enhancement of “8*Trs” part of current FR2 unknown Scell activation delay Agreement: - To keep “X1*Trs” part of current FR2 unknown Scell activation delay in the delay requirement for FR2 Scell activation enhancment. - X1 can be less than 8 in the beam sweeping factor reduction discussion for cell detection stage in L3 part based on UE capability.. FFS: - Option 1 (Apple, MTK, Xiaomi, HW (option 4b in issue 1-2-1 comments), Intel, QC, OPPO): no - Option 2 (Ericsson, vivo, Nokia, HW (option 4a in issue 1-2-1 comments)): Yes

As agreed in the WF [1], beam sweeping factor capability X1/X2 are two independent capabilities, as for the value of X1 and X2, lower Rx beam sweeping factor for Positioning can be the baseline, e.g., X1 and X2 can be {1,2,4,6}.

Proposal 5: For the value of X1 and X2, lower Rx beam sweeping factor for Positioning can be the baseline, e.g., X1 and X2 can be {1,2,4,6}.

In our views, we shall follow the previous agreement to keep “X1*Trs” part of current delay requirement and to reduce Rx beam sweeping factor for cell detection stage in L3 part based on UE capability.

Proposal 6: Follow the previous agreement to keep “X1*Trs” part of current delay requirement and to reduce Rx beam sweeping factor for cell detection stage in L3 part based on UE capability.

- Conclusion

The contribution presents our views on the solutions of unknown SCell activation delay reduction, with the following proposals:

Proposal 1: The valid reporting must meet the existing measurement delay/accuracy requirement.

Proposal 2: It’s not necessary to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

Proposal 3: When the valid L3-RSRP measurement results are reported, L3 and L1 parts can be skipped.

Proposal 4: UE needs to report valid L3 measurement result within a time threshold after SCell activation command.

Proposal 5: For the value of X1 and X2, lower Rx beam sweeping factor for Positioning can be the baseline, e.g., X1 and X2 can be {1,2,4,6}.

Proposal 6: Follow the previous agreement to keep “X1*Trs” part of current delay requirement and to reduce Rx beam sweeping factor for cell detection stage in L3 part based on UE capability.

- References

- R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction, Apple, RAN4 #106bis-e, April 2023.