3GPP TSG-RAN WG4 Meeting #107 	R4-2308023

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.1

Source: 	Ericsson

Title:	On general aspects

Document for:	Discussion

# Introduction

The agreements from RAN4#106bis-e are summarized in [12] and [13] for the 1st and 2nd threads, respectively. And the discussion summaries are available in [14] and [15] for the 1st and 2nd threads, respectively.

# Discussion

## Overheating indication

In the last meeting, it was agreed to not introduce power saving specific requirements in this WI:

Issue 1-2-5a: Whether to define power saving related requirements

<Agreement >:

- No power saving specific requirements, e.g., L1 measurements relaxation for multi-Rx operation, are considered in the WI.

In addition, there has been a discussion on whether to introduce indication of multi-rx operation:

Issue 1-2-5: Indication of multi-Rx operation

< Way forward >:

- Proposals

  - Option 1: (Nokia, Huawei, LGE, Intel, ZTE, vivo, Samsung, Apple)

    - Introduce mechanism/condition for indication of multi-Rx operation, including on/off indication of multi-Rx operation.

    - FFS on mechanism/condition for indication of multi-Rx operation

  - Option 2: (MTK, OPPO)

    - No new mechanism is needed for UE to fallback from multi-Rx to single Rx.

In our view, the existing overheating indication mechanism can be further extended for multi-rx operation to account for a possible overheating of the UE enabled with multi-RX chain operation over a long period of time. The existing RAN2 specification (e.g. TS 38.331) already specified an overheating related IE (OverheatingAssistance) which allows the UE to send overheating assistance information for some applications to the network. Considering this type of functionality is already available, we support extending the existing overheating assistance functionality also for multi-RX operation. However, the UE behaviour of turning OFF the multi-RX featuring and turning back ON the multi-RX functionality upon sending the OverheatingAssistance information to the network should still be within the network control. We think RAN4 needs to further discuss mechanism to enable multi-RX ON/OFF to ensure efficient use of power of multi-RX while preventing the overheating of the UE.

- Proposal 1 (overheating indication): Extend OverheatingAssistance mechanism to also cover multi-RX chain operation and further discuss the details for multi-RX chain ON/OFF mechanism based on OverheatingAssistance.

  - If needed, RAN4 can send LS to RAN2 to inform about the RAN4 conclusion.

## Different numerology with the simultaneous RS reception

The related agreements/discussion from RAN4#106bis-e are:

Issue 1-4-3a: UE capability of simultaneousRxDataSSB-DiffNumerology-Inter-r16

<Agreement >:

- No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI

Issue 1-4-3: UE capability of simultaneousRxDataSSB-DiffNumerology

<Way forward >:

- Proposals

  - Option 1:

    - The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI.

  - Option 2:

    - The existing UE capability simultaneousRxDataSSB-DiffNumerology is feasible if the supporting of concurrent SSB and data are consistent between FR1 and FR2-1. But it should be ultimately determined after the decision of single or multiple UE capabilities are necessary.

  - Option 3:

    - No need to specify new mix-numerology capability for multi-RX chain in FR2 when consider simultaneous SSB and data reception.

  - Option 4:

    - A new UE capability is needed to support simultaneous reception with mixed numerologies.

  - Option 5:

    - The existing UE capability to support mixed numerologies is not directly applicable for multi-RX chain operation with mixed numerologies, hence a new UE capability is needed to support simultaneous reception with mixed numerologies.

The existing relevant UE capability is below, which is, however, defined independently of the existing capability for simultaneous reception of data and RS with same numerology but with different QCL Type-D. Furthermore, the same principle is likely to apply for multi-rx, i.e., the general FR2 multi-rx capability will likely not cover the mixed numerologies case.

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

However, two separate capabilities (the existing capability simultaneousRxDataSSB-DiffNumerology and the general FR2 multi-rx capability) do not necessarily imply that the UE will also support the mixed numerologies case under multi-rx operation. Hence, a new capability is needed to define the support for the mixed numerologies under multi-rx.

As per our understanding, in real deployments for FR2, RS and data may be using different numerology (e.g., CSI-RS/SSB using 120 kHz and CSI-RS/data using 60 kHz). Considering that this is most practical deployment, we think a new UE capability may have to be defined.

- Proposal 2 (mixed numerologies): Simultaneous reception with mixed numerologies should be supported.

- Proposal 3 (mixed numerologies): RAN4 agrees that the existing UE capability simultaneousRxDataSSB-DiffNumerology is not sufficient to cover the mixed numerologies support under multi-rx operation.

- Proposal 4 (mixed numerologies): A new UE capability is needed to support simultaneous reception with mixed numerologies.

Assuming new capability is defined, RAN4 may need to update the scheduling restrictions of the existing specification as the scheduling restrictions defined in current specification assumes that the UE can receive from only single beam direction. When UE can receive from different directions, at least in some cases, we may not need to specify scheduling restrictions.

- Proposal 5 (mixed numerologies): For UEs supporting simultaneous reception with mixed numerologies no scheduling restrictions are needed.

## On multi-rx enhancements for L3 measurements

In RAN4#106, it was agreed that simultaneous L3+L1 measurements are not supported in this release for multi-rx.

However, it is still open whether simultaneous reception of L3+L3 are supported for multi-rx in Rel-18.

- Proposal 6 (L3+L3): Simultaneous reception for two L3 measurements on different panels/from different directions is not supported in R18 multi-Rx chain WI.

## Multi-rx requirements applicability conditions

RAN4 needs to collect and discuss under which conditions, the multi-rx RRM requirements shall apply.

- Proposal 7 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE has the multi-rx operation capability (to be replaced with the exact capability name, with a relevant reference in the specification).

- Proposal 8 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE is configured with dual TCI.

- Proposal 9 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE is not configured with CA or DC.

- Proposal 10 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The simultaneously received RSs are in PCell only.

- Proposal 11 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - Both RSs and their associated signals in the QCL type D infos are detectable during the entire measurement period.

For two RSs, interference levels may differ a lot in overlapping resources and non-overlapping resources, i.e., much higher in the overlapping resources. Hence, the side conditions have to be applied in the overlapping resources, which is the worst case. Otherwise, more advanced receivers may be needed to deal with the high interference in the overlapping resources.

- Proposal 12 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The applicable side conditions are the conditions during the overlapping RS occasions (the conditions can be better in the non-overlapping occasions).

- Proposal 13 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The measured RS is being received simultaneously with another RS, where the two RSs have QCL-TypeD with different references, in at least one overlapping measurement occasion during the measurement period.

## UE behaviour when an applicability condition is violated

For the conditions which are not static (unlike the UE multi-rx capability), RAN4 needs to also discuss what happens, when at least one of the necessary conditions are not met, particularly if this happens during an on-going measurement with multi-rx.

- Proposal 14 (violated applicability condition): When at least one of the conditions becomes violated, the UE may or may not be able to complete the multi-rx measurement, e.g.:

  - may need to stop and restart (in new conditions) the RS measurements, at least for some of the conditions (e.g., when TCI configuration changes, PCell changes).

  - continues the RS measurement using a single panel, but more relaxed requirements apply for such measurement, at least for some of the conditions,

  - may need to complete the measurement before switching to the operation mode where the condition is violated, at least for some of the conditions (e.g., UE receives CA configuration).

## Simultaneous RS reception with different QCL D’s configured together with QCL A/C

We think that QCL type D in combination with QCL type A/C should not be excluded in the requirements for simultaneous RS reception.

- Proposal 15 (QCL D+A/C): Scenarios where QCL type D is configured in combination with QCL type A/C should be also covered by the requirements for simultaneous RS reception.

- Proposal 16 (QCL D+A/C): The same RRM requirements for simultaneous RS reception can apply for RS configured with:

  - QCL type D,

  - QCL type D + QCL type A, or

  - QCL type D + QCL type C.

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1 (overheating indication): Extend OverheatingAssistance mechanism to also cover multi-RX chain operation and further discuss the details for multi-RX chain ON/OFF mechanism based on OverheatingAssistance.

  - If needed, RAN4 can send LS to RAN2 to inform about the RAN4 conclusion.

- Proposal 2 (mixed numerologies): Simultaneous reception with mixed numerologies should be supported.

- Proposal 3 (mixed numerologies): RAN4 agrees that the existing UE capability simultaneousRxDataSSB-DiffNumerology is not sufficient to cover the mixed numerologies support under multi-rx operation.

- Proposal 4 (mixed numerologies): A new UE capability is needed to support simultaneous reception with mixed numerologies.

- Proposal 5 (mixed numerologies): For UEs supporting simultaneous reception with mixed numerologies no scheduling restrictions are needed.

- Proposal 6 (L3+L3): Simultaneous reception for two L3 measurements on different panels/from different directions is not supported in R18 multi-Rx chain WI.

- Proposal 7 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE has the multi-rx operation capability (to be replaced with the exact capability name, with a relevant reference in the specification).

- Proposal 8 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE is configured with dual TCI.

- Proposal 9 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - UE is not configured with CA or DC.

- Proposal 10 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The simultaneously received RSs are in PCell only.

- Proposal 11 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - Both RSs and their associated signals in the QCL type D infos are detectable during the entire measurement period.

- Proposal 12 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The applicable side conditions are the conditions during the overlapping RS occasions (the conditions can be better in the non-overlapping occasions).

- Proposal 13 (applicability conditions): The following is a necessary condition in multi-rx requirements:

  - The measured RS is being received simultaneously with another RS, where the two RSs have QCL-TypeD with different references, in at least one overlapping measurement occasion during the measurement period.h

- Proposal 14 (violated applicability condition): When at least one of the conditions becomes violated, the UE may or may not be able to complete the multi-rx measurement, e.g.:

  - may need to stop and restart (in new conditions) the RS measurements, at least for some of the conditions (e.g., when TCI configuration changes, PCell changes).

  - continues the RS measurement using a single panel, but more relaxed requirements apply for such measurement, at least for some of the conditions,

  - may need to complete the measurement before switching to the operation mode where the condition is violated, at least for some of the conditions (e.g., UE receives CA configuration).

- Proposal 15 (QCL D+A/C): Scenarios where QCL type D is configured in combination with QCL type A/C should be also covered by the requirements for simultaneous RS reception.

- Proposal 16 (QCL D+A/C): The same RRM requirements for simultaneous RS reception can apply for RS configured with:

  - QCL type D,

  - QCL type D + QCL type A, or

  - QCL type D + QCL type C.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.