3GPP TSG-RAN WG4 Meeting #107 	R4-2308033

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM Impacts by CA in HST FR2 Enhanced

Agenda item:	8.13.4.2

Document for:	Discussion

Introduction

This paper discusses the open points on the topic of carrier aggregation as guided by the way forwarded agreed in the last meeting. The last meeting had the following open issues on CA topic in HST FR2 deployments [1]:

# Topic #2: Intra-band carrier aggregation (CA) scenario ## Sub-topic #2-2 Definition of HST FR2 CA requirements #### Issue 2-2-1: Discussion on the necessity of the requirements (1) PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements Agreements and way forwards: - The enhanced FR2 HST requirements for intra-frequency detection in Rel-17 FR2 HST WI are also applied for SCells inter-frequency cell detection if introduced. - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and - FFS RAN4 need to specify the requirements for time index detection. (6) SCell activation delay Way forward: - Check SCell activation delay and applicable conditions in HST scenario. (7) Inter-frequency measurement requirements in Idle mode Agreements: - Inter-frequency measurement requirements in Idle mode: - RAN4 introduce requirement in Rel-18 FR2 HST WI. - Rel-17 FR2 HST principles is reused to define inter-frequency measurement requirements. ## Sub-topic #2-3 Network signaling for Rel-18 FR2 HST CA Scenario Issue 2-3-1: HST signaling for CA enhancement Agreements: - RAN4 to postpone the agreement on network signalling until the CA requirements are defined.

The remaining open issues are defining the necessity of the requirements. This requires a specific discussion on the deployment to be supported such that only the necessary requirements can be defined. Moreover, as some requirements have been agreed in the previous meeting, the signaling related to the agreed requirements are discussed in this paper.

Discussion

Necessity of the requirements

This section discusses the subtopics, starting with the necessity of the requirements:

## Sub-topic #2-2 Definition of HST FR2 CA requirements #### Issue 2-2-1: Discussion on the necessity of the requirements (1) PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements Agreements and way forwards: - The enhanced FR2 HST requirements for intra-frequency detection in Rel-17 FR2 HST WI are also applied for SCells inter-frequency cell detection if introduced. - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and - FFS RAN4 need to specify the requirements for time index detection. (6) SCell activation delay Way forward: - Check SCell activation delay and applicable conditions in HST scenario. (7) Inter-frequency measurement requirements in Idle mode Agreements: - Inter-frequency measurement requirements in Idle mode: - RAN4 introduce requirement in Rel-18 FR2 HST WI. - Rel-17 FR2 HST principles is reused to define inter-frequency measurement requirements.

PSS/SSS and time index detection

For point (1) in the WF there are two FFSs

  - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and

  - FFS RAN4 need to specify the requirements for time index detection.

The time index detection is achieved through decoding the PBCH from the related cell. The time required for the time index detection defines the requirement. Previous studies analyzed that this is different between FR2 and FR1 cells. Rel. 17 have already defined time index detection requirement for FR2 cells. The Rel. 17 work have not specified any additional requirement and Rel. 18 work can re-use this view. The only limitation for time index detection is related to measurement gap configuration. This might require that a new time index detection shall be specified for Rel. 18.

In case it is agreed in Rel. 18 HST work that the supported deployment is down-scoped to SSBless deployment. As without the SSB broadcast by the CA cells, there will be no need to detect PSS/SSS or the time index.

Inter-frequency time index detection with measurement gap configuration and inter-frequency time period for PSS/SSS detection require further analysis for Rel. 18 HST FR2 deployments with SSB-based SCell measurements.

SCell activation delay

For point (6) in the WF the agreed way forward is:

- Check SCell activation delay and applicable conditions in HST scenario.

At RAN4#106 Toulouse meeting [2] the following agreement is reached:

Agreement: - RAN4 shall not specify requirements for deactivated SCell in FR2 HST scenario.

With the above agreement, it is clear that the scenario that covers the activation of de-activated SCell is not covered. The SCell whenever configured will be configured in an activated state. Thus, only scenario that applies for Rel. 18 HST work are the followings from TS 38.133 [3]:

- 8.3.4 Direct SCell Activation at SCell addition,

- 8.3.5 Direct SCell Activation at Handover

- 8.3.6 Direct SCell Activation at RRC Resume

Similarly, 8.3.9, 8.3.10 and 8.3.11 would be considered as also multiple SCells can be activated at the same time.

For further details on the scenario we need to look in to the current specification and the deployment assumptions of HST to see which activation time should be considered. The activation time for FR2 based SCells are defined with three different clauses listed below:

Clause 1:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3,

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

The clause 3.6.3 indicates that

For the requirements in RRC connected state specified in this version of the specification, UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in the same band in FR2. Otherwise, the UE is not supposed to satisfy any requirements for SCell

The above clause indicates that the beams for PCell and the SCells come from the same direction and the same spatial filter can be used to receive at the UE side. In case the SCells will broadcast SSBs considering collocated sites will be used for HST FR2 and UE is provided an SMTC for the target SCell, the clause 1 captures mostly the HST deployment assumption.

Clause 2:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

In case the SCells will not broadcast SSBs but the signals transmitted are quasi collocated with one active serving cell then the clause 2 would apply.

Clause 3:

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

Clause 3 applies for the case where there is no active serving cell in the same band as the SCells. It is unclear if this is a scenario to be supported for the HST in Rel. 18.

Currently, there are three requirements for SCell activation. In case there is no down-scoping of the considered scenarios, all the previous requirements should be considered for the Rel-18 FR2 SCell activation delay. Possible down-scoping would be:

- Support only SSBless scenario where an active serving cell exists in the same band.

- Support only SSB based scenario where an active serving cell exists in the same band.

- Do not support SSBless scenario..

- No restrictions..

However, the Rel. 18 HST work item description is limited to intra-band carrier aggregation.

Do not to consider CA for cells in a band different than the serving PCell as an applicable scenario for SCell activation delay.

RAN4 to consider CA for SSB-based and SSB-less scenario in the same band as the serving PCell as an applicable scenario for SCell activation delay.

Inter-frequency measurement requirements in IDLE mode

For point (7) the agreement is

- Inter-frequency measurement requirements in Idle mode:

  - RAN4 introduce requirement in Rel-18 FR2 HST WI.

  - Rel-17 FR2 HST principles is reused to define inter-frequency measurement requirements.

In order to set the scope of this work it is important to emphasize that cell re-selection enhancements are not considered in the scope of Rel. 18 work. Consequently, the idle mode measurement enhancements are meaningful for early measurement reporting (EMR) feature to enable faster dual connectivity and carrier aggregation.

The feature enables UE to conduct more measurements on idle/inactive mode and reporting them once UE goes back to connected mode to enable high throughput as fast as possible.

The EMR feature has limited benefit for FR2 and currently in Rel. 18 this feature is enhanced to enable validity of the measurements for a longer time. The feature is beneficial if UE stays in a short while in idle/inactive mode for FR2 scenario.

In case there is no interest to support this optional feature such enhancements can be unnecessary.

IDLE mode enhancements can be useful only for optional early measurement reporting (EMR) feature, if supported by the Rel-18 HST FR2 CPEs.

EMR feature can be optionally supported by Rel-18 HST FR2 CPE capable of CA.

Network signalling for Rel 18 FR2 HST CA

In last meeting the following agreement has been taken on the signalling:

## Sub-topic #2-3 Network signaling for Rel-18 FR2 HST CA Scenario Issue 2-3-1: HST signaling for CA enhancement Agreements: - RAN4 to postpone the agreement on network signalling until the CA requirements are defined.

The last meeting has progressed on some of the CA requirements for those that can be re-used from Rel. 17. This means that the signaling discussion for re-using the requirements can be discussed.

Before going further with the discussions, we re-cite the previous options discussed for signalling [2].

Way forward: - HST signaling for CA enhancement: - Option 1: RAN4 postpones the agreement on network signaling until the CA requirements are defined - Option 2: Reuse PCell signaling for SCell in HST - Option 3: Other options are not precluded

In Rel-17, new HST FR1 CA requirements can be identified in TS 38.133 with the highSpeedMeasCA-Scell-r17 and UE supports measurementEnhancementCA-r17 or when highSpeedMeasInterFreq-r17 flag is configured and UE supports measurementEnhancementInterFreq-r17.

Following TS 38.306, the UE capabilities used in HST FR2 Rel-17 may not be applicable directly because they are defined for FR1.

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

For the requirements that are inherited from Rel. 17, as suggested by option 2, the Rel. 17 signalling can be used.

Rel-17 signaling can be re-used in Rel-18 CA to indicate enhanced requirements that are inherited from Rel-17.

Conclusion

This paper discussed the open topics on the carrier aggregation topic on FR2 HST.

In the paper, the following Observations and Proposals were made:

Proposal 1: Inter-frequency time index detection with measurement gap configuration and inter-frequency time period for PSS/SSS detection require further analysis for Rel. 18 HST FR2 deployments with SSB-based SCell measurements.

Observation 1: Currently, there are three requirements for SCell activation. In case there is no down-scoping of the considered scenarios, all the previous requirements should be considered for the Rel-18 FR2 SCell activation delay. Possible down-scoping would be:

	Support only SSBless scenario where an active serving cell exists in the same band.

	Support only SSB based scenario where an active serving cell exists in the same band.

	Do not support SSBless scenario..

	No restrictions..

Proposal 2: Do not to consider CA for cells in a band different than the serving PCell as an applicable scenario for SCell activation delay.

Proposal 3: RAN4 to consider CA for SSB-based and SSB-less scenario in the same band as the serving PCell as an applicable scenario for SCell activation delay.

Observation 2: IDLE mode enhancements can be useful only for optional early measurement reporting (EMR) feature, if supported by the Rel-18 HST FR2 CPEs.

Proposal 4: EMR feature can be optionally supported by Rel-18 HST FR2 CPE capable of CA.

Proposal 5: Rel-17 signaling can be re-used in Rel-18 CA to indicate enhanced requirements that are inherited from Rel-17.

- References

- R4-2306242, Email discussion summary for [106bis-e][211] NR_HST_FR2_enh_part2 (Nokia), RAN4#106bis-e, Online Meeting, 17th April- 26th April, 2023

- R4-2220397, WF on NR FR2 HST RRM Core requirements impact, Samsung, RAN4#105, Toulouse, France, November 14th – November 18th, 2022

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management".