3GPP TSG-RAN WG4 Meeting #107 	R4-2308114

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Co-channel coexistence between NR sidelink and LTE sidelink

Agenda item:	8.31.2.4

Document for:	Discussion

Introduction

The updated WID [1] of NR sidelink evolution from 3GPP TSG RAN Meeting #99-e includes an objective related to the support of operation of sidelink in unlicensed spectrum and also has an objective related to support of new introduced features (e.g. SL unlicensed) and frequency bands. Relevant objectives are copied below.

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only - UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] - The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI. - The frequency band combinations is limited to the ITS-dedicated spectrum only (Band n47) for contiguous carrier aggregation. - Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements. - Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

The topic of sidelink co-channel coexistence was discussed in RAN4#106bis-e where the following issues and WF was agreed [R4-2306636]:

## 2-1-1: Study on requirements for LTE SL and NR SL co-channel coexistence scenario GTW Agreement - Focus on the co-channel LTE SL and NR SL mode where both could receive simultaneously but transmit in a TDM manner ## 2-1-2: Rx requirements of simultaneous reception for LTE SL and NR SL co-channel coexistence scenario Agreement - RAN4 needs to consider whether the power imbalance of co-channel is RF issue or demodulation performance issue in case of LTE SL and NR SL for simultaneous receptions ## 2-1-3: Tx requirements of LTE SL and NR SL co-channel coexistence scenario Agreement - Each TX UE will follow its own LTE SL and NR SL TX requirement respectively when UE transmits in TDM manner (either LTE SL TX or NR SL TX). - FFS whether Tx switching between LTE SL and NR SL defined in Rel-16 is applicable for the co-channel scenario. ## 2-1-4: Synchronization performance requirements of LTE SL and NR SL co-channel coexistence scenario Agreement - Discuss in RF session if any impact on RF requirement is identified ## 2-1-5: A set of evaluation parameters for LTE SL and NR SL co-channel coexistence scenario Agreement - Discuss in RF session if any impact on RF requirement is identified

In this paper we discuss open aspects on RF requirements for SL co-channel coexistence of LTE and NR SL.

Discussion

Co-channel co-existence between two RATs can be achieved in a semi-static manner by enforcing TDM, FDM resource sharing, or it can be done using a dynamic radio resource sharing mechanism. Translated into NR-LTE sidelink co-channel coexistence that corresponds to NR and LTE being configured with a non-overlapping RP (in time) for TDM, or a non-overlapping RP (in frequency) for FDM or a fully or partially overlapping RP where a set of rules and observations of the other RATs behavior decides when the device can access the RP (for dynamic). RAN1 has concluded that TDM co-channel coexistence can be achieved with no specification changes needed (hence no RAN4 impact expected there either).

In general, we agree with the WF listed above, and below address the identified open aspects. The WF on issue 2-1-2 states the following:

- RAN4 needs to consider whether the power imbalance of co-channel is RF issue or demodulation performance issue in case of LTE SL and NR SL for simultaneous receptions

However, as simultaneous reception of LTE SL and NR SL may be left for UE implementation and as we understand is not required in the in-device coexistence (IDC) framework from Release-16 we do not see a need to consider a potential power imbalance in for co-channel coexistence either.

No need to consider power imbalance between LTE SL and NR SL simultaneous reception in sidelink co-channel coexistence.

The WF on issue 2-1-3 has an FFS on whether the Tx switching time between LTE SL and NR SL from IDC in release-16 can be assumed to be applicable for co-channel coexistence as well. Our view here is that this is applicable since no changes or enhancements to the IDC framework is in the scope of the objective of sidelink co-channel coexistence.

Tx switching from in-device coexistence from Release 16 can be assumed for sidelink co-channel coexistence.

In issue 2-1-4 on synchronization impact to RF requirements, the difference from NR SL RF requirements to co-channel coexistence is that these Type-A devices might be constrained to use an LTE sidelink synchronization signal (SLSS) and in some configurations these LTE SLSS are not transmitted as frequent as NR S-SSB can be. If that ends up being the solution in RAN1 for Type-A devices, it might have RF requirement impact. It is, however, still too early to conclude this as the issue is still being discussed in RAN1.

RAN4 to wait for RAN1 to conclude on a synchronization framework for sidelink co-channel coexistence before discussing on the synchronization impact to RF requirements.

Conclusion

In the paper, the following proposals were made:

- No need to consider power imbalance between LTE SL and NR SL simultaneous reception in sidelink co-channel coexistence.

- Tx switching from in-device coexistence from Release 16 can be assumed for sidelink co-channel coexistence.

- RAN4 to wait for RAN1 to conclude on a synchronization framework for sidelink co-channel coexistence before discussing on the synchronization impact to RF requirements.

- References

- RP-230077, WID revision: NR sidelink evolution, OPPO.