3GPP TSG-RAN WG4 Meeting # 107	R4-2308118

# Incheon, KR, May. 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion on MSD UL aggressor power class ambiguity

Agenda item:	5.1.2

Document for:	Discussion

Introduction

Based on the WF[1] of NR_SL_enh2-core agreed in the last meeting, the agreed example band combination and MSD for Uu@Licensed and SL@Un-licenseded are as follows.

1-1-2: Example band combination for Uu@Licensed and SL@Un-licensed GTW Agreement: - Use n77@licensed band + n46@unlicensed band as the example band combination. - Revisit the above agreement if there is further input from operator in this meeting Agreement: - Use n78@licensed band + n46@unlicensed band as the example band combination instead of GTW agreement (n77@licensed band + n46@unlicensed band). 1-5-1: MSD for Uu@Licensed and SL@Un-licensed GTW Agreement - Define MSD requirement after the targeting band combination is clear

We observe that the MSD requirements for the band combination n78 and n46 are already defined in TS38.101-1 and we have some uncertainties in the corresponding MSD requirements, so we would like to share the MSD power class ambiguity and propose a solution.

Discussion

Table 1 TS38.101-1 [6.2F.1-1: UE Power Class]

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

Table 2 TS38.101-1 [Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1]

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n46 | n7 | 5 | 15 | 12 (RBstart=0) | 5 | 8.3 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 5 | 15 | 24 (RBstart=0) | 50 | 0.6 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 22.6 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 19.5 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 5.8 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 0.5 | NOTE 2 | UL2/DL3 |

Table 3 TS38.101-1 [Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 6.2 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 6.2 | >ACLR2 |

In the current specification, even though NR-U is supported by only power class 5, the MSD values for inter-band CA which aggressor band is shared spectrum band, are captured in Table 7.3A.4-4 and Table 7.3A.6-1 which power class 3 is aggerssor band. Therefore, We need to clarify which power class was used to analyse the Table 7.3A.4-4 and Table 7.3A.6-1 for the shared spectrum aggressor bands n46 and n96.

Proposal 1: Clarify which power class was used to analyse the Table 7.3A.4-4 and Table 7.3A.6-1 for the shared spectrum aggressor bands n46 and n96.

The MSD value for PC5 aggressor may be lower than PC3 aggressor. Therefore, it is need to separate the MSD table for PC5 aggressor table and PC3 aggressor table. Depending on the outcome of Proposal 1, there are two cases.

Proposal 2: Depending on the outcome of Proposal 1, follow the proposals below.

- Case 1: If n46 and n96 were considered as PC3.

■Proposal 2a: Keep the existing UL PC3 aggressor MSD table, and define a new UL PC5 aggressor MSD table and MSD values for inter-band CA related with n46 and n96.

- Case 2: If n46 and n96 were considered as PC5.

■Proposal 2b: Define a new PC5 UL aggressor MSD table and move the n46 and n96 MSD from the current PC3 UL aggressor MSD table to the new PC5 UL aggressor MSD table.

Conclusion

Proposal 1: Clarify which power class was used to analyse the Table 7.3A.4-4 and Table 7.3A.6-1 for the shared spectrum aggressor bands n46 and n96.

Proposal 2: Depending on the outcome of Proposal 1, follow the proposals below.

- Case 1: If n46 and n96 were considered as PC3.

■Proposal 2a: Keep the existing UL PC3 aggressor MSD table, and define a new UL PC5 aggressor MSD table and MSD values for inter-band CA related with n46 and n96.

- Case 2: If n46 and n96 were considered as PC5.

■Proposal 2b: Define a new PC5 UL aggressor MSD table and move the n46 and n96 MSD from the current PC3 UL aggressor MSD table to the new PC5 UL aggressor MSD table.

- References

[1]R4-2306636, WF on NR SL con-current operation and co-channel coexistence scenario (LGE)

[2]TS38.101-1