3GPP TSG-RAN WG4 Meeting # 107	R4-2308126

# Incheon, Korea, 22th - 26th, May, 2023

Source: 	Samsung, KT corporation

Title: 	Views on Tx requirements for band combinations with 3Tx

Agenda Item:	7.29.3.1

Document for:	Discussion

- Introduction

A new WID was approved in RAN#98 while further revised in RAN#99 as [1], in which enhancement of 3Tx for inter-band CA/EN-DC with two bands is included as one objective. In last meeting, the group had some initial discussion on the applicable Tx/Rx requirement and spec impact, the agreements are captured in [2].

In this paper, we share our views on the remaining issues for Tx requirements.

- 2. Discussion

2.1 SAR/MPE compliance

It is identified that either via 2Tx or 3Tx to achieve PC1.5, the SAR/MPE issue is supposed to be discussed and addressed. Tough duty cycle based solution is not adopted in the field for SAR/MPE compliance by UE vendors and some chipset vendors, it is still suggested to include it in spec for sake of capability completeness, but we feel there is no need to spend too much efforts on introducing a dedicated new IE with new UL duty cycle values.

Observation 1: Tough duty cycle based solution is not adopted in the field for SAR/MPE compliance by UE vendors and some chipset vendors, it is still suggested to include it in spec for sake of capability completeness.

More specifically, reusing 0.5* maxUplinkDutyCycle-interBandCA-PC2 or introducing dedicate new IE for PC1.5 inter-band CA/EN-DC is expected to be discussed, consequently clause 6.2A.1.3 of 38.101-1 needs update.

In terms of single band PC1.5 which was introduced since Rel-16, 0.5* maxUplinkDutyCycle-PC2-FR1 (If indicated) is the threshold for PC1.5 uplink duty cycle, below which the PC1.5 requirements shall apply, and above which either PC2 or PC3 requirement applies. When PC1.5 was added for n77 and n78, a new maximum duty cycle capability IE maxUplinkDutyCycle-PC1dot5-MPE-FR1 was introduced dedicatedly for FWA devices to meet Maximum Permissible Exposure (MPE) requirements though in RAN4 there is no way to limit this IE to FWA only. The details are well summarized in [3]

Similarly, for PC1.5 inter-band CA/EN-DC, 0.5* maxUplinkDutyCycle-interBandCA-PC2 (If not absent) could be used as the threshold for PC1.5 uplink duty cycle, below which the PC1.5 requirements shall apply. If the field (maxUplinkDutyCycle-interBandCA-PC2) is absent, there would be no assumed restriction on scheduled uplink duty cycle, that is to say UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary.

Proposal 1: For PC1.5 inter-band CA/EN-DC SAR/MPE compliance, it is suggested to use 0.5* maxUplinkDutyCycle-interBandCA-PC2 (if indicated) as the threshold for PC1.5 UL duty cycle, below which the requirements for PC1.5 applies; If the percentage of uplink symbols is larger than 0.5* maxUplinkDutyCycle-interBandCA-PC2 but less than maxUplinkDutyCycle-interBandCA-PC2, the requirements for PC2 applies; otherwise the requirements of default power class applies.

- If maxUplinkDutyCycle-interBandCA-PC2 is absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary.

2.2 Δ PPowerClass,CA

For PCMAX change, the WF in last meeting is to further study the applicable Δ PPowerClass,CA for PC1.5 capable UE. In our understanding, for inter-band UL NR-CA, ΔPPowerClass,CA would be impacted by IE maxUplinkDutyCycle-interBandCA-PC2, by PEMAX,CA (which is indicated by IE p-NR-FR1 or p-UE-FR1) and 10 log10∑ pEMAX,c (where pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c). However, it is not correctly reflected in current spec TS 38.101-1 (since Rel-17), some errors are identified in the referred sub-clause 6.2A.1.3 which makes ΔPPowerClass,CA incorrect and correction TEI CR is provided as R4-2308135 in this meeting.

(Samsung’s understanding on the 6 p-max relevant IE is provided in Annex for companies’ kind reference.)

(Below is reproduced from sub-clause 6.2A.4.1.3 of 38.101-1)

(Below is reproduced from sub-clause 6.2A.1.3 of 38.101-1)

The modification of R4-RXXXX is shown below:

Proposal 2: Errors are identified for PC2 ΔPPowerClass,CA for inter-band ULCA in current NR spec(since Rel-17), correction CR is provided as R4-2308135 in this meeting. The description method of PC1.5 ΔPPowerClass,CA could adopt similar way and could be checked based on the CR provided in future meetings.

In terms of ΔPPowerClass,CA value for PC1.5, in our view it could be: ΔPPowerClass,CA= 6dB applies for a power class 1.5 capable UE when the requirements of default power class are applied; ΔPPowerClass,CA =3dB applies for a power class 1.5 capable UE when the requirements of power class 2 are applied; Otherwise 0dB applies. The requirements of which power class applies is described in sub-clause 6.2A.1.3 of 38.101-1, which could be checked based on the CR provided in future meetings.

Proposal 3: ΔPPowerClass,CA=

- 6dB applies for a power class 1.5 capable UE when the requirements of default power class are applied;

- 3dB applies for a power class 1.5 capable UE when the requirements of power class 2 are applied;

- Otherwise 0dB applies.

The condition of which power class applies should be described in sub-clause 6.2A.1.3 of 38.101-1, which could be checked based on the CR provided in future meetings.

In addition, currently there is no example EN-DC band combination included in current WID for PC1.5 so in our view there might be unnecessary to update ΔPPowerClass,EN-DC  in spec for the time being, but ΔPPowerClass,EN-DC could adopt similar methodology as for ΔPPowerClass,CA.

Proposal 4: It is not needed to define ΔPPowerClass,EN-DC for PC1.5 capable UE in 38.101-3 for the time being considering there is no example PC1.5 EN-DC band combination included in current WID, but PC1.5 ΔPPowerClass,EN-DC could adopt similar methodology as for ΔPPowerClass,CA.

- 3. Conclusion

Proposal 1: For PC1.5 inter-band CA/EN-DC SAR/MPE compliance, it is suggested to use 0.5* maxUplinkDutyCycle-interBandCA-PC2 (if indicated) as the threshold for PC1.5 UL duty cycle, below which the requirements for PC1.5 applies; If the percentage of uplink symbols is larger than 0.5* maxUplinkDutyCycle-interBandCA-PC2 but less than maxUplinkDutyCycle-interBandCA-PC2, the requirements for PC2 applies; otherwise the requirements of default power class applies.

- If maxUplinkDutyCycle-interBandCA-PC2 is absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary.

Proposal 2: Errors are identified for PC2 ΔPPowerClass,CA for inter-band ULCA in current NR spec(since Rel-17), correction CR is provided as R4-2308135 in this meeting. The description method of PC1.5 ΔPPowerClass,CA could adopt similar way and could be checked based on the CR provided in future meetings.

Proposal 3: ΔPPowerClass,CA=

- 6dB applies for a power class 1.5 capable UE when the requirements of default power class are applied;

- 3dB applies for a power class 1.5 capable UE when the requirements of power class 2 are applied;

- Otherwise 0dB applies.

The condition of which power class applies should be described in sub-clause 6.2A.1.3 of 38.101-1, which could be checked based on the CR provided in future meetings.

Proposal 4: It is not needed to define ΔPPowerClass,EN-DC for PC1.5 capable UE in 38.101-3 for the time being considering there is no example PC1.5 EN-DC band combination included in current WID, but PC1.5 ΔPPowerClass,EN-DC could adopt similar methodology as for ΔPPowerClass,CA.

- 4. Reference

[1] RP-230161, 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#99

[2] R4-2306554, WF on Tx requirements for 3Tx UE, OPPO, RAN4#106-bis-e

[3] R4-220807, LS on Duty Cycle capability for PC1.5, RAN4#105, T-Mobile USA

- Annex: 6 p-max relevant IE

| IE name | Description in spec | Samsung’s understanding |
| --- | --- | --- |
| p-Max (in frequencyInfoUL) | Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. Value in dBm. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. | It’s serving cell-specific. It’s an upper bound in a serving cell, but the other power parameters (i.e. p-NR-FR1 and p-UE-FR1) have to be considered in addition p-Max, p-NR-FR1 and p-UE-FR1 are provided to UE, by RAN, and UE decides its max power for each serving cell, based on these parameters. |
| p-NR-FR1 (in PhysicalCellGroupConfig) | The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1). | It’s cell group-specific. It can be used for EN-DC as well as all CA cases (please see TS38.101-3 clause 6.2B.4.1). This parameter is optional, and is not needed for the single CC only. |
| p-UE-FR1 (in PhysicalCellGroupConfig) | The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group). | It’s UE-specific. This parameter is optional, and is not needed for the single CC only. |
| p-maxNR-FR1 (in CG-ConfigInfo) | For (NG)EN-DC and NE-DC, the field indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]). For NR-DC, it indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG. | p-maxNR-FR1, p-maxEUTRA and p-maxUE-FR1 are provided to SN, by MN, and SN decides UE max power for each serving cell belonging to SCG, based on these parameters, and SN may configure the decided max power to UE. |
| p-maxEUTRA (in SIB5 & CG-ConfigInfo) | The corresponding field description in SIB5 is as follows: The maximum allowed transmission power in dBm on the (uplink) carrier frequency, see TS 36.304 [27]. [Samsung] it is assumed for the single CC, because UE is in RRC_IDLE or RRC_INACTIVE. The corresponding field description in CG-ConfigInfo is as follows: Indicates the maximum total transmit power to be used by the UE in the E-UTRA cell group (see TS 36.104 [33]). This field is used in (NG)EN-DC and NE-DC. |  |
| p-maxUE-FR1 (in CG-ConfigInfo) | The corresponding field description in TS38.331 is as follows: Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1). And, same parameter can be found also in TS36.331, and it is provided to UE, by eNB (with RRCConnectionReconfiguration message) The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited on cell- or cell-group level. The field is optionally present, if (NG)EN-DC (nr-Config-r15) has been configured. It is absent otherwise. | The intention of p-UE-FR1 and p-maxUE-FR1 is quite similar. In our understanding, p-UE-FR1 is applicable for NR CA and NR-DC, while p-maxUE-FR1 is applicable for EN-DC. |