# 3GPP TSG-RAN WG4 Meeting # 107                                                            R4-2308224

Incheon, Korea, 22th May 2023 - 26th May 2023

Source:	China Unicom

Title:	Discussions on RedCap HPUE

Agenda item:			11.1

Document for:	Approval

1. Introduction

In RAN4-106bis-e meeting, a WF [1] on RedCap HPUE was agreed as below:

Issue 1: Form factor for PC2 RedCap HPUE

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

Issue 2: How to specify HPUE including PC2 for RedCap

Way forward:

- PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD.

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

- Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD.

- Note 1: 1Tx architecture is assumed for RedCap PC2.

- Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

In this contribution,  we share our considerations on PC2 for RedCap UE.

2. Discussion

As PC2 (high power transmission) is an optional feature, and the capability would only be reported to the network if it is supported. So there is no need to restrict on the form factor for enabling of PC2 feature.

Proposal 1:  Not restrict the applicable form factor for PC2 RedCap HPUE in the specification.

Redcap UEs share the same RF requirements with handheld UEs of the same Tx/Rx numbers for PC3 for channel bandwidth of up to 20MHz. In our view, the same applies for PC2 cases, which implies that the requirements of FDD PC2 HPUE for handheld UE are applicable for PC2 RedCap HPUE (2Rx) in band agnostic way. For 1Rx RedCap PC2 UE, some Rx requirements degradation would be expected due to smaller number of Rx chains.

Proposal 2: The requirements of FDD PC2 HPUE for handheld UE are applicable for PC2 RedCap HPUE (2Rx) in band agnostic way.

Observation 1: For 1Rx RedCap PC2 UE, some Rx requirements degradation would be expected due to smaller number of Rx chains.

3. Conclusion

In this contribution, we shared our views on RedCap HPUE, and made the following proposals and observations:

Proposal 1:  Not restrict the applicable form factor for PC2 RedCap HPUE in the specification.

Proposal 2: The requirements of FDD PC2 HPUE for handheld UE are applicable for PC2 RedCap HPUE (2Rx) in band agnostic way.

Observation 1: For 1Rx RedCap PC2 UE, some Rx requirements degradation would be expected due to smaller number of Rx chains.

3. Reference

[1] R4-2306650	WF on RedCap HPUE, China Telecom