3GPP TSG-RAN WG4 Meeting # 107												R4-2308230

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	TP for TR 38.891 on test related issue of beam correspondence in initial access

Agenda Item:	8.7.3.3

Document for:	Approval

- Introduction

The test related issue has been discussed for a long time and in the last meeting, most companies agreed that the details of the test setup can be left to RAN5. In this contribution, we try to summarize the discussion about test related issues so far.

- References

[1] 	3GPP TS 38.213

[2]	3GPP TS 38.321

[3]	R5-231834	Response LS on testability for beam correspondence in initial access, Apple, RAN5#98

[4]	R4-2216131	Discussion on test related issue of beam correspondence, vivo, RAN4#104e-bis

[5]	R4-2215856	On beam correspondence test issues in RRC_IDLE or RRC_INACTIVE for Rel-18 NR FR2, Huawei, HiSilicon, RAN4#104e-bis

[6]	R4-2215702	FR2 beam correspondence requirement and test for random access, Samsung, RAN4#104e-bis

- Text proposal

## <<Start of Change for TR 38.891>>

## 6.4	Testing impact

6.4.1 Beam correspondence in initial access

Unlike traditional RF requirements defined in the RRC_CONNECTED state, beam correspondence for initial access needs to be verified in RRC_IDLE, and several potential testability issues were raised during the discussion. The following aspects of test related issues were identified:

- Verification in RRC_IDLE

In general, the power test needs to be averaged over a period of time, which requires the UE to remain in RRC_IDLE and continuously send PRACH. The easiest way is to hold RAR to avoid the UE entering the subsequent initial access procedure.

- Maximum output power

To guarantee stable test results, the requirement is defined and verified under maximum output power (MOP). Since the open loop power control is used for PRACH, how to ensure that the UE reaches and maintains MOP is a problem that has to be faced. The power control for PRACH is defined in [1] as:

[dBm],

The parameter  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers as specified in clause 5.1.3 of [2]:

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

UE can ramp up its power when RAR is held since it will trigger PRACH retransmission. However, UE also can achieve MOP by setting the preambleReceivedTargetPower properly which is the initial preamble power configured by RRC signalling. Based on the analysis above, The PRACH can achieve MOP with well-defined parameters, and the details of the parameter configured in the test can be left to RAN5.

- Beam lock function

Currently, the beam lock function is also only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., uncertainty due to the beam change, difficulty to measure the θ (or φ) component EIRP under φ (or θ) link polarization, etc. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved and the details of beam lock function design can be left to RAN5.

6.4.2 Beam correspondence for SDT

Considering the same requirements are applied to initial access and SDT, only the beam correspondence in initial access needs to be verified to alleviate the test overhead.

## <<End of Change>>