3GPP TSG-RAN WG4 Meeting # 107	R4-2308242

# Incheon, KR, May 22nd – May 26th, 2023

Source: 	vivo

Title: 	Tx Requirements of 3Tx for band combinations with two bands

Agenda Item:	7.29.3.1

Document for:	Discussion

Introduction

The UE 3Tx for inter-band UL CA and EN-DC feature had been introduced in RAN#98-e in [1]. In RAN4#106bis-e, a WF [2] has been agreed on Tx requirements for 3Tx UE. The detailed topic summary is in [3].

In this paper, more analysis is provided for Tx requirements for 3Tx.

Discussion

How to specify inter-band CA + TxD

In last meeting, it has been agreed that inter-band UL CA + UL-MIMO would be defined in specific sections in the spec and the band combinations can be defined in MOP tables of clause with suffix H. However, for the UL CA + TxD, still no conclusion is made.

During the discussion, it has been pointed out that the just expanding the requirements to support TxD in current CA requirements is not appropriate since only limited band combinations would be allowed for this inter-band UL CA + TxD feature. In order to clearly indicate this, simply expanding current CA requirements to cover 3Tx case may not be so attractive though some indication can still be done in CA MOP combination tables.

Observation 1: The band combinations for inter-band UL CA + TxD also need to be clearly specified in the spec.

In order to achieve the effect, it is proposed to also use similar method as inter-band UL CA + UL-MIMO, that to have dedicated sections and define specific MOP tables for inter-band UL CA + TxD case. A draft TP is also attached here for reference.

Proposal 1: Add dedicated sections and define specific MOP tables for inter-band UL CA + TxD case. A draft TP is also provided below for 38.101-1.

- 6.2J.2	Transmitter power for inter-band UL CA with Tx Diversity

6.2J.2.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2J.2.1-1. The period of measurement shall be at least one sub frame (1 ms). The requirements shall be met with the Tx Diversity requirements specified in clause 6.2G.1 for the component carrier configured with Tx Diversity.

Table 6.2J.2.1-1: UE Power Class for inter-band UL CA with Tx Diversity in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n78A |  |  | 263 | +2/-3 | 23 | +2/-3 |
| CA_n26A-n78A |  |  | 263 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 263 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 263 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n71A | 294 | +2/-3 | 263 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Power class 3 is the default power class unless otherwise stated. NOTE 3: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 4: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

6.2J.2.2	UE maximum output power reduction for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2G.2 apply for the component carrier configured with Tx Diversity and the requirements in clause 6.2.2 apply for the other component carrier.

6.2J.2.3	UE additional maximum output power reduction for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, unless specified in Table 6.2A.3.1.3-1, the requirements in clause 6.2.3 apply only to the indicated carrier, and the maximum output power is defined as the sum of the maximum output power at each UE antenna connector for the component carrier configured with Tx Diversity.

6.2J.2.4	Configured transmitted power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 respectively for the component carrier configured with Tx Diversity.

PC1.5 inter-band CA/EN-DC SAR compliance

In last meeting, both the delta Ppowerclass,CA and the SAR scheme for CA was discussed, but no conclusion is made. The delta Ppowerclass,CA is related with SAR scheme, and there are discussions on how to apply the current UE capabilities (PC2-related maxUplinkDutyCycle) and whether to use 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL duty cycle.

In order to have a complete picture of the current duty cycle related SAR scheme for PC2 inter-band UL CA/EN-DC, the following spec is referenced, start with inter-band UL CA for PC2 in 38.101-1.

The delta power class definition is as following:

As the proponent of this scheme for PC2, it needs to be pointed out that the threshold of maxUplinkDutyCycle-interBandCA-PC2 is only part of the scheme, and the corresponding average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). In this equation, one of the underlying principles is that it is PC2 or PC3 in any band, and there is only the PC2/PC3 in the scheme.

In the current WID, the following scope is provided:

      - CA power class or EN-DC power class is PC1.5

        - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

Observation 2: Current SAR scheme for inter-band UL CA HPUE is for PC2, and simply adding a scaling factor for the overall parameter maxUplinkDutyCycle-interBandCA-PC2 seems not enough since the scheme also have other underlying principles such as per-band power class in the average equation.

Based on this observation, the following proposal is provided:

Proposal 2: Overall scheme would have to be re-evaluated for PC1.5 case for inter-band UL CA, and not restricted to use 0.5*maxUplinkDutyCycle-interBandCA-PC2.

For EN-DC, different SAR schemes are exist for FR1 for E-UTRA TDD + NR TDD Inter-band EN-DC and E-UTRA FDD + NR TDD Inter-band EN-DC. However, since the time for recent meetings are quite limited, those analysis seems would start later. Here we copied the E-UTRA FDD + NR TDD Inter-band EN-DC SAR scheme below for reference since this is more close to the combination in current WID scope.

Observation 3: The EN-DC SAR scheme is different from CA SAR scheme for inter-band UL and needs separate analysis.

Proposal 3: Further study SAR scheme for PC1.5 EN-DC case.

Conclusion

In this paper, some more analysis is provided for Tx requirements for 3Tx.

Observation 1: The band combinations for inter-band UL CA + TxD also need to be clearly specified in the spec.

Proposal 1: Add dedicated sections and define specific MOP tables for inter-band UL CA + TxD case. A draft TP is also provided below for 38.101-1.

Observation 2: Current SAR scheme for inter-band UL CA HPUE is for PC2, and simply adding a scaling factor for the overall parameter maxUplinkDutyCycle-interBandCA-PC2 seems not enough since the scheme also have other underlying principles such as per-band power class in the average equation.

Proposal 2: Overall scheme would have to be re-evaluated for PC1.5 case for inter-band UL CA, and not restricted to use 0.5*maxUplinkDutyCycle-interBandCA-PC2.

Observation 3: The EN-DC SAR scheme is different from CA SAR scheme for inter-band UL and needs separate analysis.

Proposal 3: Further study SAR scheme for PC1.5 EN-DC case.

- References

[1] RP-223517, Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC, RAN 98-e

[2] R4-2306554, WF on Tx requirements for 3Tx UE, OPPO, RAN4#106bis-e

[3] R4-2306275, Topic summary for [106-bis-e][112] NR_3Tx-4Rx_WI , Moderator (OPPO), RAN4#106bis-e