3GPP TSG-RAN WG4 Meeting # 107	R4-2308248

# Incheon, KR, May 22nd – May 26th, 2023

Source: 	vivo

Title: 	Discussion on power scaling factor s and impact to RAN4

Agenda Item:	4.1

Document for:	Discussion

Introduction

In RAN4#106bis-e, there is a paper [1] in FR1 enh WI 4Tx agenda that discussed the relation between configured transmitted power and TPMI configuration. This topic was discussed but no conclusion is made, as documented in the WF [2] and topic summary [3].

Since this problem is closely related to RAN1 and actually start from Rel-15 and stabilized in Rel-16, it is proposed to be separated from Rel-18 UE RF agenda and discuss this in the maintenance agenda.

In this paper, a complete analysis was provided with observations and proposals. A draft LS is also proposed in the Annex and also in a separate doc [6].

Discussion

Background

In [1], the following observations and proposals are provided:

Observation 1: Current TS38.101-1 doesn’t reflect what TS38.213 expects in terms of configured transmitted power range.

Observation 2: Current TS38.101-1 allows higher power more than TS38.213 expects.

Observation 3: Without considering the impact of port configuration on configured transmitted power, UE behaviours in terms of power (and also applicable MPR/A-MPR conditions) is not clear.

Observation 4: Uncertainty of expected achievable UE power (and MPR/A-MPR) may lead to inefficient allocation of resources.

Proposal: Address the identified issue to enable UE behaviours in terms of power to be interpreted uniquely.

In addition, a brief summary of the background is also provided in the WF [2]:

This is not an issue specific for 4Tx, but for all the cases when power scaling is needed for legacy case and also Full Power transmission Mode when the full power cannot be applied for some TPMI configurations. Here are some of the status:

- According to RAN1, there are power scaling Rel-15 and in certain TPMI configurations for ULFPTx modes to reduce the maximum output power;

- The UL-MIMO maximum power requirements do not verify the power scaling behavior;

- The configurated transmitted power definition also do not consider the power scaling;

- The PHR calculation do not consider power scaling.

Spec Status

Power Scaling (TS38.213)

This is where the power scaling s and the PHR equation definition. Here are some of the status.

Power Scaling definition in Rel-15:

It can be seen in release 15, this scaling is already taking effect, though this definition was not named as “s” yet.

The above power scaling specified in RAN1 from Rel-15 is due to the understanding of implementation restriction at that time, and is actually undesirable.

Power Scaling definition from Rel-16:

This part has been stable since Release-16. From this release, full power transmission was added, and the power scaling factor was defined as s. For full power transmission, there are many cases that corresponds to different implementations, that would ensure s =1, thus no more scaling is needed. However, there are still many configurations, e.g. some TPMIs in Mode 1/2, that still have a scaling factor s smaller than 1, which is similar to Rel-15. Still, more UE implementations had ensured that full power can be achieved, and network would also have much more information.

In addition, it should be noted that s is applied for PPUSCH,b,f,ci,j,qd,l, not for Pcmax.

Observation 1: The power scaling in RAN1 is due to RAN1’s understanding of UE implementation restriction and actually not that desirable.

Observation 2: The power scaling fact s is applied to PPUSCH,b,f,ci,j,qd,l.

Definition of PPUSCH,b,f,c(i,j,qd,l) (TS 38.213)

The definition of PPUSCH,b,f,c(i,j,qd,l) is just close to the power scaling factor as following:

It should be noted this could be either Pcmax or the calculation target of for open loop power control.

Based on the previous observation, the current understanding is that the neither Pcmax nor the power control target {} was directly changed by the power scaling, this is very important feature that would be used in PHR definition.

Observation 3: The power scaling s does not directly change Pcmax or {}.

Definition of Power Headroom (TS 38.213)

PHR reporting in NR basically reuse the scheme of LTE, including the type 1/2/3 that were used. Here we use type 1 as a typical example as following:

It can be seen that this power headroom definition is just the difference between the two items that were selected of min value in the definition of PPUSCH,b,f,c(i,j,qd,l) , that is, the supposed the maximum configured power and the power control target which we would use a more simple expression {Target} in the following context of this contribution. The equation would be simplified to PH = Pcmax – {Target} in the remainder of this contribution.

Currently, the power scaling factor is not considered in the PH definition. Based on the previous observation, the power scaling factor s does not directly change Pcmax or {Target}, so it can be assumed that the two items in the equation of PH would not be impacted by the power scaling factor s.

Observation 4: Currently the power scaling factor does not impact power headroom definition and calculation.

PCMAX,f,c reporting (RAN2 38.321)

It has been defined in 38.321 that with the type 1/3 power headroom reporting, the Pcmax,f,c would also be reported:

obtain the value for the corresponding PCMAX,f,c field from the physical layer

During the process, also no more power scaling is mentioned.

PCMAX,f,c Definition (RAN4 38.101-1/2)

Taking FR1 as an example for simplicity as following :

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

It can be seen that RAN4 also do not consider the power scaling factor s as defined in RAN1 for Pcmax,f,c. What is the actual impact on this definition would be discussed in later sections.

PHR and Pcmax Analysis

One of the main concerns of this power scaling is whether the current PHR reporting is precise or not in case the power scaling factor s does not equal to 1. In order to have more understanding on this, some detailed analysis is provided below. The baseline assumption is Pcmax and {Target} would not be impacted by the power scaling factor in PH = Pcmax – {Target} as a simplified equation, and some simplified values would be also used. The power scaling factor s = 1/2 is used for these examples.

PH is positive

In case PH is positive, this means that there are still room for the actual power, and the power is not reaching upper limit. E.g. In case Pcmax = 23dBm, {Target} = 17dBm, then:

(1) The calculated PH = Pcmax - {Target} = 6dB without considering the power scaling.

(2) Next we analyse on what is the “real” PH after considering the power scaling:

Ppusch is used as simplified illustration for PPUSCH,b,f,c(i,j,qd,l) , then Ppusch = min {Pcmax, {Target}} = 17dBm. Then the actual configured power in physical layer after power scaling is s*linear (Ppusch) = Ppusch – 3dB = 14dBm;

In this particular case, the achievable maximum configurable power would also be impacted is s* linear (Pcmax) = Pcmax – 3dB = 20dBm.

Then the “real” PH that considered the power scaling is 20dBm – 14dBm = 6dB; it can be seen that this is the same to the calculated the PH which do not consider power scaling s.

Observation 5: In case PH is positive, the PH reported would be the same to actual achievable PH even in case of power scaling is not equal to 1.

PH is negative

In case PH is negative, this means that maximum output power is needed and the power is already reaching upper limit. The MCS and the power network configured may need to be lower in this case. E.g. In case Pcmax = 23dBm, {Target} = 26dBm, then:

(1) The calculated PH is PH = Pcmax - {Target} = -3dB without considering the power scaling;

(2) Ppusch = min {Pcmax, {Target}} = 23 dBm. Then the actual configured power in physical layer after power scaling is s*linear (Ppusch) = Ppusch – 3dB = 20dBm;

In this particular case, the achievable maximum configurable power would also be impacted is s* linear (Pcmax) = Pcmax – 3dB = 20dBm.

Then the “real” PH that considered the power scaling is 20dBm – 26dBm = -6dB; it can be seen this is indeed different from calculated PH and there may be some problem.

However, it should be noted that though -6dB is different from -3dB, this degree of difference may not have serious impact to network. Since the network would anyway adjust the parameters such as MCS etc. The feedback and stabilize effect would always work apart from 3dB difference. And, from the power scaling defined in section 7.1 in 38.213, network has knowledge of this behaviour when certain TPMI indices are used.

Observation 6: In case PH is negative, the PH reported may have an offset to actual achievable PH in case of power scaling not equal to 1. However, this may have quite limited impact to the whole system.

Based on the analysis, it seems the PH calculation with no consideration of power scaling factors s, may bring some unclear understandings of PH calculation, though the impact may not be large.

It is obvious that the previous analysis is mostly based on RAN1’s concept, and even some of the possible unclear part is also mainly RAN1 issue, thus it seems unlikely to solve this in RAN4 alone.

Observation 7: The PH calculation with no consideration of power scaling factors s, may bring some unclear understandings of PH calculation, though the impact may not be large.

Pcmax Analysis

During the analysis process , it can be seen in case the factor s is not equal to 1 and applied to PPUSCH,b,f,ci,j,qd,l, the PPUSCH,b,f,ci,j,qd,l would indeed not achieve the Pcmax,f,c in any case. E.g. when S =1/2 2Tx UL MIMO when indicated TPMI is 0 or 1, the actual achievable maximum configured power is indeed would be 3dB lower than the Pcmax,f,c value currently defined in RAN4 and used in RAN1 spec.

However, simple revision of Pcmax,f,c may also be not attractive, since even RAN1 spec do not have direct scaling on this concept.

Since this is start from Rel-15, and the full power transmission from Rel-16 also exist. And, UL full power transmission modes are specifically introduced in Rel-16 to address the issue unachievable maximum configured power.

Observation 8: The understanding of Pcmax may indeed be impacted by the power scaling factor s.

Proposals

Since this problem is from Rel-15 and mainly RAN1 issue, it is proposed to send a LS to RAN1 to consult their understandings and next step way forward. It is difficult to solve this in RAN4 alone.

Up till now, no serious negative effect has been discovered yet even this problem exists from Rel-15. Based on the analysis, the need to further revise those concepts is still not clear.

Based on the previous observations and status, the following two proposals are provided:

Proposal 1: Send a LS to RAN1 to ask RAN1’s view on this issue.

Proposal 2: Not to change RAN4 spec until more feedback from RAN1. Any future revision of Pcmax related concept relating to this feature may also need communication with RAN1 to avoid undesirable impacts to RAN1.

A draft LS is attached in the Annex and also in separated Tdoc [6].

Conclusion

In this paper, a comprehensive analysis are provided for power scaling factor s and impact to RAN4.

Observation 1: The power scaling in RAN1 is due to RAN1’s understanding of UE implementation restriction and actually not that desirable.

Observation 2: The power scaling fact s is applied to PPUSCH,b,f,ci,j,qd,l.

Observation 3: The power scaling s does not directly change Pcmax or {}.

Observation 4: Currently the power scaling factor does not impact power headroom definition and calculation.

Observation 5: In case PH is positive, the PH reported would be the same to actual achievable PH even in case of power scaling is not equal to 1.

Observation 6: In case PH is negative, the PH reported may haven an offset to actual achievable PH in case of power scaling not equal to 1. However, this may have quite limited impact to the whole system.

Observation 7: The PH calculation with no consideration of power scaling factors s, may bring some unclear understandings of PH calculation, though the impact may not be large.

Observation 8: The understanding of Pcmax may indeed be impacted by the power scaling factor s.

Based on the observations, the following two proposals are provided:

Proposal 1: Send a LS to RAN1 to ask RAN1’s view on this issue.

Proposal 2: Not to change RAN4 spec until more feedback from RAN1. Any future revision of Pcmax related concept relating to this feature may also need communication with RAN1 to avoid undesirable impacts to RAN1.

A draft LS is attached in the Annex and also in separated Tdoc [6].

- References

[1] R4-2304161, Relation between configured transmitted power and TPMI configuration, Nokia, RAN4#106bis-e

[2] R4-2306595, WF on 4Tx UE RF requirements, vivo, RAN4#106bis-e

[3] R4-2306286, Topic summary for [106][124] FR1_enh2_part2, Moderator (vivo) , RAN4#106bis-e

[4] TS 38.213

[5] TS 38.321

[6] R4-2308249, [Draft] LS on power scaling and PHR in 38.213, vivo, RAN4#107

- Annex Draft LS to RAN1

3GPP TSG-RAN WG4 Meeting # 107	R4-2308249

# Incheon, KR, May 22nd – May 26th, 2023

Title:	[Draft] LS on power scaling and PHR in 38.213

Response to:

Release:	Rel-15, Rel-16

Work Item:	NR_newRAT-Core, NR_eMIMO-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 discussed the power scaling behaviour defined in the beginning of clause 7.1 of TS 38.213. In case the factor s is not equal to 1 and applied to PPUSCH,b,f,ci,j,qd,l, RAN4’s understanding is that the PPUSCH,b,f,ci,j,qd,l would not achieve the Pcmax,f,c in any case, thus causing the actually achievable maximum configured power 3dB lower than the Pcmax,f,c value for 2Tx UL MIMO when indicated TPMI is 0 or 1. However, in the PHR calculation, i.e. in clause 7.7.1 in 38.213, this power scaling factor s is not considered.

RAN4 discussed this issue, and is not sure whether this power scaling would have an impact to the PHR calculation, and whether the power scaling should be considered in RAN4 specification for Pcmax,f,c.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to consider the issue above and provide feedback on RAN1’s understanding.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108                  21st August – 25th August 2023    Toulouse, FR

TSG-RAN WG4 Meeting #108-bis               9th October– 13th October 2023    Xiamen, CN