3GPP TSG-RAN WG4 Meeting #107                                                       R4-2308253   
Incheon, KR, May 22 – May 26, 2023

Agenda item:	 8.16.2.2

Source:	 vivo

Title:	Discussions on RC test method and Harmonization activity

Document for:	 Approval

- Introduction

In this contribution, we share our views on RC test method and harmonization activity.

- Discussions

In the WF [1], the minimum coherence bandwidth of RC has been agreed as 5 time of SCS.

Issue 3-1-1: spatial uniformity and verification for RC

Agreements:

  - Endorse the procedure in Annex 1 of R4-2305906, also listed in the annex part of this WF. Formal TP is needed next meeting. Further supplementary details might be needed.

Issue 3-1-2: Minimum Coherence bandwidth of RC

Agreements:

  - Coherence bandwidth (CBW) of a Reverberation Chamber (RC) shall be large enough with respect to subcarrier spacing (SCS), the minimum CBW of RC should be at least [5] times of Sub-Carrier Spacing. Further study an assessment of the influence of the frequency response into the wider channel bandwidths in RAN4.

Issue 3-1-3: Test procedure to verify Coherence bandwidth of RC

Proposal:

  - Endorse the procedure in Annex 2 of R4-2305906, also listed in the annex part of this WF. Formal TP is needed next meeting. Further supplementary details might be needed.

In this paper, we provide additional 50MHz CBW AC measurement results with the same test parameters used for RC testing in Table 1, in [2], to further check impacts of different NR bandwidth.

The TRS measurement is performed at band n78 with 50MHz CBW and 30kHz SCS. For each channel, detailed test parameters are aligned with the test parameters used in [2]. The test parameters are also copied in Table 1 for easy understanding.

Table 1: Anechoic chamber TRS Test parameters for band n78 with 50MHz CBW

| Test method | Band | CBW | UL RB | DL RB | Channel | Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| AC | n78 | 50M | 128@0 | 133@0 | 621668 | 3325.02 |
|  |  |  |  |  | 636666 | 3549.99 |
|  |  |  |  |  | 651666 | 3774.99 |

We use the same devices for Rel-17 lab alignment activity (i.e. LAD1 and LAD2) for the measurements. The results in [2] and this paper are merged, as shown in Figure 1.

Figure 1: TRS measurement results, RC with different coherence bandwidth (CB), reference 100MHz AC results from Rel-17 lab alignment activity and new 50MHz AC measurement results in this paper. (left LAD1, right LAD2)

Observation 1: There is no clear trends for 50MHz and 100MHz TRS comparison between AC and RC with different coherent bandwidth. Gap between AC and RC results are small, i.e. within 1dB.

Observation 2: Increasing the coherence bandwidth of RC larger than 2MHz does not provides much difference.

Proposal 1: RAN4 should confirm at least 2MHz coherence bandwidth of RC system is sufficient for NR FR1 measurement.

Proposal 2: Encourage companies to provide more measurement or analysis to confirm 5 time of SCS is sufficient for NR FR1.

For RC lab alignment activity, it is agreed that the collection of volunteered test labs can be started after RAN4#106-bis meeting. The test scope is still FFS (same as Rel-17 and/or Rel-18 AC lab alignment test cases).

Issue 3-2-1: General for Harmonization of RC vs AC

Agreements:

  - The RC harmonization activity should be started after the completion of the test method (including finalization of preliminary MU assessment).

  - The collection of volunteered test labs for RC harmonization activity can be started in RAN4#106-bis meeting

Issue 3-2-2: Harmonization framework

Agreements:

  - Approve the following high-level aspects for RC harmonization activity.

    - At least 3 RC labs are needed to ensure the statistical analysis of harmonization outcome

    - Adopt the same test cases of Rel-17 and/or Rel-18 AC lab alignment activity, i.e. same bands, browsing mode and/or talk mode

    - The measurement results will be analyzed to show RC lab alignment outcome and RC vs AC harmonization conclusion

    - FFS whether same pass/fail limits for RC lab alignment and RC vs AC harmonization

Considering there are several months before August meeting, and it is expected that RAN5 would make progress on RC MU assessment. It is proposed to confirm the volunteered labs for RC harmonization in this meeting, and conclude work management based on test scope and number of volunteered labs.

To provide a full set of results, the RC harmonization test scope should consider both Rel-17 and Rel-18, i.e. at least browsing mode and talk mode at band n41/n78.

Proposal 3: RC harmonization test scope should consider both Rel-17 and Rel-18, i.e. at least browsing mode and talk mode at band n28 and n78.

Proposal 4: Collect and confirm the volunteered test labs for harmonization activity:

- Confirm the following volunteer labs for Rel-18 RC harmonization activity.

| Volunteer lab | City | Contact |
| --- | --- | --- |
| vivo | Dongguan, Guangdong | ruixin. wang@vivo.com |
| To be added |  |  |

Note that the volunteered test lab for harmonization activity just needs to provide RC measurement results.

These RC measurement results will be used to analysis both RC lab performance variation and RC vs AC comparison. The AC results will use Rel-17 lab alignment measurements in [3], and new Rel-18 AC lab alignment results.

- Conclusion

In this contribution, we share the measurement results of RC method and have the following observations and proposal:

Observation 1: There is no clear trends for 50MHz and 100MHz TRS comparison between AC and RC with different coherent bandwidth. Gap between AC and RC results are small, i.e. within 1dB.

Observation 2: Increasing the coherence bandwidth of RC larger than 2MHz does not provides much difference.

Proposal 1: RAN4 should confirm at least 2MHz coherence bandwidth of RC system is sufficient for NR FR1 measurement.

Proposal 2: Encourage companies to provide more measurement or analysis to confirm 5 time of SCS is sufficient for NR FR1.

Proposal 3: RC harmonization test scope should consider both Rel-17 and Rel-18, i.e. at least browsing mode and talk mode at band n28 and n78.

Proposal 4: Collect and confirm the volunteered test labs for harmonization activity:

- Confirm the following volunteer labs for Rel-18 RC harmonization activity.

| Volunteer lab | City | Contact |
| --- | --- | --- |
| vivo | Dongguan, Guangdong | ruixin. wang@vivo.com |
| To be added |  |  |

Note that the volunteered test lab for harmonization activity just needs to provide RC measurement results.

# References

- R4-2305904	WF for Rel-18 TRP TRS WI, vivo

- R4-2301557, Reverberation chamber test methodology, vivo

- R4-2212817, Outcome of 3GPP TRP TRS lab alignment activity, vivo