3GPP TSG-RAN4 Meeting #107	R4-2308293

# Incheon, Korea, May 22th – May. 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3205 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE specific CBW RMCs_R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Perf |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CAT.A CR of R4-2310108 Two UE-specific CBW RMCs (CBW.1.1 and CBW.1.2) are defined in 38.133 A.3.25: - CBW.1.1: startRB = pointA; - CBW.1.1: startRB = carrier lower edge; However, these RMCs conflicts with RAN5 test frequencies definition. According to 38.508-1 cl.4.3.1.1 and c.4.3.1.2, three test frequencies are defined for most of bands, and the offset between PointA and carrier lower edge (i.e. offsetToCarrier) is given by: - “Low”: offsetToCarrier = 0; - “Mid”: offsetToCarrier = 102; - “High”: offsetToCarrier = 504; As a result, - On “Low” frequency, CBW.1.1 and CBW.1.2 are exactly the same since RBx = 0, which makes CBW change test case meaningless. - On “Mid” and “High” frequencies, offsetToPointA is already larger than carrier BW, which means DLCBW.1.1 can’t fully contain CD-SSB and CORESET 0 (depicted in following figure) which may cause UE failing the test. To fix this issue, we suggest change the offsetToPointA for CBW.1.1 to “the offset between point A and the lowest PRB index to guarantee the CBW including SSB and CORESET#0”, depicted as follows. In this way, although “Low” frequency still can’t be used for CBW change TCs, at least we can use “Mid” or “High” frequencies (“Mid” frequency shall be used by default according to 38.533 Annex E). |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE-specific CBW RMCs are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3.25	Channel bandwidth (CBW) configurations

A.3.25.1	DL UE specific CBW

Table A.3.25.1-1: DL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | DLCBW.1.1 | DLCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx, and CBW should include SSB and CORESET#0. |  |  |  |

A.3.25.2	UL UE specific CBW

Table A.3.25.2-1: UL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | ULCBW.1.1 | ULCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx. |  |  |  |

<End of modified section 3>