3GPP TSG-RAN WG4 Meeting # 107	R4-2308323

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.8.3.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on receiving timing difference between different directions for NR FR2 multi-Rx

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM impacts of NR FR2 multi-Rx chain DL reception were further discussed, with agreement captured in [1][2]. The scope and scenarios were further clarified. In this paper, we further provide our views RTD conditions for FR2 multi-Rx.

# Discussion

Regarding the receiving timing difference conditions, the current status is summarized as follows with tentative agreements in RAN4#106 and 106bis.

## MRTD Handling - Agreements in online session: - Define RRM requirements for MRTD < CP - FFS whether to support MRTD > CP and the discussion can take place after RAN4 #107 - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier Issue 3-1-3: MRTD for UE not supporting RTD>CP Agreed to postpone the discussion of MRTD > CP until RAN4#107.

Since it is schedule to discuss MRTD > CP from RAN4#107 , we provide analysis in the contribution for better understanding about the RTD in mTRP/multi-Rx scenario. This is a trade-off problem since Rel-16 when RTD within CP is the basic assumptions. In Rel-18 MIMO_evo discussion, it was agreed to consider RTD lager than CP as an optional UE capability. Though one can argue that RAN1’s agreement is related to UL transmission discussed in different WI, as described in WID, the two TA operation is highly depending on multiple panel architecture.

From Oct 10th GTW session Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

Based RAN4 discussion, it was agreed that MRTD and MTTD value is 8 and 8.5 us.

Observation 1: Based on RAN4 and RAN1 agreements, UE can support RTD > CP (as an optional UE capability) with MRTD/MTTD of 8/8.5us for both intra-cell and inter-cell mTRP.

From RRM requirements perspective, the only spec impact is the exact value of MRTD/MTTD which is already agreed. For the requirements under discussion in Rel-18 Multi-Rx, the only specification impact is the applicability conditions. The main concerns to support RTD larger than CP is the UE implementation complexity. However, since such capability is already agreed to be introduced, there is no need to repeat the discussion on whether to consider such implementation/capability.

Observation 2: There is minor specification impact to support RTD larger than CP since it only works as applicability conditions.

Observation 3: There is no need to repeat the discussion on implementation complexity since it was already agreed to be introduced as an optional UE capability.

Thus, based on the observation above, we do not see the strong reason to preclude the case when RTD>CP for Rel-18 Multi-Rx. Based on the analysis above, the requirement enhancement under discussion are also applies to RTD>CP, and specific requirements can be discussed when necessary.

Proposal 1: Define requirements for RTD>CP with optional UE capability for FR2 multi-Rx.

Proposal 2: Requirement enhancement under discussion are also applies to RTD>CP, and specific requirements can be discussed when necessary.

# Conclusions

Observation 1: Based on RAN4 and RAN1 agreements, UE can support RTD > CP (as an optional UE capability) with MRTD/MTTD of 8/8.5us for both intra-cell and inter-cell mTRP.

Observation 2: There is minor specification impact to support RTD larger than CP since it only works as applicability conditions.

Observation 3: There is no need to repeat the discussion on implementation complexity since it was already agreed to be introduced as an optional UE capability.

Proposal 1: Define requirements for RTD>CP with optional UE capability for FR2 multi-Rx.

Proposal 2: Requirement enhancement under discussion are also applies to RTD>CP, and specific requirements can be discussed when necessary.

# References

[1] R4-2306318	WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo

[2] R4-2306319	WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2), Qualcomm