3GPP TSG-RAN WG4 Meeting # 107	R4-2308390

# Incheon, South Korea, May 22nd – 26th, 2023

Title: 	Further discussion on RF requirements for 5G Broadcast

Agenda Item:	9.3.3

Source: 	MediaTek Inc, Apple.

Document for:	Approval

# Introduction

For UE RF for 5G Broadcast, RAN4 has continued the discussion in RAN4#106bis-e, and one WF was agreed with three open issues, and the below shows two of them related to REFSENS and ACS [1].

Refsens - The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with Option 1. 6, 7, and 8 MHz PMCH for reference sensitivity Option 2. 10 MHz PDSCH for reference sensitivity Way forward: FFS these options - ACS Option 1: 33 dB (for all 6, 7, and 8 MHz bandwidths) Option 2: 29 dB (for all 6, 7, and 8 MHz bandwidths) Option 3: Channelization; Power in transmission bandwidth configuration; PInterferer; ACS. 10 MHz (baseline); REFSENS + 14 dB; REFSENS + 45.5; 33 dB. 6 MHz; REFSENS + 14 dB; REFSENS + 28.5; [16] dB. 7 MHz; REFSENS + 14 dB; REFSENS + 30.0; [17.5] dB. 8 MHz; REFSENS + 14 dB; REFSENS + 35.0; [22.5] dB. Way forward: FFS these options

In this contribution, we present our views and proposals on these open issues.

# Discussion

The WID clearly assumes 10MHz UE receive channel filter as shown below, with an understanding that the intention is to reuse UE LTE hardware.

NOTE 2: The later RAN4 WI (part 2) shall assume that coexistence among different systems in the portion of the UHF band allocated to broadcast (~470 - ~694/698 MHz) is ensured through coordination, in line with regional and national regulation. NOTE 3: New (dedicated) channel filters (e.g. 6/7/8 MHz filters) are not considered for the UE.

However, the operation of filter BW wider than desired channel BW operation is unusual in LTE design. Therefore, the discussion about the REFSENS by applying such specific implementation/configuration is ongoing. Here, we would like to share information for further discussion.

Under cells coverage, regarding UE RX 10MHz CBW operation, it is difficult to preclude unexpected interferer/noise aliasing or leakage into desired channel (e.g., 6/7/8 MHz) because the interferer/noise is not fully rejected by using 10MHz filters chain. Basically, we assume that the undesired interferer/noise level may be not equal to adjacent channel level. If the undesired interferer/noise level is really close to adjacent channel level, REFSENS relaxation is expected (i.e., the “REFSENS + 14 dB” is allowed). However, it is confusing about the management for considering that undesired noise level is quite lower than adjacent channel level and the undesired noise is not efficiently rejected because of applying filters BW wider than desired channel BW based on the unusual CBW application.

Regarding another scenario (e.g., different UE implementation under conductive RFESENS verification), when filters BW is wider than desired channel BW, the operation of down-sampling/decimation or sample-rate converter could not preclude the undesired noise (e.g., spur, sigma-delta ADC noise) aliasing into desired channel. Probably, UE companies showing specific performance concern could further add/design the 6/7/8 MHz modes for their UE for removing risk. However, this additional work does not fulfil the WI intension. In other words, to consider whether 10MHz CBW for noise integration would be applicable.

Proposal 1: Regarding REFSENS, to specify the 470 – 698 MHz band and/or the 612 – 652 MHz band with 10 MHz PDSCH for reference sensitivity, i.e., Option 2.

It is also observed that WI objective states “Reuse existing requirements for 10 MHz as much as possible”. Therefore, regarding the ACS requirement for 6/7/8 MHz desired channel, there are three options for further discussion.

After further check the results from [2], we observed that the adjacent channel partial energy is still within the 10MHz filter BW and cause the FFT dynamic range (bits number) to be higher compared to FFT operation under legacy LTE ACS testing. In other words, legacy LTE adjacent channel/interferer is knocking down by ACS before entering FFT. Regarding the 10MHz filter operation for 6MHz desired channel, one way to have same FFT dynamic-range operation as before is to use the option 1 as stated in [2]. However, if the adjacent 5 MHz adjacent channel/interferer could not be outside the channel filter due to requested broadcast scenario, we consider that using the option 2 proposed in [2] for ACS is another solution.

Proposal 2: Regarding the desired 6 MHz channel, if the adjacent 5 MHz ACS interferer could not be outside the channel filter due to requested broadcast scenario, to consider defining ACS case 1 as -16 dB and 5 MHz adjacent channel interferer with frequency offset at 5.5125 MHz.

# Conclusion

In the contribution, the consideration about REFESNS and ACS is proposed.

Proposal 1: Regarding REFSENS, to specify the 470 – 698 MHz band and/or the 612 – 652 MHz band with 10 MHz PDSCH for reference sensitivity, i.e., Option 2.

Proposal 2: Regarding the desired 6 MHz channel, if the adjacent 5 MHz ACS interferer could not be outside the channel filter due to requested broadcast scenario, to consider defining ACS case 1 as -16 dB and 5 MHz adjacent channel interferer with frequency offset at 5.5125 MHz.

# References

[1] R4-2306566, “WF on UE RF requirements for 5G Broadcast”, Qualcomm Inc.

[2] R4-2302704, “In-channel ACS for 5G broadcast”, Qualcomm Incorporated.