# 3GPP TSG-RAN WG4 Meeting # 107                                 R4-2308404

# Incheon, KR, May 22 – May 26, 2023

# Title: Discussion on demodulation requirement for intra-band 	non-collocated EN-DC/NR-CA deployment

# Source: 	ZTE Corporation

# Agenda item:	8.12.4

# Document for:	Approval

# 1 Introduction

A new WI for intra-band non-collocated ENDC/NRCA was approved in RAN#95-e meeting. In RAN4#106 meeting, it was concluded that the demodulation requirement needs to be discussed[1].

Issue 1-2-2: Test scope Whether to define the corresponding UE demodulation requirements for Type 2 UE Way forward: - Option 1: Define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario. - Interested companies assume the maximum received time difference value (33us) and maximum power imbalance assumption (25dB). - Option 2: Not define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario.

In this contribution, we give some initial discussion on the demodulation aspect.

# 2	Discussion

## 2.1 Type 2 UE

In the RF session, the Type 2 architecture was reached as below

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | NRCA ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |

As independent RF chains with 2Rx for each CC are used for n77(2A)/n78(2A), data carried on one CC is not to be affected by the other one.

For demodulation performance, it is found that the demodulation performances of ENDC(intra-band contiguous and intra-band non-contiguous) with power imbalance and CA with power imbalance were defined in 38101-4. And the power imbalance defined in the current specification is 6 dB which is copied below[2]:

Table 5.2A.2.2-3: Minimum performance for FDD CA with 15 kHz SCS Test Number; Bandwidth (MHz); Reference channel; Power at antenna port (dBm/Hz); Reference value Fraction of Maximum Throughput (%). PCell; SCell; PCell; SCell; for PCell; for Scell; PCell; SCell. 1; Selected Channel bandwidth as per section 5.1.1.6; Derived as per section 5.1.3.2 of TS 38.214 [12]; NA; -112; -106; 85; NA. Table 5.2A.2.2-4: Minimum performance for TDD CA with 30 kHz SCS Test Number; Bandwidth (MHz); Reference channel; Power at antenna port (dBm/Hz); Reference value Fraction of Maximum Throughput (%). PCell; SCell; PCell; SCell; for PCell; for Scell; PCell; SCell. 1; Selected Channel bandwidth as per section 5.1.1.6; Derived as per section 5.1.3.2 of TS 38.214 [12]; NA; -112; -106; 85; NA. Table 9.5B.1.1.1-1: Minimum performance for FDD EN-DC with 15kHz SCS Test Number; Bandwidth (MHz); Reference channel; Power at antenna port (dBm/Hz); Reference value Fraction of Maximum Throughput (%). NR SCG CC; E-UTRA MCG CC (Note 1); NR SCG CC; E-UTRA MCG CC; NR SCG CC; E-UTRA MCG CC (Note 1); NR SCG CC. 1; Selected EN-DC combination as per the test procedure; NA; Derived as per section 5.1.3.2 of TS 38.214 [12]; -106; -112; NA; 85. Note 1: The number of E-UTRA MCG carriers depend on the intra-band and inter-band contiguous EN-DC configuration and bandwidth combination set. Table 9.5B.1.1.1-2: Minimum performance for TDD EN-DC with 30kHz SCS Test Number; Bandwidth (MHz); Reference channel; Power at antenna port (dBm/Hz); Reference value Fraction of Maximum Throughput (%). NR SCG CC; E-UTRA MCG CC (Note 1); NR SCG CC; E-UTRA MCG CC; NR SCG CC; E-UTRA MCG CC (Note 1); NR SCG CC. 1; Selected EN-DC combination as per the test procedure; NA; Derived as per section 5.1.3.2 of TS 38.214 [12]; -106; -112; NA; 85. Note 1: The number of E-UTRA MCG carriers depend on the intra-band and inter-band contiguous EN-DC configuration and bandwidth combination set.

As Type 2 UE is to be introduced, so the demodulation requirements should be defined for Type 2 UE, which is different from that of Type 1 UE.

Proposal 1. The demodulation requirements of Type 2 UE need to be reflected in the specification.

# 3 Conclusion

Proposal 1. The demodulation requirements of Type 2 UE need to be reflected in the specification.

# 4	Reference

[1]  R4-2305891, WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario, Ericsson

[2]  TS 38.101-4-h80