# 3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308409

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on NCR-MT demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	8.29.6

# Document for:	Approval

# 1 Introduction

During the last RAN4 meeting, some conclusions has been reached for NCR-MT demodulation requirements. In this contribution, we want to share analysis on demod requirement impacts from RAN4 perspective based on the listed feature from RAN1/RAN2 perspective. Generally speaking, since the NCR-MT is equipped with baseband functionality, then its demod requirement should be defined to check its practical performance. In the following section, we will provide the detailed assumptions for NCR-MT demod requirements.

# 2 Discussion

  - Sub-topic 1-1 General

Issue 1-1-2: Whether define new requirements for FR1

Agreements: - Define new requirements for FDD/15 KHz and TDD/30 KHz for 1 layer only by reusing/adapting requirements for IAB-MT. - 10M/15kHz; 40M/30kHz - FFS whether need to consider additional CHBW 5MHz/15kHz

With regard to channel bandwidth, NCR-MT will operate in all NR bands with different channel bandwidth. Currently, the minimum bandwidth supported by NR bands is 5MHz. Considering 5MHz is not a typical scenario for FDD, we don’t have strong will to consider additional CHBW 5MHz/15kHz.

Proposal 1. No need to consider additional CHBW 5MHz/15kHz

Issue 1-1-3: Whether define new requirements for FR2

Agreement: - Reuse IAB-MT requirements for NCR-MT demodulation and only consider Rank 1 with 100MHz/120kHz - FFS new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

- In our understanding, the biggest change of NCR compared to traditional repeaters is that the beam direction can be controlled by the base station. So three different access link beam indications have been defined including periodic, semi-persistent and aperiodic. And for periodic and semi-persistent are based on MAC-CE signaling transmission. In general, for the sake of reliability, lower modulation order(e.g., QPSK) is used for signaling transmission. Aperiodic beam indication is indicated by DCI. RAN1 also defined new type DCI 2_8 for NCR-MT beam indication. Since, in our understanding, there is no need to consider new requirements.

Proposal 2. No need to consider new requirements for signaling of Access link beam indication.

  - Sub-topic 1-2 PDSCH requirements

Issue 1-2-1: How to define PDSCH requirements

Agreements: - Both 70% and 30% throughput cases agreed as baseline assumption for the test cases reused from IAB-MT requirements - Other values higher than 70% not precluded - FFS for the test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced

- As mentioned above, periodic and semi-persistent are based on MAC-CE signaling transmission. In general, for the sake of reliability, lower modulation order(e.g., QPSK) is used for signaling transmission. Since there is no need to consider PDSCH for signaling of Access link beam indication. Also there is no need to consider new test metric for PDSCH.

Proposal 3. No need to consider new test metric for PDSCH.

Issue 1-2-2: MCS

Agreements: - QPSK and 16QAM

In previous meeting, we agreed that QPSK and 16QAM as modulation for NCR-MT. Since we would like to further clarify the modulation order. So we propose to consider MCS 4 for QPSK and MCS 13 for 16QAM.

Proposal 4. To consider MCS 4 for QPSK and MCS 13 for 16QAM.

  - Sub-topic 1-3 PDCCH requirements

- Issue 1-3-1: How to define PDCCH requirements

Agreements: - Reuse UE requirements for FDD and TDD - FFS for adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication.

Firstly, we would like to further clarify that DCI type 5_0 has been modified to DCI type 2_8 in RAN1 #112bis-e meeting[2]. And for new DCI 2_8, since RAN4 didn’t define requirements before. Maybe we can consider define new requirements for new DCI 2_8.

Proposal 5. To consider define new requirements for DCI 2_8.

- Issue 1-3-2: Coreset for FDD

Agreements: - Option 1: - 1TX: coreset duration 2 - 2TX: coreset duration 1 - Option 2: - 1TX: coreset duration 1 - 2TX: coreset duration 1

For FDD CORESET, we notice that existing requirements [3] defined CORESET duration 1 and 2 for 1Tx as shown in Table 5.3.2.1.1-1 and Table 5.3.3.1.1-1. And in previous meeting, we made a agreement that aggregation level 2 and 4 for 1Tx. From Table 5.3.2.1.1-1 and Table 5.3.3.1.1-1,we can find that only CORESET duration 2 is covered aggregation level 2 and 4. On the other hand, it is clearly indicated in WI [3] that the existing requirements defined in RAN4 can be reused if applicable. Based on the above analysis, we prefer CORESET duration 2 for 1TX.

Proposal 6. To consider CORESET duration 2 for 1TX.

Table 5.3.2.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

Table 5.3.3.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 2.7 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | -0.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -3.2 |

  - Sub-topic 2-2 PMI requirements

Issue 2-2-1:Whether define PMI requirements

Agreements: - FFS whether need to define PMI reporting requirements

Considering that the position of NCR-MT is fixed and the channel condition is also relative stable. If UE reporting PMI, the value of PMI is also invariable. Therefore, we don’t have strong view to define PMI requirements.

Proposal 7. No need to define PMI requirements.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for NCR-MT demodulation requirements , The conclusions are:

Proposal 1. No need to consider additional CHBW 5MHz/15kHz

Proposal 2. No need to consider new requirements for signaling of Access link beam indication.

Proposal 3. No need to consider new test metric for PDSCH.

Proposal 4. To consider MCS 4 for QPSK and MCS 13 for 16QAM.

Proposal 5. To consider define new requirements for DCI 2_8.

Proposal 6. To consider CORESET duration 2 for 1TX.

Proposal 7. No need to define PMI requirements.

# 4 References

- R4-2305934 WF for NCR-MT demodulation requirements. RAN4 #106 bis-e meeting. ZTE Corporation.

- R1-2304264

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.8.0.

- R4-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips, 3GPP TSG RAN Meeting #98-e, Electronic Meeting, December 12-16, 2022.